| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27-07 APRIL , 2012 | Document: JCTVC-0590 |
| --- | --- |

| Title: | Non-CE1: On SAO Type sharing between color component |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Informative contribution |  |  |
| Author(s) or Contact(s): | Elena Alshina, Tammy Lee, JeongHoon Park Samsung #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Guillaume LAROCHE Christophe GISQUET Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | Elena Alshina +82-10-3026-1305 {elena_a.alshina, tammy.lee, jeonghoon}@samsung.com +33(0)299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Samsung Electronics Co. Ltd, Canon Research Centre France. |  |  |

_____________________________

# Abstract

In JCTVC-I0183, a proposal to share information between the U and V components was presented, reportedly allowing a reduction in SAO parameters for Chroma components, as well as coding gains. The present informative contribution investigates sharing only the SAO type between the color components, which is asserted to offer opportunities for more efficient SAO implementation.

# Algorithm description

In HM 6.0 SAO LCU-interleave design, all SAO parameters are assigned per LCU. The parameter SAO, type index defined if SAO is enabled or disabled. When SAO is enabled, this parameter defined also the type of SAO: Edge Offset (EO) or Band Offset (BO). If SAO is used in the current LCU (i.e. either EO or BO type), then 4 offset values are encoded. Additionally, if the SAO type is BO, then the position of the first corrected band is signaled.

If EO type is selected, then each pixel in the LCU should be classified as either belonging to an edge, or not. To determine whether it is an edge, we compare the sample value with its 2 neighbors. Use of 1D templates allows to keep reasonable balance between complexity and efficiency. There are four possible directions for those 1D templates: vertical, horizontal, 135 or 45 diagonal as shown in Fig. 1.

The encoder selects one directional pattern for each LCU based on an RD optimization, and the best EO type is sent in the bitstream as side information.

Fig. 1. 1D patterns for edges classification: vertical, horizontal 135 or 45 diagonal.

Currently each color component in LCU may use its own edge type, e.g. Y component may be corrected using EO with vertical edges, while U may use BO, and finally V may use EO with another direction than vertical. The Luma-from-chroma, as well as the Derived-Mode in chroma (where the chroma angular prediction direction is inferred to be the same as luma) are proof that strong relationship exists between the components. Therefore, there seems to be redundant freedom in allowing different SAO type for each color component. Selecting the same SAO type for each color component seems a worthwhile study.

Another positive aspect of this sharing is the allowed simplifications. For instance, some implementations use the NV12 layout to store components for 4:2:0 sampling, where U and V samples are interleaved, allowing efficient memory accesses. Using different SAO types for the color component introduces irregular accesses in this scheme, so having the same type is an important benefit. Of course, equivalent simplifications are applicable to other sampling formats such as 4:4:4 or 4:2:2.

Two sharing methods in an LCU are evaluated here:

- Variant 1: Luma uses its own SAO Type, while U and V use their own;

- Variant 2: All 3 color components share the same SAO type.

# Simulation Results

Both variants 1 and 2 were implemented on top of CE1 s/w and tested according to [1] for different LCU sizes using SAO Interleaving mode. Table 1 reports summary of simulation results vs. the CE1 anchor. We report average gain across all 8 test cases: AI-main/ he10, RA-main/ he10, LD-main/ he10, LDP-main/ he10. Class F is included into averaging when Table 1 numbers were calculated.

Table 1. Test results summary compare to CE1 anchor (SAO Interleaving mode, class F is included).

| LCU size | U and V share SAO Type |  |  | Y, U & V shares SAO Type |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y, BD-rate | U, BD-rate | V, BD-rate | Y, BD-rate | U, BD-rate | V, BD-rate |
| 64x64 | 0.0% | -0.1% | 1.4% | 0.3% | 0.0% | -0.2% |
| 32x32 | 0.0% | 0.0% | 0.7% | 0.6% | 1.4% | -0.2% |
| 16x16 | 0.0% | 0.0% | 0.0% | 0.3% | 0.3% | -0.2% |

More details about test results can be found in appendix (Tables2.x – 3.x) and excel spreadsheet attached to this contribution.

# Implementation description

Due to the limited time to prepare the results within the meeting schedule, simpler modifications to the encoder SAO parameters estimation for interleaved mode were applied in both of the variants tests.

For variant 1, SAO type is encoded for Y and U components. During decoding SAO parameters for V component decoder skips SAO type parsing.

For variant 2, SAO type is encoded for I component only. During decoding SAO parameters for U&V components decoder skips SAO type parsing.

For variant 1, SAO parameters estimations for Luma and first Chroma component were exactly the same as in CE1 s/w. During the second Chroma component SAO parameters estimation, its SAO type is forced to be the same as SAO type for the first chroma component.

For variant 2, SAO parameters estimation for Luma is exactly the same as in CE1 s/w. During Chroma SAO parameters estimation, SAO type is forced to be the same as the Luma SAO type the same LCU.

Joint estimation of SAO parameters should incur even smaller performance losses, and are an encoder-only modification.

# Conclusions

Sharing SAO Type between 2 Chroma components in LCU shows negligible performance change:

0.0% Luma BD-rate;

-0.1%....1.4% Chroma BD-rate;

but brings clear benefits for memory access during simultaneous SAO processing for 2 Chroma components and so helps efficient SAO implementation.

Base on mentioned above Samsung and Canon propose to establish an AhG on SAO design simplification targeting to help efficient SAO implementation and to include SAO type sharing between different color component as one of the mandates.

# Patent rights declarations

Samsung Electronics Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-H1100, February 2012

- G. Laroche, T. Poirier, C. Gisquet, P. Onno (Cannon) “Non-CE1: On SAO parameters reduction for Chroma,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0183, April, 2012

# Appendix: Test results

Table 2.1. Test results for Variant 1 (U and V share the same SAO Type).  Anchor is CE1 s/w. LCU size 16x16.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 90% |  |  | 91% |  |  |
| Dec Time[%] | 92% |  |  | 86% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E |  |  |  |  |  |  |
| Class F | 0,0% | 0,0% | 0,2% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 90% |  |  | 90% |  |  |
| Dec Time[%] | 96% |  |  | 86% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | 0,1% | 0,0% | 0,1% | 0,0% |
| Class C | 0,0% | 0,0% | 0,2% | 0,0% | -0,2% | 0,1% |
| Class D | 0,0% | -0,2% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
| Class F | 0,0% | 0,0% | -0,6% | 0,0% | -0,1% | 0,5% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% |
|  | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,1% |
| Enc Time[%] | 91% |  |  | 91% |  |  |
| Dec Time[%] | 96% |  |  | 84% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | 0,3% | 0,0% | 0,0% | -0,2% |
| Class C | 0,0% | 0,0% | 0,3% | 0,0% | -0,1% | 0,0% |
| Class D | 0,0% | -0,3% | -0,5% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | -0,1% | -0,1% | -0,1% | -0,1% | -0,2% |
| Class F | 0,0% | -0,3% | 0,3% | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | -0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
|  | 0,0% | -0,2% | 0,1% | 0,0% | 0,0% | -0,1% |
| Enc Time[%] | 91% |  |  | 91% |  |  |
| Dec Time[%] | 96% |  |  | 84% |  |  |

Table 2.2. Test results for Variant 1 (U and V share the same SAO Type).  Anchor is CE1 s/w. LCU size 32x32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,2% | 0,0% | 0,0% | 1,0% |
| Class B | 0,0% | 0,0% | 0,5% | 0,0% | 0,0% | 0,2% |
| Class C | -0,1% | 0,0% | 1,0% | 0,0% | 0,0% | 0,4% |
| Class D | 0,0% | 0,0% | 0,3% | 0,0% | 0,0% | 0,1% |
| Class E | 0,0% | 0,0% | 0,3% | 0,0% | 0,0% | 0,1% |
| Class F | 0,0% | 0,0% | 1,3% | 0,0% | 0,0% | 0,7% |
| Overall | 0,0% | 0,0% | 0,6% | 0,0% | 0,0% | 0,4% |
|  | 0,0% | 0,0% | 0,6% | 0,0% | 0,0% | 0,3% |
| Enc Time[%] | 90% |  |  | 90% |  |  |
| Dec Time[%] | 93% |  |  | 86% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,1% | 0,0% | 0,7% | -0,1% | 0,1% | 1,1% |
| Class B | 0,0% | 0,0% | 0,7% | 0,0% | 0,0% | 0,2% |
| Class C | 0,0% | 0,1% | 1,4% | 0,0% | 0,0% | 0,3% |
| Class D | -0,1% | 0,1% | 0,6% | 0,0% | -0,2% | 0,2% |
| Class E |  |  |  |  |  |  |
| Class F | 0,0% | -0,1% | 2,4% | 0,0% | -0,1% | 1,3% |
| Overall | 0,0% | 0,0% | 0,8% | 0,0% | 0,0% | 0,5% |
|  | 0,0% | 0,0% | 0,8% | 0,0% | 0,0% | 0,5% |
| Enc Time[%] | 90% |  |  | 89% |  |  |
| Dec Time[%] | 98% |  |  | 84% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,1% | 0,2% | 0,0% | 0,0% | 0,2% |
| Class C | 0,0% | -0,2% | 2,8% | -0,1% | 0,1% | 1,7% |
| Class D | 0,0% | -0,3% | 0,7% | -0,1% | 0,0% | 0,0% |
| Class E | 0,0% | 0,1% | 0,3% | 0,0% | -0,1% | 0,1% |
| Class F | -0,1% | 0,1% | 1,6% | 0,1% | -0,3% | 0,7% |
| Overall | 0,0% | -0,1% | 1,1% | 0,0% | -0,1% | 0,6% |
|  | 0,0% | 0,0% | 1,1% | 0,0% | -0,1% | 0,4% |
| Enc Time[%] | 91% |  |  | 90% |  |  |
| Dec Time[%] | 97% |  |  | 83% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,0% | 0,3% | 0,0% | 0,0% | 0,3% |
| Class C | 0,0% | 0,0% | 2,2% | 0,0% | 0,0% | 1,4% |
| Class D | 0,0% | -0,4% | 0,6% | 0,0% | 0,2% | -0,1% |
| Class E | 0,0% | -0,1% | 0,4% | 0,0% | 0,0% | 0,3% |
| Class F | -0,1% | -0,4% | 1,5% | 0,1% | 0,0% | 1,7% |
| Overall | 0,0% | -0,2% | 1,0% | 0,0% | 0,0% | 0,7% |
|  | 0,0% | -0,2% | 1,0% | 0,0% | 0,1% | 0,7% |
| Enc Time[%] | 90% |  |  | 91% |  |  |
| Dec Time[%] | 95% |  |  | 83% |  |  |

Table 2.3. Test results for Variant 1 (U and V share the same SAO Type).  Anchor is CE1 s/w. LCU size 64x64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,9% | 0,0% | 0,0% | 1,8% |
| Class B | 0,0% | 0,0% | 1,4% | 0,0% | 0,0% | 0,8% |
| Class C | -0,1% | -0,1% | 1,4% | 0,0% | 0,0% | 0,7% |
| Class D | 0,0% | 0,0% | 0,9% | 0,0% | 0,0% | 0,5% |
| Class E | 0,0% | 0,0% | 1,3% | 0,0% | 0,0% | 0,5% |
| Class F | -0,1% | -0,1% | 1,6% | 0,0% | 0,0% | 1,1% |
| Overall | 0,0% | 0,0% | 1,2% | 0,0% | 0,0% | 0,9% |
|  | 0,0% | 0,0% | 1,2% | 0,0% | 0,0% | 0,8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 1,6% | -0,1% | -0,3% | 1,7% |
| Class B | 0,0% | -0,1% | 1,7% | 0,0% | 0,1% | 1,0% |
| Class C | 0,0% | 0,0% | 2,0% | 0,0% | 0,0% | 0,9% |
| Class D | 0,0% | 0,1% | 1,2% | 0,0% | -0,1% | 0,8% |
| Class E |  |  |  |  |  |  |
| Class F | -0,1% | -0,1% | 2,7% | 0,0% | -0,1% | 1,9% |
| Overall | 0,0% | 0,0% | 1,6% | 0,0% | -0,1% | 1,1% |
|  | 0,0% | 0,0% | 1,6% | 0,0% | -0,1% | 1,1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,1% | 1,7% | 0,0% | 0,1% | 0,6% |
| Class C | 0,0% | -0,1% | 3,4% | 0,0% | 0,0% | 1,9% |
| Class D | 0,0% | -0,1% | 2,2% | 0,0% | -0,3% | 0,8% |
| Class E | -0,1% | -0,4% | 1,5% | 0,0% | -0,1% | 0,3% |
| Class F | -0,1% | -0,5% | 2,9% | -0,1% | 0,1% | 2,3% |
| Overall | 0,0% | -0,2% | 2,3% | 0,0% | 0,0% | 1,2% |
|  | 0,0% | -0,3% | 2,3% | 0,0% | -0,1% | 1,2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | -0,1% | 1,7% | 0,0% | -0,1% | 0,0% |
| Class C | -0,1% | -0,3% | 2,8% | 0,0% | -0,2% | 1,1% |
| Class D | 0,0% | -0,1% | 1,7% | 0,0% | -0,2% | 1,5% |
| Class E | 0,1% | -0,2% | 1,3% | 0,0% | -0,5% | -0,7% |
| Class F | -0,1% | 0,2% | 2,2% | -0,1% | -0,3% | 2,2% |
| Overall | 0,0% | -0,1% | 2,0% | 0,0% | -0,2% | 0,9% |
|  | 0,0% | -0,1% | 2,0% | 0,0% | -0,2% | 0,9% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 31. Test results for Variant 2 (3 color component share the same SAO Type).  Anchor is CE1 s/w. LCU size 16x16.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% |
| Class B | 0,2% | 0,2% | 0,2% | 0,0% | 0,1% | 0,1% |
| Class C | 0,3% | 0,3% | 0,3% | 0,1% | 0,1% | 0,1% |
| Class D | 0,2% | 0,2% | 0,2% | 0,1% | 0,1% | 0,1% |
| Class E | 0,2% | 0,2% | 0,2% | 0,0% | 0,0% | 0,0% |
| Class F | 0,6% | 0,6% | 0,4% | 0,4% | 0,4% | 0,4% |
| Overall | 0,3% | 0,3% | 0,2% | 0,0% | 0,1% | 0,1% |
|  | 0,3% | 0,3% | 0,2% | 0,0% | 0,1% | 0,1% |
| Enc Time[%] | 90% |  |  | 91% |  |  |
| Dec Time[%] | 92% |  |  | 86% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,1% | 0,1% | 0,0% | 0,1% | 0,0% |
| Class B | 0,6% | 0,5% | 0,5% | 0,2% | 0,2% | 0,2% |
| Class C | 0,5% | 0,5% | 0,4% | 0,2% | 0,2% | 0,2% |
| Class D | 0,5% | 0,3% | 0,3% | 0,1% | 0,2% | 0,2% |
| Class E |  |  |  |  |  |  |
| Class F | 0,8% | 0,6% | 0,1% | 0,4% | 0,4% | 0,3% |
| Overall | 0,4% | 0,4% | 0,3% | 0,1% | 0,2% | 0,1% |
|  | 0,4% | 0,3% | 0,3% | 0,1% | 0,2% | 0,2% |
| Enc Time[%] | 90% |  |  | 90% |  |  |
| Dec Time[%] | 95% |  |  | 87% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,3% | 0,3% | 0,3% | 0,1% | -0,1% | -0,1% |
| Class C | 0,4% | 0,2% | 0,4% | 0,0% | 0,1% | 0,2% |
| Class D | 0,3% | 0,1% | -0,4% | 0,0% | 0,2% | 0,2% |
| Class E | 0,3% | 0,4% | 0,2% | 0,0% | 0,2% | 0,1% |
| Class F | 1,0% | 0,4% | -0,8% | 0,4% | 0,5% | 0,9% |
| Overall | 0,5% | 0,3% | 0,0% | 0,1% | 0,2% | 0,3% |
|  | 0,5% | 0,2% | -0,1% | 0,1% | 0,2% | 0,3% |
| Enc Time[%] | 91% |  |  | 91% |  |  |
| Dec Time[%] | 94% |  |  | 84% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,9% | 0,6% | 1,0% | 0,2% | 0,2% | 0,2% |
| Class C | 0,9% | 0,7% | 0,6% | 0,3% | 0,4% | 0,3% |
| Class D | 0,8% | 0,3% | 0,6% | 0,3% | 0,5% | 0,4% |
| Class E | 1,0% | 0,7% | 0,4% | 0,1% | 0,1% | 0,1% |
| Class F | 1,1% | 0,5% | 0,6% | 0,5% | 0,6% | 0,0% |
| Overall | 0,9% | 0,6% | 0,7% | 0,3% | 0,4% | 0,2% |
|  | 0,9% | 0,5% | 0,7% | 0,3% | 0,3% | 0,2% |
| Enc Time[%] | 91% |  |  | 91% |  |  |
| Dec Time[%] | 95% |  |  | 85% |  |  |

Table 3.2. Test results for Variant 2 (3 color component share the same SAO Type).  Anchor is CE1 s/w. LCU size 32x32.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 0,2% | 0,2% | 0,2% | 0,3% | 1,0% |
| Class B | 0,4% | 0,4% | 0,4% | 0,3% | 0,3% | 0,4% |
| Class C | 0,3% | 0,4% | 0,4% | 0,3% | 0,4% | 0,4% |
| Class D | 0,3% | 0,0% | 0,0% | 0,2% | 0,2% | 0,2% |
| Class E | 0,6% | 0,7% | 0,5% | 0,3% | 0,5% | 0,4% |
| Class F | 0,3% | 0,3% | 0,2% | 0,3% | 0,3% | 0,4% |
| Overall | 0,4% | 0,3% | 0,3% | 0,2% | 0,3% | 0,5% |
|  | 0,4% | 0,3% | 0,3% | 0,2% | 0,3% | 0,4% |
| Enc Time[%] | 90% |  |  | 91% |  |  |
| Dec Time[%] | 92% |  |  | 86% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,5% | 0,5% | 0,4% | 0,2% | 0,4% | 0,5% |
| Class B | 0,7% | 0,3% | 0,3% | 0,4% | 0,4% | 0,4% |
| Class C | 0,6% | 0,5% | 0,4% | 0,3% | 0,6% | 0,3% |
| Class D | 0,4% | 0,1% | 0,0% | 0,2% | 0,0% | 0,1% |
| Class E |  |  |  |  |  |  |
| Class F | 0,7% | 0,6% | 0,5% | 0,4% | 0,5% | 0,2% |
| Overall | 0,5% | 0,4% | 0,3% | 0,3% | 0,4% | 0,4% |
|  | 0,5% | 0,4% | 0,3% | 0,3% | 0,3% | 0,4% |
| Enc Time[%] | 90% |  |  | 90% |  |  |
| Dec Time[%] | 97% |  |  | 84% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,9% | -0,7% | -1,8% | 0,4% | 0,4% | 0,2% |
| Class C | 0,8% | -0,5% | -0,6% | 0,4% | 0,6% | 1,0% |
| Class D | 0,6% | -0,7% | -1,1% | 0,3% | 0,1% | -0,3% |
| Class E | 1,4% | -1,2% | -1,9% | 0,4% | 0,5% | 0,5% |
| Class F | 1,4% | 0,3% | -1,7% | 1,0% | -0,6% | -1,4% |
| Overall | 1,0% | -0,5% | -1,4% | 0,5% | 0,2% | 0,0% |
|  | 1,0% | -0,4% | -1,3% | 0,5% | 0,1% | -0,1% |
| Enc Time[%] | 91% |  |  | 91% |  |  |
| Dec Time[%] | 97% |  |  | 84% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1,4% | -0,8% | -1,5% | 0,6% | 0,5% | 0,4% |
| Class C | 1,0% | -0,7% | -0,9% | 0,6% | 0,3% | 0,8% |
| Class D | 0,8% | -1,2% | -2,1% | 0,5% | 0,5% | -0,8% |
| Class E | 2,7% | -1,5% | -1,9% | 1,0% | 0,7% | 0,8% |
| Class F | 1,5% | -0,5% | -1,8% | 1,2% | 0,0% | 0,1% |
| Overall | 1,4% | -0,9% | -1,6% | 0,8% | 0,4% | 0,2% |
|  | 1,4% | -0,9% | -1,6% | 0,8% | 0,3% | 0,2% |
| Enc Time[%] | 90% |  |  | 91% |  |  |
| Dec Time[%] | 96% |  |  | 83% |  |  |

Table 3.3. Test results for Variant 2 (3 color component share the same SAO Type).  Anchor is CE1 s/w. LCU size 64x64.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | 0,3% | 0,3% | 0,0% | 0,2% | 0,8% |
| Class B | 0,1% | 0,3% | 0,7% | 0,1% | 0,5% | 0,8% |
| Class C | -0,1% | 0,7% | 1,0% | 0,0% | 0,6% | 0,9% |
| Class D | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |
| Class E | 0,0% | 1,0% | 0,9% | 0,1% | 1,2% | 0,9% |
| Class F | -0,1% | 1,1% | 1,4% | 0,0% | 0,9% | 1,1% |
| Overall | 0,0% | 0,5% | 0,7% | 0,0% | 0,5% | 0,7% |
|  | 0,0% | 0,5% | 0,7% | 0,0% | 0,5% | 0,7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,3% | 1,4% | 0,5% | 0,1% | 0,4% | 0,6% |
| Class B | 0,3% | 0,3% | 0,6% | 0,2% | 0,5% | 0,7% |
| Class C | 0,2% | 0,6% | 0,7% | 0,2% | 0,6% | 0,8% |
| Class D | 0,1% | 0,7% | 0,3% | 0,1% | 0,2% | 0,1% |
| Class E |  |  |  |  |  |  |
| Class F | 0,3% | 0,9% | 1,0% | 0,2% | 0,8% | 0,7% |
| Overall | 0,2% | 0,7% | 0,5% | 0,1% | 0,4% | 0,6% |
|  | 0,3% | 0,7% | 0,5% | 0,1% | 0,4% | 0,5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,6% | -0,6% | -1,6% | 0,4% | -0,4% | -0,8% |
| Class C | 0,4% | -0,3% | 0,1% | 0,3% | 0,2% | 0,2% |
| Class D | 0,2% | 0,1% | -0,3% | 0,1% | 0,1% | -0,9% |
| Class E | 1,5% | -2,1% | -3,1% | 0,8% | -0,6% | -1,0% |
| Class F | 0,8% | -0,2% | -0,2% | 0,6% | 0,7% | -0,7% |
| Overall | 0,7% | -0,6% | -1,0% | 0,4% | 0,0% | -0,6% |
|  | 0,7% | -0,5% | -0,9% | 0,4% | 0,0% | -0,6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,8% | -1,6% | -2,1% | 0,5% | -0,8% | -1,0% |
| Class C | 0,4% | -0,4% | -0,4% | 0,3% | -0,1% | -0,3% |
| Class D | 0,2% | 0,1% | -1,2% | 0,1% | -0,4% | -0,4% |
| Class E | 2,1% | -3,0% | -3,2% | 1,0% | -1,9% | -1,5% |
| Class F | 0,7% | 0,3% | -0,4% | 0,6% | 0,1% | -0,6% |
| Overall | 0,8% | -0,8% | -1,4% | 0,5% | -0,5% | -0,7% |
|  | 0,8% | -0,8% | -1,2% | 0,5% | -0,5% | -0,7% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |