| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0591 |
| --- | --- |

| Title: | Report of BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm USA Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Email: | 1-858-651-8345 yekuiw@qualcomm.com +1 201 478 6245 jill@vidyo.com |
| Source: | BoG on high-level syntax |  |  |

_____________________________

This document contains draft and unofficial meeting notes from the break-out group (BoG) on high-level syntax.

### 5.12.8 High-level syntax cleanup

JCTVC-I0037 Removal of Syntax Redundancies [Y. Morigami, K. Sato (Sony)]

Abstract: In HEVC CD text specification, SPS (sequence parameter set) contains syntax element “chroma_format_idc”. Chroma_format_idc = 0 means that input video is in monochrome format. At the 8th JCT-VC meeting in San Jose, it was proposed by JCTVC-H0457 to remove monochrome format support from HEVC specification, and meeting note says: “We will have some profile that does not support monochrome, and an indicator flag in VUI”. Main profile currently does not support monochrome input, but HEVC syntax itself still supports monochrome. There would be profiles that support monochrome format either in version 1 or 2 of HEVC.

If input is monochrome, syntax elements related to Cb/Cr are redundant. In fact such redundancy is excluded on weighted prediction in the current HEVC specification, but not on other syntax elements as:

- chroma_pred_from_luma_flag

- bit_depth_chroma

- cb_qp_offset and cr_qp_offset

- sao_cb_enable_flag and sao_cr_enable_flag

- alf_cb_enable_flag and alf_cr_enable_flag

- intra_chroma_pred_mode

- pcm_sample_chroma

- scaling list

This document proposes to remove such redundancies from HEVC syntax so long as there exists profiles that support monochrome either in version 1 or 2 of HEVC. Otherwise, syntax redundancy removal on weighted prediction would not be necessary.

Comments:

The proposed syntax introduces a parsing dependency of PPS on SPS.

If we don't anything now, in a potential new profile in the future that supports 4:0:0, we can still add a condition in a way that is backwards compatible. In such a potential new profile, it should be useful to do this.

Recommend no action taken.

JCTVC-I0127 Syntax cleanup [T. Lee, J. Park (Samsung)]

Document abstract: There are syntaxes having dependency each other. To clarify the dependency and reduce redundant bits, the conditional coding or differential coding of the information related temporal motion predictor, dQP signalling unit and entropy slice are proposed.

1) Makes the current signaling of collocated_from_l0_flag and collocated_ref_idx further conditioned on enable_temporal_mvp_flag

Resolved by decision of I0266.

2) To fix a bug in the semantics of max_cu_qp_delta_depth, and further proposes a syntax cleanup for bits saving.

Recommend adoption. (remember to put a value range when integrating).

3) Makes entropy_slice_flag conditional on !first_slice_in_pic_flag, and also all SH syntax elements and syntax structures from five_minus_max_num_merge_cand conditional on !entropy_slice_flag.

Recommend  adoption.

JCTVC-I0165 On Signalling of Several Syntax Elements in Slice Header [J. An, X. Guo, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, some conditions and constrains are proposed to be added on the signaling of two syntax elements in slice header, aps_id and inherit_dbl_params_from_aps_flag. It is reported that the proposed method can save some redundant bits, as well as avoid the potential contradictory between related syntax elements.

No longer relevant due to APS related decisions at this meeting.

JCTVC-I0144 Removal of input picture size constraint and picture cropping offset parameters in sequence parameter set [I.-K Kim, Y. Park, J. H. Kim, J. H. Park (Samsung)]

In this contribution, removal of a constraint on picture width and height (picture width and height shall be integer multiple of minimum CU size) is proposed. Based on the proposal, it is also suggested to move the position of picture cropping offset parameters from sequence parameter set (SPS) to supplemental enhancement information (SEI). By this modification, general cropping functionality is kept as optional and only mandatory cropping process is kept in the decoding process.

BoG recommends no action.

JCTVC-I0420 High-level Syntax: Proposed fix on signalling of TMVP disabling flag [C. S. Lim, S. Mon Thet Naing (Panasonic)]

This contribution proposes to explicitly signal the enable_temporal_mvp_flag at every P and B slice header. It is purported that the benefit includes cleaner design for TMVP disabling process. It is suggested that the JCT-VC consider this proposal in refining the design of TMVP disabling process.

1) To remove the marking process

Recommend to adopt

2) Explicitly signaling the flag in the slice header

Suggestion: To have a flag in the SPS, if that flag is equal to 1, then no flag in the slice header, otherwise the slice header flag is sent.

Suggestion: The flag in the slice header shall be the same for all slices in a picture.

Recommend to adopt the above suggestions.

3) Encoder constraint to enable the refresh point (no functionality change)

No action recommended.

JCTVC-I0170 Parsing robustness issue for deblocking filter syntax [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

According to the current HM6.0 the deblocking control parameters can be signalled either in the Adaptation Parameter Set (APS) syntax structure or in the slice header. It is noted in the proposal that when the deblocking control parameters are sent in the APS, the parsing of the syntax element slice_loop_filter_across_slices_enabled_flag in the slice header depends on the DB control parameters that are sent in the APS. This results in a parsing issue for the slice data, since the APS is expected to be subject to packet losses in the communication network. The contribution provides a solution for the parsing problem with a small change in the slice header syntax. With the help of the proposal the slice data can be parsed independently from the APS.

No longer relevant due to APS related decisions at this meeting.,

### 5.12.10 Multi-topic contributions

JCTVC-I0143 High level syntax modifications [B. Li, H. Li (USTC), H. Yang (Huawei)]

This document presents several modifications of the high level syntax of the current HEVC. The proposed modifications include: (1) the nal_ref_flag restriction of CRA picture; (2) differentially signaling of per-temporal layer information; (3) the position of signaling per-temporal information in SPS; and (4) the derivation of POC when num_reorder_pics is 0.

- CRA restriction on nal_ref_flag: nal_ref_flag shall be equal to 1 for CRA pictures.

Recommend to adopt

- Need to introduce a variable. Save some bits in the SPS. Relevant SW NB comment to disallow smaller values.  Recommend to note the restriction recommended in the NB comment.

- It was commented that the picture resolution should be earlier than these parameters. No action.

- No action.

- Should be reviewed by a different group, or ignored because it is a late contribution. It was noted that this may have already been adopted in JCTVC-I0414. Open.

JCTVC-I0330 Comments on HEVC text spec draft 6 [Y.-K. Wang, M. Coban, Y. Chen, R. L. Joshi, J. Chon, A. K. Ramasubramonian (Qualcomm)]

The contribution proposes to remove the constraint on maxPicOrderCnt – minPicOrderCnt.

It was asked whether, in the current specification, the picture corresponding to prefRefPic needs to be in the RPS. A participant indicated that there is currently no expressed requirement in this regard.

It was suggested that if we keep the constraint, the prefRefPic picture should be included in the set of pictures over which the difference constraint applies. Open.

Many parts.

- temporal_id semantics, distinguish between VCL and non-VCL NAL units.  Define temporal_id of an access unit.

May want to add to definition section also.

Revisit after language of NOTE revised.  The relationship with TLA was questioned.

- Tile syntax in SPS and PPS

Already addressed by JCTVC-I0113 decision in the parallel processing BoG.

- picture size signaling in multiples of 8

It was noted that other standards referring to the specification may benefit from referring directly to a syntax element.   No action.

- slice header byte alignment

In the JEVC text, the slice header may or may not be an integer number of bytes.

The HM always has slice headers aligned.

It is noted that ALF and SAO no longer use slice header signaling.

Revisit: This should be addressed by the parallel processing BoG.

- Editorial, new VLC code

Editors can consider.  No action.

- Entropy slices.

Revisit: This should be addressed by the parallel processing BoG.

- coding of slice address in the slice header

Already resolved by JCTVC-I0113 decision in the parallel processing BoG.

- Applicability of inter-RPS prediction

It was suggested to further change the inter-RPS to add a parameter to indicate whether it is being called by the SPS or the slice header.

Revisit when text for new text for the syntax structure is available.

- Relationship between TLA and temporal_id_nesting_flag

It was suggested to always mark as TLA (when not an IDR or CRA).

Recommend to adopt: When temporal_id_nesting_flag is equal to 1 and temporal_id>0, nal_unit_type shall be equal to 3.  .

- POC constraint

Revisit when Gary is available.

- Replace rbsp_trailing_bits( ) with byte_alignment( ) in slice_data

Recommend to adopt #11.

- Change  num_reorder_pics[ i ] semantics

Recommend to adopt #12

- Semantics of list_modification_present_flag

SW NB comment about integration problem on H0412.  This an editorial issue and should be addressed by the editors.

- Editorial comments.

Editors can consider.  No action.

### 5.12.6 SEI messages

JCTVC-I0218 Changes to the hash SEI calculation for improved usability [T. Hellman, W. Wan (Broadcom)]

This contribution claims that the two hash SEI messages, as presently defined, are difficult for a real-time encoder or decoder to use.  It states that three independent post-decode memory passes are necessary to compute the hashes.  It recommends two changes: computing separate MD5 hashes for luma and chroma, rather than just one, and replacing the second hash type (CRC) with a checksum.  It claims that the first change allows real-time decoders to compute the MD5 at display time, and the second allows real-time computation of the hash at both encode time and decode time.

It was suggested that the size of the MD5sum could be reduced, and that 16 bytes is stronger than needed.

Replacing CRC with checksum allows order independent processing, but offers less protection. It was noted that H.271 uses CRC32.

It was noted that checksum or CRC may be useful for error detection of parameter sets.

A suggestion was made to use a checksum method as a 3rd method, not removing the CRC.

A suggestion was made to clarify operation if calculation overflows 32 bits.

Recommend to adopt solution 2 of I0218 the checksum method, in addition to the CRC and MD5sum. 3 checksums/hashes, one per color component.

JCTVC-I0245 LCU based Input order scanning for hash SEI Message [R. R. Srinivasan, M. Mody, C. Ghone (TI)]

Hash SEI messages are defined to provide checksum of the decoded picture in the current access unit. With the current definition, computation of these checksum can be done only as an end of frame operation, as there is a sequential dependency between different color components and within in a color component, lines taken in raster scan order are input to checksum engine. This proposal suggests breaking dependency between different color components and also proposes a way to do this checksum calculation during LCU decoding process immediately after in-loop filtering process. There by making the checksum calculation as a LCU based operation, instead of a line-based operation. This reduces frame-end computation complexity and reduces memory bandwidth.

Propose to send 3 separate MD5sum values, one for each color component.

Not aligned with LCU grid because of post-filtering.

The interaction with slices and tiles was questioned.  The coding order of the SEI message w.r.t. the coded picture should be considered.  Perhaps a POC in the SEI message.  There is a restriction now that SEI messages must precede the VCL NAL units of an access unit, and perhaps this should be relaxed.

First issue: 1 vs. 2 vs. 3 MD5sums.  It was remarked that having 3 could help identify which color component had a problem.

Recommend to have 3 MD5sums, one per color component.  If we keep the CRC, it should also have 3.

It was remarked that if the checksum were used, individual LCU operation and offset aligment is unnecessary.

See decision in I0218.

JCTVC-I0044 Modification of recovery point SEI message [K. Kazui, J. Koyama, S.   Shimada, A. Nakagawa (Fujitsu)]

TBP

JCTVC-I0057 Frame packing arrangement SEI extension for HEVC [O. Nakagami, T. Suzuki (Sony)]

In the current draft, frame packing arrangement SEI is defined as the same as SEI defined in AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10). This contribution proposes to modify the frame packing arrangement SEI for HEVC to support coding tools in HEVC.

Two extensions to the frame packing arrangement SEI are proposed. The proposed extension links the view information and coding structure. One is an extension for frame sequential coding using temporal_id. The other is an extension for frame packing coding using tile.

The proposed extensions enable to represent view information with fewer bits. Furthermore, decoders can output only the base view without entire decoding process since the additional view is independent of the base view in the coding structure point of view.

Suggested use case using temporal_id.  Has no normative change.

Another use case using tiles, requires change to SEI message syntax.

Frame packing SEI message uses tile parameters from the PPS, but doesn’t have a pps_id.

Semantics for temporal_id option is unclear.  Intended decoder operation is not defined.

Recommend no action.

No action.JCTVC-I0058 On stereo 3D coding using frame packing arrangement SEI [O. Nakagami, T. Suzuki (Sony)]

Frame packing stereo 3D coding using frame packing arrangement SEI is supported in the current draft. However, one issue is compatibility among decoders since decoding SEI messages is not a mandatory process. This contribution proposes three alternative methods to realize the 2D compatibility among the decoders.

- Has 3 options. Add a 1-bit flag in VUI to indicate the existence of the SEI message.b

Uses “should” wording which isn’t done in the spec.

- If some conditions in SEI message are met, normative decoding cropping process is changed.

It was mentioned that DVB uses this method for AVC frame packing.

IT NB comment to fix such schemes in HEVC.

Revisit in track.

JCTVC-I0072 Usage of cropping rectangle and sample aspect ratio to support 2D-compatible services in “Frame compatible Plano-Stereoscopic 3DTV” within the HEVC specification [M. Arena, P. Sunna (RAI)]

Revisit in track.

JCTVC-I0062 Comments on field indication SEI message [M. Li, P. Wu (ZTE), C. Fogg (Harmonic)]

TBP

JCTVC-I0393 How to use the field sequence SEI [C. Fogg (Harmonic), A. Wells (Ambarella)]

TBP