| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0592 |
| --- | --- |

| Title: | DST quantization matrix for DST in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jian Lou Yue Yu Wei-Ying Kung Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current HEVC committee draft, 4x4 discrete sine transform (DST) is employed for 34 out of 35 intra prediction modes. However, the human visual system (HVS) based quantization matrix for the 4x4 DCT is used for the 4x4 DSTs resulting in visual artifacts. This document describes two modifications on top of a proposed DST in intra prediction proposal from Nokia to reduce complexity, improve subjective quality, and harmonize coding process of luma and chroma components.

# Introduction

In the current HEVC committee draft (CD) [1], 4 transform combinations of 1D-DCT and 1D-DST are used for 4x4 intra TU depending on its chosen intra prediction mode. Regardless of transform combination used for a 4x4 intra TU, there is only one option for quantization matrix, which is designed based on 4x4 DCT. In [2], 3 additional quantization matrices were proposed so that each transform combination has its own matching quantization matrix.

In [3], it was reported that the current 4 transform combinations based on intra prediction mode complicates the decoder design and two simplifications were described. In the simplification 1, DST is used for all intra prediction modes of 4x4 TUs, while in the simplification 2, DST is used for all prediction modes except DC prediction.

The first part of this contribution reports performance results of DST quantization matrix when DST is used for all intra prediction modes for 4x4 luma TU. The DST quantization matrix used in this test is the same as the one proposed in [2], as shown in Table 1.

Table 1 Quantization matrix for 4x4 Intra TUs with 1D vertical DST and 1D horizontal DST in HEVC [2]

| 5 | 7 | 8 | 10 |
| --- | --- | --- | --- |
| 7 | 12 | 15 | 17 |
| 8 | 15 | 29 | 43 |
| 10 | 17 | 43 | 87 |

The second part of this contribution describes a solution to resolve inconsistency in transform matrix used in intra 4x4 luma TU and intra 4x4 chroma TU when 2D-DST is used for all intra prediction modes for 4x4 luma TU. The proposed solution utilizes 2D-DST for all intra prediction modes for intra 4x4 TU; i.e., 2D-DST is used for both intra 4x4 luma and intra 4x4 chroma TUs. This modification guarantees that there will be no transform matrix switching under 8x8 block level.

# Simulation results

The two modifications were implemented on top of [3] to evaluate their performance. Common test conditions are used in the simulation, except for scaling matrix option. Cross check of the results were performed by TI and I2R.

Experiment results show that the usage of DST quantization matrix for DST in intra luma prediction show 13.8% BD rate loss for luma and 18.7% BD rate gain for chroma on average compared to [3] for all intra case as shown in Table 1. Experimental results also show that the usage of 2D-DST for all intra 4x4 TU show 0.2% BD rate gain for luma and 1.8% BD rate loss for chroma on average compared to [3] for all intra case as shown in Table 2.

Table 1 Experimental results of DST quantization matrix for DST in intra luma prediction

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 11.1% | -15.4% | -19.2% | 11.5% | -15.6% | -18.3% |
| Class B | 10.7% | -16.3% | -19.3% | 10.7% | -15.6% | -17.8% |
| Class C | 16.6% | -18.6% | -19.5% | 16.3% | -17.1% | -17.5% |
| Class D | 17.1% | -20.0% | -20.4% | 16.9% | -19.3% | -19.6% |
| Class E | 12.9% | -21.3% | -23.3% | 13.3% | -18.6% | -22.1% |
| Overall | 13.8% | -18.4% | -20.2% | 13.8% | -17.3% | -18.9% |
|  | 13.8% | -18.0% | -19.9% | 13.8% | -17.0% | -18.6% |
| Class F | 17.2% | -10.3% | -7.9% | 16.8% | -9.8% | -7.8% |

Table 2 Experimental results of 2D-DST for all intra 4x4 TU

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 1.5% | 2.1% | 0.0% | 1.4% | 2.3% |
| Class B | -0.1% | 1.5% | 1.7% | -0.1% | 1.7% | 2.2% |
| Class C | -0.3% | 1.3% | 1.6% | -0.3% | 1.9% | 2.4% |
| Class D | -0.2% | 1.6% | 1.6% | -0.3% | 1.9% | 2.0% |
| Class E | -0.2% | 2.4% | 1.3% | -0.3% | 3.2% | 1.7% |
| Overall | -0.2% | 1.6% | 1.6% | -0.2% | 2.0% | 2.1% |
|  | -0.2% | 1.6% | 1.6% | -0.2% | 2.0% | 2.1% |
| Class F | -0.2% | 1.2% | 1.0% | -0.2% | 1.6% | 1.7% |

# Change to CD text

The proposed syntax changes to support two modifications are shown below. The part with red strike through is removed and the part in yellow highlight shows changes from current text.

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are set equal to 1. specified as Error: Reference source not found with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 7-4 – Specification of default values of ScalingList[ 0 ][ MatrixID ][ i ] with i=0..15

| - i | - 0 | - 1 | - 2 | - 3 | - 4 | - 5 | - 6 | - 7 | - 8 | - 9 | - 10 | - 11 | - 12 | - 13 | - 14 | - 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - ScalingList[0][0..2][i] | - 5 | - 7 | - 7 | - 8 | - 12 | - 8 | - 10 | - 15 | - 15 | - 10 | - 17 | - 29 | - 17 | - 43 | - 43 | - 87 |
| - ScalingList[0][3..5][i] | - 16 | - 16 | - 16 | - 17 | - 17 | - 17 | - 21 | - 21 | - 21 | - 21 | - 24 | - 24 | - 24 | - 36 | - 36 | - 57 |

# References

[1] B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 6,” JCTVC-H1003

[2] J. Lou, L. Wang, “Quantization matrices for 4x4 DSTs in HEVC,” JCTVC-I0268

[3] K. Ugur, O. Bici, “Performance evaluation of DST in intra prediction,” JCTVC-I0582

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).