| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0594 |
| --- | --- |

| Title: | Bypass bins for reference index and delta QP coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Xianglin Wang, Wei-Jung Chien, Jianle Chen, Marta Karczewicz (*) Vivienne Sze, Madhukar Budagavi (**) (*) Qualcomm 5775 Morehouse Drive San Diego, CA 92121, USA (**) Texas Instruments | Tel: Email: | vseregin@qualcomm.com sze@ti.com |
| Source: | Qualcomm and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution asserts that the worst case number of context coded bins for ref_idx and cu_qp_delta should be reduced.  Simplification in reference index coding is proposed according to which all bins after the second bin are coded with CABAC bypass mode [JCTVC-I0352]. Experimental results show no performance change and 8% of bypass bins in average under four common test configurations. The proposal also results in one context removal. For delta QP coding, it proposes using a combination of truncated unary and Exp-Golomb coding (TU4EG0), similar to what is used for motion vector difference [JCTVC-I0204].  Since delta QP is not signaled under common conditions, it was tested in HM-6.1 under the conditions specified by CE4 on Quantization.  There is negligible coding efficiency impact (between -0.02 to 0.03).

# Introduction and proposal

Currently the ref_idx (reference index) and cu_qp_delta (delta QP) account for 28% of the number of context coded bins in the worst case.

There are two proposals JCTVC-I0204 and JCTVC-I0352 related to the context bins reduction. In the first one [2] it was proposed to use TU4EG0 for both reference index and delta QP with EG0 coded in bypass mode. In the second contribution [1] it was proposed to keep the current binarization with truncated unary (TU) code unchanged for reference index, but apply bypass mode after the second bin.

After further discussion and analysis, proponents suggest that for reference index truncated unary is used with bypass coding after the second bin, and for delta QP coding TU4EG0 should be used. Such a conclusion is explained in the next two sections.

# Reference index

Truncated unary and TU4EG0 binarization are summarized in the next table 1.

Table 1. Comparison between TU and TU4EG0.

| Ref index | Truncated unary | TU4EG0 | Bin difference (TU – TU4EG0) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Non-last index | Last index |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 10 | 10 | 0 | 0 |
| 2 | 110 | 110 | 0 | 0 |
| 3 | 1110 | 1110 | 0 | 0 |
| 4 | 11110 | 1111 0 | 0 | 0 |
| 5 | 111110 | 1111 100 | -1 | -2 |
| 6 | 1111110 | 1111 101 | 0 | -1 |
| 7 | 11111110 | 1111 11000 | -1 | -2 |
| 8 | 111111110 | 1111 11001 | 0 | -1 |
| 9 | 1111111110 | 1111 11010 | 1 | 0 |
| 10 | 11111111110 | 1111 11011 | 2 | 1 |
| 11 | 111111111110 | 1111 1110000 | 1 | 0 |
| 12 | 1111111111110 | 1111 1110001 | 2 | 1 |
| 13 | 11111111111110 | 1111 1110010 | 3 | 2 |
| 14 | 111111111111110 | 1111 1110011 | 4 | 3 |
| 15 | 111111111111111 | 1111 1110101 | 4 | 4 |

Since number of reference frames are known exactly and fixed, truncated unary code represents the reference index without extra unused bins and is a better choice in this case.

The numbers in the above table show the difference in number of bins for reference index coding between truncated unary code and truncated unary combined with Exponential Golomb code TU4EG0. In the fourth column (Non-last index), bin difference is summarized when reference index is not the last one and in the fifth column, the same difference is summarized, but for the case when the reference index form the first column is the last one.

For example, let us consider reference index equal to 5 but there are more than six reference frames. The two code words with each under different binarization are the following:

111110 vs 1111  100, truncated unary is one bin shorter and -1 is put in the fourth column.

If reference index is equal to 5 and there are six reference frames, reference index equal to 5 is last possible one. In this case truncated unary will be one bin shorter and related code words are the following:

11111 vs 1111  100, truncated unary is two bins shorter and -2 is put in the fifth column.

In summary, there is a bin saving in the cases from 5 to 8 which are more frequently used than those larger reference indices. So it is better to keep the current binarization unchanged for reference index coding, but apply bypass coding after the second bin. Such a simplification also removes one context.

Results based on HM6.0 and common test conditions [3] are summarised in the following Table 2. No noticeable performance change was observed.

Table 2. Experimental results for proposed method

|  | RA-Main | RA-HE10 | LB-Main | LB-HE10 |
| --- | --- | --- | --- | --- |
| Proposal BD-rate | 0.0 | 0.0 | 0.0 | 0.0 |
| Bypass bins | 7% | 6% | 9% | 8% |

# Delta QP

Relative to reference index, the range of delta QP variation, from –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2), is much higher than for reference index. The last bin for delta QP is generally far from 0. In this case, Exponential Golomb binarization (TU4EG0) has an advantage of resulting less bins in average.  Prefix of Exponential Golomb (TU4) is context coded and the suffix (EG0) is bypass coded.

In addition, in HM-6.1 where only TU is used, the cMax of TU is depends on the sign value of cu_qp_delta.  Thus, the sign must be sent before the magnitude bins.  In HM-6.1, to code cu_qp_delta requires switching from context coded bin, to bypass and then to context coded bins again.

With TU4EG0, there is no dependency on the sign bin.  Furthermore, the sign could be moved to the end of the cu_qp_delta and be grouped with the bypass bins of the suffix.

To summarize, the benefits of using TU4EG0 over the HM-6.1 where only TU is used are:

- Reduction of total context coded bins in worst case (26 bins  5 bins)

- Reduction of total bins in worst case  (27 bins  16 bins)

- Removes dependency of cMax on sign value

- Can group bypass bins and fewer switches between context coded bins and bypass

Since delta QP is not signaled under common test conditions, it was tested in HM-6.1 under the conditions specified by CE4 on Quantization.  There is negligible coding efficiency impact between -0.02 to 0.03.

# Conclusion

Based on this analysis, we propose to adopt truncated unary scheme with bypass mode after the second bin for reference index and truncated unary with cMax=4 combined with 0th order Exponential Golomb code for delta QP coding.

# References

- V. Seregin et al, “Bypass bins for reference index coding,” JCTVC-I0352, Geneva, Apr. 2012.

- V. Sze et al, “Reduction in context coded bins for ref_idx and cu_qp_delta,” JCTVC-I0204, Geneva, Apr. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-H1100, San Jose, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-H1003.

## Reference index

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialization process

| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | intra_chroma_pred_mode | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_flag | Error: Reference source not found |  | 0..3 | 4..7 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Table 9-2 |  | 0..2 0..1 | 3..5 2..3 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 1 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 2 | 3 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |

Table 9-2 – Values of variable initValue for ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_lc, ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 2 | 4 3 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| ref_idx_lc | 1 | TU, cMax = num_ref_idx_lC_active_minus1 | 2 1 | Table 9-2 | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |
| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 2 1 | Table 9-2 | 0 |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 2 1 | Table 9-2 | 0 |
|  | 2 |  | 2 1 | Table 9-2 | 3 2 |

Table 9-4 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values

| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| ref_idx_l0 | Table 9-2 | 0 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |
| ref_idx_l0, ref_idx_l1, ref_idx_lc | Table 9-2 | 3 | 0 | 1 | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) | 2 na (uses Decode Bypass) |

## Delta QP

| Table 9-5 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initialisationType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| cu_qp_delta | 0 | as specified in subclause | Prefix: 2 Suffix: na | Prefix: Error: Reference source not found Suffix: na | Prefix: 0 Suffix: na, (uses Decode Bypass) |
|  | 1 |  | Prefix: 2 Suffix: na | Prefix: Error: Reference source not found Suffix: na | Prefix: 3 Suffix: na, (uses Decode Bypass) |
|  | 2 |  | Prefix: 2 Suffix: na | Prefix: Error: Reference source not found Suffix: na | Prefix: 6 Suffix: na, (uses Decode Bypass) |

9.2.2.6 Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

The prefix bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	cNum = min (absV − 1,cMax)
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}

If absVal-1 > =cMax, then the suffix absVal-1-cMax is binarized using Exp-Golomb (EG0) process defined in 9.2.2.4