| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0595 |
| --- | --- |

| Title: | Performance Comparison of HM 6.0 with Existing Still Image Compression Schemes Using a Test Set of Popular Still Images |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | T. Nguyen D. Marpe Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This input document provides an intra-only coding performance analysis of HM 6.0 by using a test set of photographic still images. Furthermore, compression performance comparison with H.264/AVC, WebP, JPEG 2000, and JPEG XR for still image coding is provided. JPEG baseline is also included and serves as the anchor for the coding performance evaluation.

# Introduction

Preliminary performance comparisons between the test model encoder of the upcoming high efficiency video coding (HEVC) standard and an H.264/AVC High Profile conforming encoder report average bit-rate savings of about 42% and 23% for a random-access and an all-intra configuration, respectively [1]. This document presents intra-only coding performance results for HM 6.0 and H.264/AVC together with popular image-coding standards like JPEG, JPEG 2000, and JPEG XR as well as the proprietary WebP scheme. By using a representative test set of photographic still images, a clear ranking in terms of objective coding performance with average bit-rate savings in the range of 16 – 43% for HEVC intra-only coding relative to all other competing coding schemes has been observed.

# Performance Setup

The performance analysis employs the latest available reference software implementations for all candidates. For the hybrid video-compression schemes, the reference implementation of H.264/AVC JM (version 18.1) from [2] and version 6.0 of the HM for HEVC from [3] is used. For the still image compression standards, the official reference implementation for JPEG, JPEG 2000, and JPEG XR, as listed in [4] are used. Finally, the reference implementation for WebP was obtained in version 0.1.3 from [5].

The test set for the performance evaluation consists of 34 still images with various content and characteristics. In addition to two well-known grayscale test images used for image processing purposes, two classes of color images representing a wide range of content are included. In total, the test set is a composition of three different classes. Each class consists of images with the same resolution. Table 1 gives an overview of the class specification. Class A contains the well-known images ‘barbara’ and ‘lena’, both as 8-bit grayscale image of dimension 512×512 pixels. Class B consists of 24-bit RGB color images with 1280×1600 pixels as obtained from [4] and class C consists of the 24 color images from the Kodak test set available at [6].

|  | Width | Height | # Images | Type |
| --- | --- | --- | --- | --- |
| Class A | 512 | 512 | 2 | 8 bit grayscale |
| Class B | 1280 | 1600 | 8 | 24 bit color |
| Class C | 768 | 512 | 24 | 24 bit color |

Table 1: Overview of the test set classes

The original images of the test set were given as Windows Bitmap (BMP) files in case of class A and class B, and as lossless Portable Network Graphics (PNG) files in case of class C. This implies that the color space of the original data is RGB with 8-bit data representation for each color component. The implementation of the anchor compression scheme JPEG accepts BMP files as input. However, the encoder converts the color space internally from RGB to YCbCr with a 4:2:0 sub-subsampling of the chroma components before entering the compression stage. Both JPEG 2000 and JPEG XR accept YCbCr 4:2:0 subsampled input data in addition to BMP files. In case of JM and HM, only YCbCr input is possible, while HM currently supports 4:2:0 subsampled input data only. In order to provide the same input for all tested codecs, the original RGB data of the BMP files were converted to 4:2:0 subsampled YCbCr data by using the same color conversion matrix as in JPEG. Furthermore, the same smooth downsampler for the subsampling of the chroma components are applied as used in JPEG in the second conversion stage. By doing so, it is ensured that the input is the same for all candidates. For the two grayscale images related to class A of the test set, the luma component is extended by 4:2:0 sub-sampled chroma components consisting of zero sample values only.

# Results

Fig. 1: RD plot for barbara

Fig. 2: RD plot for lena

| Anchor ↓ | HEVC | H.264/AVC | JPEG 2000 | JPEG XR | WebP | JPEG |
| --- | --- | --- | --- | --- | --- | --- |
| HEVC |  | 19.0 | 29.7 | 43.6 | 45.9 | 76.3 |
| H.264/AVC | -15.8 |  | 9.3 | 20.5 | 21.6 | 48.1 |
| JPEG 2000 | -22.6 | -8.2 |  | 10.2 | 11.4 | 35.6 |
| JPEG XR | -30.0 | -16.8 | -9.6 |  | 0.7 | 23.4 |
| WebP | -31.0 | -17.6 | -9.5 | -0.3 |  | 23.2 |
| JPEG | -43.0 | -32.3 | -26.0 | -18.9 | -18.6 |  |

Table 2: Averaged BD rate relative to different anchors

# References

- B. Li, G. J. Sullivan, and J. Xu, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile”, Doc. JCTVC-H0360, San Jose, Jan. 2012.

- Joint Model 18.1, download available at:
http://iphome.hhi.de/suehring/tml/download/

- HEVC test model 6.0, download available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-6.0/

- “JPEG core experiment for the evaluation of JPEG XR image coding”
http://jahia-prod.epfl.ch/site/mmspl/op/edit/page-58334.html

- WebP 0.1.3, download available at:
http://code.google.com/speed/webp/

- “Kodak Lossless True Color Image Suite”
http://r0k.us/graphics/kodak/