| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0596 |
| --- | --- |

| Title: | On Main Profile Definition: Testing results of LM Mode |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Information] |  |  |
| Author(s) or Contact(s): | Lingzhi Liu (HiSilicon) Xun Guo (MediaTek) Oscar C. Au (The Hong Kong University of Science & Technology) Jungsun Kim (LG) | Tel: Email: | +1 4083304996 lingzhi.liu@huawei.com xun.guo@mediatek.com eeau@ust.hk jungsun.kim@lge.com |
| Source: | HiSilicon, MediaTek, HKUST and LGE |  |  |

_____________________________

# Abstract

This contribution provides the testing results of chroma from luma intra prediction mode, which is also referred as LM mode, on HM6.0 software. The BD-Rate of LM mode are: AI-Main: Y: -0.7%, U: -7.5%, V: -5.7%; AI-HE10: Y: -0.7%, U: -10.1%, V: -6.7%; RA_Main: Y: -0.2%, U: -9.4%, V: -6.4%; RA_HE10: Y: -0.3%, U: -12.4%, V: -8.2%; LDB_Main: Y: -0.1%, U: -2.7%, V: -2.1% and LDB_HE10: Y: -0.1%, U: -2.6%, V: -2.3%.

# Testing results

Test results are provided here as well as the attached excel file.

Table 1 Intra_main+LMChroma vs Intra_main

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -1.2% | -16.0% | -9.3% |
| Class B | -0.6% | -7.0% | -3.8% |
| Class C | -1.0% | -6.4% | -7.5% |
| Class D | -0.6% | -4.8% | -4.8% |
| Class E | -0.1% | -2.3% | -2.8% |
| Overall | -0.7% | -7.5% | -5.7% |
|  | -0.7% | -7.4% | -5.6% |
| Class F | -2.0% | -6.2% | -7.6% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Intra_HE10-LMChroma vs Intra_HE10

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 1.2% | 25.2% | 12.4% |
| Class B | 0.7% | 8.4% | 4.1% |
| Class C | 1.0% | 7.8% | 8.8% |
| Class D | 0.6% | 5.4% | 5.3% |
| Class E | 0.1% | 2.2% | 2.7% |
| Overall | 0.7% | 10.1% | 6.7% |
|  | 0.7% | 9.9% | 6.6% |
| Class F | 2.0% | 7.7% | 8.7% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3 RA_Main+LMChroma vs RA_Main

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -18.0% | -10.4% |
| Class B | -0.3% | -8.4% | -4.2% |
| Class C | -0.4% | -6.7% | -7.1% |
| Class D | -0.3% | -5.0% | -4.3% |
| Class E |  |  |  |
| Overall | -0.2% | -9.4% | -6.4% |
|  | -0.3% | -9.4% | -6.3% |
| Class F | -0.9% | -6.4% | -6.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4 RA_HE10-LMChroma vs RA_HE10

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 29.4% | 18.1% |
| Class B | 0.3% | 9.4% | 4.1% |
| Class C | 0.5% | 6.6% | 7.1% |
| Class D | 0.3% | 5.1% | 4.6% |
| Class E |  |  |  |
| Overall | 0.3% | 12.4% | 8.2% |
|  | 0.3% | 12.3% | 8.0% |
| Class F | 0.9% | 7.5% | 7.5% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 5 LDB_Main+LMChroma vs LDB_Main

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -3.5% | -1.9% |
| Class C | -0.1% | -2.9% | -3.0% |
| Class D | 0.0% | -1.8% | -1.7% |
| Class E | 0.0% | -2.4% | -1.7% |
| Overall | -0.1% | -2.7% | -2.1% |
|  | -0.1% | -2.7% | -2.2% |
| Class F | -0.5% | -5.1% | -5.5% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 5 LDB_HE10-LMChroma vs LDB_HE10

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 3.4% | 2.1% |
| Class C | 0.2% | 3.3% | 3.3% |
| Class D | 0.1% | 2.0% | 1.9% |
| Class E | 0.0% | 1.2% | 2.0% |
| Overall | 0.1% | 2.6% | 2.3% |
|  | 0.1% | 2.7% | 2.4% |
| Class F | 0.4% | 4.7% | 8.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

The LM mode shows not only sufficient gain for chroma components but also non-negligible gain for Luma component. It is suggested to include the LM in the main profile.