| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0599 |
| --- | --- |

| Title: | Cross check of coding of SAO merge left and merge up (JCTVC-I0507) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ehsan Maani 1730 N. First St. San Jose, CA 95112 | Tel: Email: | (408) 352 4710 ehsan.maani@am.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This contribution reports the performance of modified merge flag coding proposed in [1]. JCTVC-I0507 [1] proposes to use the merge left flag to signal whether all color components are merge left, and to use the merge up flag to signal whether all color components are merge up. Otherwise, SAO offset parameters are signaled for each color component. In the HM6.0 process, on the other hand, the coding of the SAO merge left and merge up is performed for each color component separately.

# Cross check procedure

The source code for [1] was provided by Motorola Mobility to Sony. Sony compiled the source code using Microsoft Visual Studio and the simulation was performed on a Windows HPC cluster. The algorithm in the provided source code accords with the algorithm presented in [1].

# Results

Results are obtained according to the test settings of CE1 and compared against CE1 anchors (timing may not be accurate)

|  | Low delay B Main CU64x64 |  |  | Low delay B HE10 CU64x64 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.7% | -0.9% | -0.1% | 0.3% | 0.6% |
| Class C | -0.1% | -0.8% | -0.8% | -0.2% | 0.2% | 0.4% |
| Class D | -0.4% | -0.9% | -0.4% | -0.4% | 0.2% | -0.1% |
| Class E | -1.2% | -2.3% | -2.1% | -1.2% | -0.6% | -0.4% |
| Overall | -0.4% | -1.1% | -1.0% | -0.4% | 0.1% | 0.2% |
|  | -0.4% | -1.1% | -1.0% | -0.4% | -0.1% | 0.2% |
| Class F | -0.4% | -0.6% | -0.6% | -0.2% | 1.1% | 1.1% |
| Enc Time[%] | 100% |  |  | 107% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |
|  | Low delay B Main CU32x32 |  |  | Low delay B HE10 CU32x32 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.4% | -0.8% | -0.2% | -0.1% | 0.0% |
| Class C | -0.3% | -0.2% | -0.1% | -0.3% | 0.2% | 1.0% |
| Class D | -0.6% | -1.1% | -1.0% | -0.6% | -0.5% | -0.9% |
| Class E | -1.9% | -2.4% | -2.7% | -1.7% | -1.9% | -1.9% |
| Overall | -0.7% | -0.9% | -1.0% | -0.6% | -0.5% | -0.3% |
|  | -0.7% | -0.9% | -1.1% | -0.6% | -0.5% | -0.4% |
| Class F | -0.7% | -0.2% | -0.6% | -0.5% | -1.8% | -0.3% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B Main CU16x16 |  |  | Low delay B HE10 CU16x16 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -1.0% | -1.2% | -0.8% | -0.9% | -0.7% |
| Class C | -0.6% | -0.4% | -0.5% | -0.6% | -0.7% | -0.5% |
| Class D | -0.9% | -1.1% | -1.1% | -0.8% | -0.2% | -0.2% |
| Class E | -4.1% | -3.6% | -3.8% | -4.2% | -3.6% | -3.8% |
| Overall | -1.4% | -1.4% | -1.5% | -1.4% | -1.2% | -1.1% |
|  | -1.4% | -1.4% | -1.5% | -1.4% | -1.2% | -1.1% |
| Class F | -1.6% | -2.3% | -1.6% | -1.4% | -3.4% | -2.5% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# References

- K. Minoo, D. Baylon, I. S. Chong, M. Karczewicz, “Non-CE1: coding of SAO merge left and merge up”, JCTVC-I0507, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.