| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0601 M25260 |
| --- | --- |

| Title: | Evaluation of combination of LCU SAO enable flag coding methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1562 wskim@ti.com +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In 9th JCT-VC meeting, there have been several proposals dealing with signaling methods to turn off SAO. In JCTVC-I0193, an index is signaled to turn off Y, Cb, and Cr combinations, where one bin can be used to turn off SAO for all three color components for each LCU. In JCTVC-I0199, SAO on/off flag is signaled for each color component separately, and neighboring LCUs are used to select context model. In JCTVC-I0184, SAO is turned off for the whole LCUs in a frame based on the statistics of the previous frame.

In this document, the coding efficiency is investigated when these methods are combined. Experimental results reportedly show that the combined method of JCTVC-I0193 + JCTVC-I0199 can improve coding efficiency by 0.0%, 0.2%, and 0.9% for LCU size of 64, 32, and 16, respectively, for luma component compared to JCTVC-I0119 in case of low-delay B main, when interleaving mode is used.

The coding efficiency of the combination of JCTVC-0193 + JCTVC-I0199 with JCTVC-0184 is reported in this document as well. It is observed that this combination improves coding efficiency by 0.2%, 0.3% and 0.4% for LCU size of 64, 32, and 16, respectively, for luma component compared to JCTVC-I0184 in case of low-delay B main.

# Introduction

In HM-6.0 [1], each LCU can have its own SAO parameter sets, which can increase bitrate to signal those parameters. To reduce the bitrate there have been several proposals to efficiently turn SAO off for each LCU. In JCTVC-I0193 [2], an index is signaled to turn off Y, Cb, and Cr combinations, where one bin can be used to turn off SAO for all three color components for each LCU. In JCTVC-I0199 [3], SAO on/off flag is signaled for each color component separately, and neighboring LCUs are used to select context model. In JCTVC-I0184 [4], SAO is turned off for the whole LCUs in a frame based on the statistics of the previous frames.

In this document, the coding efficiency is investigated when these methods are combined.

# Description of Combining Methodologies

2.1 JCTVC-I0193 + JCTVC-I0199

2.1.1 Combined SAO flag for Y/Cb/Cr

First bin is used to indicate to turn on or off SAO for all three color components, and this bin is coded using the first context model. If this bin is 0, no further signaling occurs. Otherwise, three more bins are signaled as follows. Second bin is used to indicate to turn on or off SAO for luma component, and this bin is coded using the second context model. Third and fourth bins are used to indicate to turn on or off SAO for Cb and Cr component, respectively, and the third context model is used for both bins.

This method provides an efficient method to turn off SAO for each LCU by signaling only one bin as in JCTVC-I0193, while separate flag coding for each component comes from JCTVC-I0199. Note that three context models are used. However, no reference to neighboring LCUs are necessary, which does not require a line buffer.

As SAO off is decoupled from SAO type, this requires modification of SAO type in HM-6.0 as shown in Table 1.

Table 1. Proposed SAO type table.

| - sao_type_idx | - SAO type |
| --- | --- |
| - 0 | - 1D 0-degree edge |
| - 1 | - 1D 90-degree edge |
| - 2 | - 1D 135-degree edge |
| - 3 | - 1D 45-degree edge |
| - 4 | - Band |

2.1.2 Efficient Merge Flag Signaling

In both JCTVC-I0193 and JCTVC-I0199, an efficient merge flag signaling method is proposed. If an upper or left LCU is available, SAO parameters can be copied from those of upper or left LCU using merge up or merge left flag. When SAO is turned off in the upper or left LCU, it is not necessary to copy SAO parameters from it.

By use of LCU SAO enable flag as proposed in section 2.1.1, unnecessary signaling of merge flags can be effectively removed by checking neighboring LCU’s LCU SAO enable flag. For each color component, SAO parameters or merge flags are signaled only when LCU SAO enable flag is equal to 1. In this case, if upper/left LCU’s SAO enable flag is 0, we propose not to send merge up/left flag.

2.2 Difference between JCTVC-I0193 + JCTVC-I0199 and JCTVC-I0199

In summary, there are two major differences between JCTVC-I0193 + JCTVC-I0199 and JCTVC-I0199.

First, Y/Cb/Cr SAO on/off flag is introduced in this new combined method. If Y/Cb/Cr SAO flag is 0, no further parsing is necessary. If Y/Cb/Cr SAO flag is 1, SAO flag of each component is parsed. One benefit of this Y/Cb/Cr SAO flag is that it can reduce bits when SAO is not enabled for all color components. This benefit becomes significant as LCU size becomes smaller in which most of LCU does not use SAO for all color components. Another benefit is that it provides encoder with flexibility to turn off SAO for all color components without any cost. For example, if we want to turn chroma SAO off when luma SAO is off, we can simply send one Y/Cb/Cr SAO flag, rather than sending three SAO flag for Y, Cb and Cr as in JCTVC-I0199.

Second, context models for SAO flag are changed. Unlike JCTVC-I0199, neighboring (up/left) LCUs’ SAO flag is not used to select context model in JCTVC-I0193 + JCTVC-I0199. Instead, Y/Cb/Cr SAO flag uses one context, and Y SAO flag uses the second context, and Cb and Cr SAO flags use the third context, without dependency on neighboring LCUs’ SAO flag.

2.3 JCTVC-I0193 + JCTVC-I0199 + JCTVC-I0184

When JCTVC-I0184 is combined with the method described in Section 2.1, first, the statistics of the previous frame is checked to turn off the whole LCUs in the current slice as in JCTVC-I0184. If it is not turned off, the combined method in Section 2.1 is applied.

# Experimental Results

To evaluate the coding efficiency of the combined methods, the CE1 common test condition [5] is used for the interleaving mode. We evaluated the combined method (i.e. JCTVC-I0193 + JCTVC-I0199) in various points of view. Table 2 summarizes pairs of algorithms tested in this report. Please note that, in this document, test results only for low-delay B configuration are reported. Full results will be available in excel spreadsheets.

First, “JCTVC-I0193 + JCTVC-I0199” is compared with JCTVC-I0199 in Table 3, to see the gain of this combined method over JCTVC-I0199. For luma component, this combined method improves coding gain by 0.0%, 0.2% and 0.9% for LCU size of 64, 32 and 16, respectively, compared to JCTVC-I0199.

Second, “JCTVC-I0193 + JCTVC-I0199” is compared with JCTVC-I0184 in Table 4, to see the gain of this combined method over JCTVC-I0184. For luma component, this combined method improves coding gain by 0.2%, 0.2% and 0.0% for LCU size of 64, 32 and 16, respectively, compared to JCTVC-I0184.

Third, “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I0184 is compared with JCTVC-I0184 in Table 5, to see the additional gain by “JCTVC-I0193 + JCTVC-I0199” when it is implemented on top of JCTVC-I0184. For luma component, “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I084 improves coding gain by 0.2%, 0.3% and 0.4% for LCU size of 64, 32 and 16, respectively, compared to JCTVC-I0184.

Finally, “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I084 is compared with “JCTVC-I0193 + JCTVC-I0199” in Table 5, to see the additional gain by JCTVC-I0184, when it is implemented on top of “JCTVC-I0193 + JCTVC-I0199”. For luma component, “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I084 improves coding gain by 0.0%, 0.1% and 0.4% for LCU size of 64, 32 and 16, respectively, compared to “JCTVC-I0193 + JCTVC-I0199”.

Table 2. Descriptions of pairs of algorithms compared in Tables 3 ~ 6.

| Table No. | Test Algorithm | Anchor Algorithm | Luma BD-Rate Change* (LCU 64/32/16) |
| --- | --- | --- | --- |
| Table 3 | “JCTVC-I0193 + JCTVC-I0199” | JCTVC-I0199 | 0.0% / -0.2% / -0.9% |
| Table 4 | “JCTVC-I0193 + JCTVC-I0199” | JCTVC-I0184 | -0.2% / -0.2% / 0.0% |
| Table 5 | “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I0184 | JCTVC-I0184 | -0.2% / -0.3% / -0.4% |
| Table 6 | “JCTVC-I0193 + JCTVC-I0199” + JCTVC-I0184 | “JCTVC-I0193 + JCTVC-I0199” | 0.0% / -0.1% / -0.4% |

* BD-Rate Change: Negative value means gain. It is for low-delay B Main configuration.

Table 3. BD-rate results of the combined method, “JCTVC-I0193 + JCTVC-I0199” compared to JCTVC-I0199 (Anchor: JCTVC-I0199, Test: “JCTVC-I0193 + JCTVC-I0199”).

- LCU 64x64

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

- LCU 32x32

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class C | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.9% | -0.7% | -0.8% | -1.0% | -0.8% | -0.8% |
| Overall | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
|  | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
| Class F | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |

- LCU 16x16

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -0.6% | -0.6% | -0.7% | -0.6% | -0.6% |
| Class C | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class D | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% | -0.2% |
| Class E | -3.2% | -2.9% | -2.8% | -3.2% | -2.8% | -2.8% |
| Overall | -0.9% | -0.8% | -0.8% | -1.0% | -0.8% | -0.8% |
|  | -0.9% | -0.8% | -0.8% | -1.0% | -0.9% | -0.8% |
| Class F | -1.3% | -1.1% | -1.1% | -1.4% | -1.2% | -1.2% |

Table 4. BD-rate results of the combined method, “JCTVC-I0193 + JCTVC-I0199” compared to JCTVC-I0184 (Anchor: JCTVC-I0184, Test: “JCTVC-I0193 + JCTVC-I0199”).

- LCU 64x64

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.7% | -0.8% | -0.1% | -0.1% | -0.1% |
| Class C | -0.2% | -0.5% | -0.3% | -0.1% | -0.3% | -0.1% |
| Class D | -0.2% | -0.3% | -1.2% | -0.2% | 0.1% | -0.3% |
| Class E | -0.4% | -1.0% | 0.1% | -0.3% | 0.0% | -0.7% |
| Overall | -0.2% | -0.6% | -0.6% | -0.2% | -0.1% | -0.3% |
|  | -0.2% | -0.6% | -0.6% | -0.2% | -0.1% | -0.3% |
| Class F | -0.9% | -0.4% | -0.6% | -0.4% | -0.1% | -0.1% |

- LCU 32x32

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.1% | 0.2% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Class D | -0.1% | -0.1% | 0.3% | -0.3% | -0.2% | 0.6% |
| Class E | -0.3% | -0.1% | -0.6% | -0.3% | -0.1% | -0.6% |
| Overall | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% | 0.1% |
|  | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | 0.0% |
| Class F | -0.8% | -0.3% | -1.2% | -0.3% | -0.3% | -0.1% |

- LCU 16x16

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.6% | 0.1% | 0.0% | 0.4% |
| Class C | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | 0.3% |
| Class D | -0.1% | 0.2% | -0.1% | -0.2% | 0.4% | 0.5% |
| Class E | 0.1% | 0.3% | 0.3% | 0.2% | 0.1% | 0.0% |
| Overall | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
|  | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Class F | 0.1% | 0.3% | 1.0% | 0.1% | 1.4% | 0.7% |

Table 5. BD-rate results of “JCTVC-I0193 + JCTVC-I0199” + I0184 compared to JCTVC-I0184 (Anchor: JCTVC-I0184, Test: “JCTVC-I0193 + JCTVC-I0199” + I0184).

- LCU 64x64

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.5% | -0.9% | -0.1% | -0.1% | -0.3% |
| Class C | -0.1% | -0.4% | -0.3% | -0.1% | -0.3% | -0.2% |
| Class D | -0.2% | -0.4% | -0.6% | -0.2% | 0.0% | -0.5% |
| Class E | -0.6% | -1.5% | -0.8% | -0.4% | -0.8% | -0.7% |
| Overall | -0.2% | -0.6% | -0.6% | -0.2% | -0.2% | -0.4% |
|  | -0.2% | -0.6% | -0.7% | -0.2% | -0.2% | -0.5% |
| Class F | -0.3% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% |

- LCU 32x32

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.3% | -0.2% | 0.0% | -0.3% |
| Class C | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | -0.4% | 0.0% | -0.2% | 0.0% | 0.6% |
| Class E | -0.8% | -0.9% | -1.4% | -0.8% | -0.7% | -0.8% |
| Overall | -0.3% | -0.4% | -0.4% | -0.3% | -0.2% | -0.1% |
|  | -0.3% | -0.3% | -0.4% | -0.3% | -0.1% | -0.1% |
| Class F | -0.3% | -0.1% | -1.1% | -0.4% | -0.9% | -1.5% |

- LCU 16x16

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | 0.3% | -0.2% | -0.3% | 0.2% |
| Class C | -0.2% | -0.1% | -0.3% | -0.3% | 0.0% | 0.0% |
| Class D | -0.3% | 0.0% | 0.2% | -0.3% | 0.3% | 0.4% |
| Class E | -1.2% | -1.0% | -1.1% | -1.1% | -1.1% | -1.0% |
| Overall | -0.4% | -0.3% | -0.1% | -0.4% | -0.2% | 0.0% |
|  | -0.4% | -0.3% | -0.1% | -0.4% | -0.2% | -0.1% |
| Class F | -0.4% | 0.0% | 0.0% | -0.4% | -0.3% | -0.5% |

Table 6. BD-rate results of “JCTVC-I0193 + JCTVC-I0199” + I0184 compared to JCTVC-I0193 + JCTVC-I0199” (Anchor: JCTVC-I0193 + JCTVC-I0199”, Test: “JCTVC-I0193 + JCTVC-I0199” + I0184).

- LCU 64x64

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.6% | 0.0% | -0.1% | -0.2% |
| Class E | -0.2% | -0.5% | -0.9% | -0.1% | -0.7% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class F | 0.6% | 0.2% | 0.5% | 0.3% | 0.1% | 0.1% |

- LCU 32x32

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | -0.4% | -0.1% | 0.0% | -0.4% |
| Class C | 0.0% | -0.2% | -0.2% | -0.1% | 0.0% | -0.1% |
| Class D | -0.1% | -0.3% | -0.3% | 0.1% | 0.1% | 0.0% |
| Class E | -0.5% | -0.8% | -0.7% | -0.5% | -0.6% | -0.2% |
| Overall | -0.1% | -0.3% | -0.4% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% |
| Class F | 0.5% | 0.2% | 0.1% | -0.1% | -0.6% | -1.4% |

- LCU 16x16

|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.2% |
| Class C | -0.2% | 0.0% | -0.3% | -0.1% | 0.0% | -0.3% |
| Class D | -0.2% | -0.2% | 0.3% | -0.1% | 0.0% | -0.2% |
| Class E | -1.4% | -1.3% | -1.3% | -1.3% | -1.2% | -1.0% |
| Overall | -0.4% | -0.4% | -0.3% | -0.4% | -0.3% | -0.4% |
|  | -0.4% | -0.4% | -0.3% | -0.4% | -0.3% | -0.4% |
| Class F | -0.6% | -0.3% | -0.9% | -0.6% | -1.6% | -1.2% |

# Conclusion

In this document the coding efficiency of the SAO on/off signaling method is investigated, which is combinations of JCTVC-I0193, JCTVC-I0199, and JCTVC-I0184. Experimental results show that the combined method of JCTVC-I0193 + JCTVC-I0199 shows improved coding efficiency compared to JCTVC-I0199.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1003, Feb. 2012.

- W.-S. Kim, D.-K. Kwon, I. S. Chong, and M. Karczewicz, “Non-CE1: LCU SAO enable flag coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0193, Apr., 2012.

- C.-W. Hsu, C.-M. Fu, T.-D. Chuang, Y.-W. Huang, S. Lei, W.-S. Kim, D.-K. Kwon, I. S. Chong, and M. Karczewicz, “Non-CE1: Decoupling SAO on/off from SAO type with neighbor-based contexts”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0199, Apr., 2012.

- G. Laroche, T. Piorier, and P. Onno, “Non-CE1: Encoder modification for SAO interleaving mode”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I0184, Apr., 2012.

- Y.-W. Huang, E. Alshina, I. S. Chong, W. Wan, and M. Zhou, “Description of Core Experiment 1 (CE1): Sample adaptive offset filtering”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H1101, Feb., 2012.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).