| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I1101 |
| --- | --- |

| Title: | Description of Core Experiment 1: Intra transform mode dependency simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | K. Ugur A. Saxena | Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com asaxena@sta.samsung.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This document is a description of Core Experiment 1: Intra transform mode dependency simplifications.

# Introduction

During the 9th JCT-VC meeting, JCTVC-I0582 proposed some simplifications for DCT-DST adaptation in HEVC. This CE evaluates the performance of DST and the potential simplifications proposed in JCTVC-I0582.

# Participants

| Participant | Person, Email address |  | P | C |
| --- | --- | --- | --- | --- |
| Nokia | K. Ugur O. Bici J. Lainema | kemal.ugur@nokia.com ext-oguz.bici@nokia.com jani.lainema@nokia.com | X |  |
| Samsung | A. Saxena E. Alshina | asaxena@sta.samsung.com elena.alshina@gmail.com |  | X |
| Microsoft | Ji-Zheng Xu | jzxu@microsoft.com |  | X |
| I2R | C. Yeo | chyeo@i2r.a-star.edu.sg |  |  |
| Docomo | F. Bossen T.K. Tan | bossen@docomoinnovations.com tktan@m-sph.com |  |  |
| Motorola | K. Panusopone | krit@motorola.com |  |  |
| TI | M. Zhou M. Budagavi | zhou@ti.com madhukar@ti.com |  | X |
| Qualcomm | R. Joshi | rajanj@qualcomm.com |  | X |
| Mediatek | S. Lei Y. Huang | shawmin.lei@mediatek.com yuwen.huang@mediatek.com |  | X |
| MERL | R. Cohen | cohen@merl.com |  | X |
| Broadcom | W. Wan | wwan@broadcom.com |  |  |
| Sony | A. Tabatabai C. Auyeung | Ali.Tabatabai@am.sony.com Cheung.Auyeung@am.sony.com |  |  |
| HHI | B. Bross | benjamin.bross@hhi.fraunhofer.de |  |  |
| Canon | P. Onno G. Laroche E. Francois | Patrice.onno@crf.canon.fr Guillaume.laroche@crf.canon.fr Edouard.francois@crf.canon.fr |  |  |
| RIM | G. Martin-Cocher | gmartincocher@rim.com |  |  |
| NEC | K. Chono | chono@ct.jp.nec.com |  |  |
| ETRI | H. Y. Kim, Sung-Chang Lim J. Lee K. Y. Kim | hykim5@etri.re.kr sclim@etri.re.kr jinosoul@etri.re.kr kimky@khu.ac.kr |  |  |
| Intel | Yi-jen Chiu W. Zhang L. Xu | yi-jen.chiu@intel.com wenhao.zhang@intel.com lidong.xu@intel.com |  |  |
| Toshiba | A. Tanizawa | akiyuki.tanizawa@toshiba.co.jp |  |  |
| BBC | M. Mrak | Marta.Mrak@bbc.co.uk |  |  |

# Tests

Two simplifications will be tested:

## Simplification 1

- DST for both horizontal and vertical directions are used for all intra prediction modes of luma intra 4x4 TUs.

Specification text changes over HEVC CD:

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are set equal to 1. specified as Table 8-2 with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## Simplification 2

- For Intra_DC mode of luma intra 4x4 TUs, DCT for both horizontal and vertical directions are used.

- For all other prediction modes of luma intra 4x4 TUs, DST for both horizontal and vertical directions are used.

Specification text changes over HEVC CD:

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, Log2(nW*nH) is equal to 4, and cIdx is equal to 0, and IntraPredMode is not equal to INTRA_DC the variables horizTrType and vertTrType are set equal to 1. specified as Table 8-2 with IntraPredMode as input.

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-2 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| horizTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

# Experiments

Following are the experiments planned to be conducted within the CE. All experiments will test both simplifications described in Section 3:

Test 1 Configuration:

- Main, All-Intra, Common Conditions QP Range

Tester: Nokia, Cross-Checkers: Samsung, MERL

Test 2 Configuration:

- Main, All Intra, Low QP Range: 2, 7, 12, 17

Tester: Nokia, Cross-Checkers: Qualcomm, TI

Test 3 Configuration :

- Main, All Intra, Common Conditions QP Range, Transform Skipping ON.

Tester: Nokia, Cross-Checker: Microsoft

In addition to objective evaluation in terms of BD-rate, visual evaluation is planned to be conducted at the 10th JCT-VC meeting.

# Timeline

T1: 07-May-2012: Final CE description and test condition definition

T2: 22-May-2012: HM-7.0 available

T3: 5-June-2012: Completion of tool integration. Start of cross-check

T4: 27-June-2012: Input document and summary report uploaded.