| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0009 |
| --- | --- |

| Title: | JCT-VC AHG report: High-level syntax (AHG9) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan (AHG meeting co-chair) Ye-Kui Wang (AHG chair & AHG meeting co-chair) Jill Boyce Ying Chen Miska M. Hannuksela Kimihiko Kazui Thomas Schierl Rickard Sjöberg T. K. Tan Wade Wan Ping Wu (AHG vice-chairs) | Email: | garysull@microsoft.com yekuiw@qualcomm.com jill@vidyo.com cheny@qualcomm.com miska.hannuksela@nokia.com kazui.kimihiko@jp.fujitsu.com thomas.schierl@hhi.fraunhofer.de rickard.sjoberg@ericsson.com ttk@pacific.net.sg wwan@broadcom.com pwuzte@gmail.com |
| Source: | High-level syntax AHG (AHG9) |  |  |

_____________________________

# Abstract

This report summarizes the activities of the high-level syntax ad hoc group (AHG9) between the 9th JCT-VC meeting and the 10th JCT-VC meeting, and lists the related input documents submitted to this meeting according to their topics.

A face-to-face AHG meeting was held from 9:00 to 18:00 on July 10, 2012, in the same meeting venue as the 10th JCT-VC meeting. Meeting minutes and AHG recommendations made during the face-to-face meeting are also included in the report.

# Mandates

- Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and slice header syntax designs.

- Study SEI messages and VUI syntax designs, including checking and fixing texts for SEI messages currently included by referring to the AVC specification.

- Study the hypothetical reference decoder (HRD) syntax and operations, including making sure that the correct use of max_dec_pic_buffering[ i ] in the HDR text, as well as the text for bitstream conformance and decoder conformance.

- Work towards simplification and general minor cleanup of the high-level syntax.

- Assist in software development and text drafting for the high-level syntax in the HEVC design.

# Reflector discussions

There were some discussions on the following topics:

- Constraint on number of bits per coding tree block

- Usefulness of restricted_ref_pic_lists_flag and related

- Slice granularity and end_of_slice_flag

There are input documents to this meeting addressing the issues related to the first topics.

# Related contributions

High-level syntax related contributions registered by July 5, 2012 to this JCT-VC meeting are listed below, according to the topics. Documents that are obviously more belonging to other high-level syntax closely-related AHGs (than AHG11), namely the following, are not listed herein:

- AHG4: High-level parallelism

- AHG10: Hooks for scalable coding

- AHG13: Reference picture buffering and list construction

The AHG9 input documents are allocated to different topics as follows, wherein the numbers indicates the numbers of documents of the corresponding topic categories:

- AHG9 (High-level syntax): 59

  - NUH (NAL unit header): 6

  - RAP (random access point): 6

  - Splicing (and editing): 2

  - TLA (temporal layer access): 3

  - POC (picture order count): 3

  - Slice: 5

  - RPL&WP (reference picture list and weighted prediction): 6 (incl. 1 cross-check)

  - APS (adaptation parameter set): 1

  - MVP (motion vector prediction related): 2

  - Misc.: 1

  - Cleanups: 5

  - HRD: 3

  - SEI (supplemental enhancement information): 8

    - FPA (frame packing arrangement): 4

    - Other SEIs: 4

  - P&L (profile and level related, particularly level related in AHG9): 8

    - DPB size (6)

    - Other limits (2)

The list AHG9 input documents is as follows, sorted according to firstly category and secondly JCT-VC number:

| JCT-VC number | Category | Title | Authors |
| --- | --- | --- | --- |
| JCTVC-J0039 | AHG4 tiles AHG9 | AHG4/AHG9: Syntax for restricting slices and tiles | C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek) |
| JCTVC-J0042 | AHG4 tiles AHG9 | AHG4/AHG9: Syntax modifications for tile width constraint | C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek) |
| JCTVC-J0085 | AHG4 tiles AHG9 | AHG9: On number of tile rows limit | M. Zhou(TI) |
| JCTVC-J0235 | AHG4 tiles AHG9 | AHG4/AHG9: On tile size restrictions in HEVC | A. Fuldseth (Cisco) |
| JCTVC-J0032 | AHG4 WPP | An HEVC transcoder converting non-parallel bitstreams to/from WPP | G. Clare, F. Henry |
| JCTVC-J0040 | AHG4 WPP AHG9 | AHG4/AHG9: Syntax for restricting slices and WPP | C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek) |
| JCTVC-J0072 | AHG9 APS | AHG9 High-Level Syntax: APS loss detection | M. M. Hannuksela (Nokia) |
| JCTVC-J0183 | AHG9 cleanup | Syntax Issues | K Sato (Sony) |
| JCTVC-J0220 | AHG9 cleanup | cu qp delta enabling syntax | T. Lee, J. Park (Samsung) |
| JCTVC-J0273 | AHG9 cleanup | A simple ordering issue for VUI parameters syntax | Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki |
| JCTVC-J0300 | AHG9 cleanup | Slice Header Syntax Cleanup | Yue Yu, Jian Lou, Limin Wang(Motorola Mobility) |
| JCTVC-J0047 | AHG9 cleanup AHG6 | AHG6/AHG9: Cleanups and fixes for APS | C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-J0136 | AHG9 HRD | AHG9: Improvement of HRD for sub-picture based operation | K Kazui, J Koyama, S Shimada, A Nakagawa (Fujitsu) |
| JCTVC-J0272 | AHG9 HRD | Simplifications of HRD parameters for Temporal Scalability | Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki |
| JCTVC-J0306 | AHG9 HRD | AHG10: On Sub-picture Based CPB | S. Deshpande (Sharp) |
| JCTVC-J0290 | AHG9 misc. | High layer syntax issues | C. Fogg (Harmonic), A. Wells (Ambarella) |
| JCTVC-J0071 | AHG9 MVP | High-level Syntax: Motion vector prediction issue for long-term reference picture | Y. Takahashi, O. Nakagami, T. Suzuki (Sony) |
| JCTVC-J0187 | AHG9 MVP | On temporal_mvp_enable_flag | K Sato (Sony) |
| JCTVC-J0063 | AHG9 NUH | AHG9: Syntax for NAL Packet Priority | Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He (InterDigital) |
| JCTVC-J0231 | AHG9 NUH | On nal_ref_flag | TK Tan, Junya Takiue (NTT DOCOMO) |
| JCTVC-J0250 | AHG9 NUH | Indication of non-reference pictures | R. SjÃ¶berg, J. Samuelsson (Ericsson) |
| JCTVC-J0174 | AHG9 NUH AHG10 | AHG 9 / AHG 10: On NAL unit header | T. Thang (UoA), J. Kang, H. Lee, J. Lee (ETRI), Hendry, B. Jeon (LG) |
| JCTVC-J0112 | AHG9 NUH misc. AHG10 | AHG9: Various comments on HEVC draft 7 | Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-J0080 | AHG9 P&L | AHG9: On number of slices per picture limit | M. Zhou (TI) |
| JCTVC-J0082 | AHG9 P&L | AHG9: On number of bits per LCU limit | M.Zhou, V. Sze (TI) |
| JCTVC-J0111 | AHG9 P&L DPB | AHG9: On DPB size | R. L. Joshi, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCTVC-J0163 | AHG9 P&L DPB | AHG9: On DPBsize reduction | S. Lu, T. Suzuki (Sony) |
| JCTVC-J0199 | AHG9 P&L DPB | AHG 9: Comments on DPB Size | T. Davies, A. Fuldseth (Cisco) |
| JCTVC-J0210 | AHG9 P&L DPB | MaxDpbSize setting based on the picture size | A.Fujibayashi, T.K.Tan(NTT DOCOMO) |
| JCTVC-J0283 | AHG9 P&L DPB | Field Coding and MaxDpbSize constraints | D. Flynn, A. Gabriellini, M. Mrak, M. Naccari (BBC) |
| JCTVC-J0367 | AHG9 P&L DPB | On level considerations for interlaced content coding | JÃ©rÃ´me ViÃ©ron, Pierre Larbier, Jean-Marc Thiesse (ATEME) |
| JCTVC-J0084 | AHG9 POC | AHG9: Restrict Picture Order Count to 40-bit | M. Zhou (TI) |
| JCTVC-J0110 | AHG9 POC | AHG9: On POC | R. L. Joshi, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-J0248 | AHG9 POC | On prevRefPic definition | J. Samuelsson, R. SjÃ¶berg (Ericsson) |
| JCTVC-J0107 | AHG9 RAP | AHG9: On RAP pictures | Y.-K. Wang, Y. Chen, R. L. Joshi, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-J0215 | AHG9 RAP | AHG 9: On NAL unit type | Hendry, B. Jeon (LG) |
| JCTVC-J0229 | AHG9 RAP | AHG9: Comments and clarification on CRA, BLA and TFD pictures | TK Tan (NTT DOCOMO) |
| JCTVC-J0251 | AHG9 RAP | Restrictions on leading pictures of CRA and BLA | J. Samuelsson, R. SjÃ¶berg (Ericsson) |
| JCTVC-J0310 | AHG9 RAP | Revival of decodable backward predicted pictures that are output preceding a RAP picture | Arturo Rodriguez (Cisco), A. K Katti, H-Y Hwang |
| JCTVC-J0344 | AHG9 RAP | Refinement of random access point support | S. Kanumuri, G. J. Sullivan (Microsoft) |
| JCTVC-J0345 | AHG9 RAP | Editorial modifications to HEVC text specification relating to reference picture sets and random access points | G. J. Sullivan, S. Kanumuri (Microsoft) |
| JCTVC-J0055 | AHG9 RPL&WP | AHG9: Weighted Prediction Parameter Signalling | Yong He, Yan Ye (InterDigital) |
| JCTVC-J0221 | AHG9 RPL&WP | AHG9: Clean-up of semantics and decoding process on weighted prediction | A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba) |
| JCTVC-J0222 | AHG9 RPL&WP | AHG9: Improved weighted prediction parameter signaling | A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba) |
| JCTVC-J0252 | AHG9 RPL&WP | AHG9: Cross-check of JCTVC-J0221: Clean-up of semantics and decoding process on weighted prediction | P.Bordes, P.Andrivon (Technicolor) |
| JCTVC-J0120 | AHG9 RPL&WP AHG13 | AHG9 & AHG13: Identical reference picture lists | Y. Chen, M. Coban, Y.-K. Wang, M. Karczewicz (Qualcomm) |
| JCTVC-J0182 | AHG9 RPL&WP AHG13 | AHG 9 / AHG 13: On identical reference picture lists | Hendry, Y. Jeon, S. Park, B. Jeon (LG) |
| JCTVC-J0038 | AHG9 SEI | Exif data SEI message | K. Ugur, J. Lainema, M. Hannuksela (Nokia) |
| JCTVC-J0064 | AHG9 SEI | SEI messages for gradual decoding refresh | H. Aoki, K. Chono (NEC) |
| JCTVC-J0149 | AHG9 SEI | Signalling of Luminance Dynamic Range in Tone mapping information SEI | S. Hattori, T. Suzuki (Sony) |
| JCTVC-J0245 | AHG9 SEI | SEI Message for profile and level signaling for temporal scalability and extensions | J. Boyce, D. Hong, W. Jang |
| JCTVC-J0070 | AHG9 SEI FPA | 2D compatible frame packing arrangement SEI | O. Nakagami, T. Suzuki (Sony) |
| JCTVC-J0073 | AHG9 SEI FPA | AHG9 High-Level Syntax: Graceful Handling of Frame-Packed Pictures by Ignorant Decoders | M. M. Hannuksela (Nokia) |
| JCTVC-J0198 | AHG9 SEI FPA | AHG9: Frame packing arrangement SEI message modification | M. Arena, P. Sunna |
| JCTVC-J0247 | AHG9 SEI FPA | AHG9: On frame packing arrangement SEI | B. Choi, C. Kim, J. Kim, J. Park (Samsung) |
| JCTVC-J0083 | AHG9 slice | AHG9: On slice header parsing overhead reduction | M. Zhou, M. Mody (TI) |
| JCTVC-J0109 | AHG9 slice | AHG9: Header parameter set (HPS) | Y. Chen, Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia) |
| JCTVC-J0216 | AHG9 slice | AHG 9: Signalling slice index to detect lost slice earlier | Hendry, B. Jeon (LG) |
| JCTVC-J0217 | AHG9 slice | On dependent slices | T. Lee, J. Park (Samsung) |
| JCTVC-J0255 | AHG9 slice | AHG9: Slice Prefix for sub-picture and slice level HLS signalling | T. Schierl, V. George, R. Skupin, D. Marpe (HHI) |
| JCTVC-J0108 | AHG9 splicing | AHG9: Splicing-friendly coding of some parameters | Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-J0137 | AHG9 splicing | AHG9: New high-level syntax for simple HEVC stream editing | K Kazui, J Koyama, S Shimada, A Nakagawa ((Fujitsu) |
| JCTVC-J0246 | AHG9 TLA | AHG10: On temporal layer access pictures | B. Choi, Y. Park, I. Kim, J. Kim, J. Park (Samsung) |
| JCTVC-J0305 | AHG9 TLA | AHG10: On Gradual Temporal Layer Access | S. Deshpande (Sharp) |
| JCTVC-J0156 | AHG9 TLA AHG10 | AHG 9/10: Generalized definition of the TLA for scalable extension | C. Kim, Hendry, B.Jeon (LG) |

# Meeting minutes and recommendations made during the face-to-face AHG meeting

The AHG held a meeting from 0900‒1800 on Tuesday 10 July 2012 at the Conference Center venue of 10th JCT-VC meeting that was to begin on the following day. The AHG meeting was chaired by Ye-Kui Wang and Gary Sullivan. Due to lack of time, the AHG only reviewed contributions in the following three topic areas:

- NAL unit header (4 contributions)

- Picture order count (3 contributions)

- Slices (6 contributions)

Detailed notes of the discussion in each of these areas are found in the following sections. Certain key aspects are highlighted with yellow highlighting for the review and attention of the JCT-VC.

## NAL unit header

J0063, J0231, J0250, J0112, J0113, and J0174 were all suggested to be related. Only the first four of these were discussed in the AHG meeting.

JCTVC-J0063 AHG9: Syntax for NAL Packet Priority [Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He (InterDigital)]

Pictures in the same temporal level in a hierarchical-B structure can have different influence on error propagation and on decoded video quality. Currently in HEVC draft 7, the NAL unit header does not indicate packet priority within the same temporal layer. This contribution proposes two syntax options to indicate such priority of a NAL unit.

R. Sjoberg expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was remarked that the prioritization may depend on the loss concealment method.

It was noted that the nal_ref_flag is basically always currently equal to 1 except at the highest temporal layer.

"Method 1" would change nal_ref_flag to be a nal_priority_flag indicating relative priority within a temporal level.

"Method 2" would provide a priority_id in the AUD, which could carry more bits than the nal_ref_flag.

It was noted that there would be no normative purpose for the proposed priority_id – that it is just metadata – and could be an SEI message.

Some participants expressed some skepticism about the value of the proposed priority_id.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0231 On nal_ref_flag [T. K. Tan, J. Takiue (NTT Docomo)]

This contribution seeks to clarify the purpose of the nal_ref_flag.  The nal_ref_flag does not seem to have any use in the decoding process apart from the final marking of the picture as reference or non-reference picture.

This contribution proposes to remove the nal_ref_flag syntax element from the NAL unit header and create 3 new NAL unit types for coded slices that can be either used for reference or not.

J0463 is reportedly a (late) cross-check.

It was suggested that an alternative would be to use the maximum temporal later value as a non-reference picture indication, rather than using the NUT or nal_ref_flag.

This (or something like it) seemed promising.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0250 Indication of non-reference pictures [R. Sjöberg, J. Samuelsson (Ericsson)]

This contribution proposes to change the semantics of nal_ref_flag so that in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer. This contribution claims that this makes it possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed and thus indicate whether that picture can safely be removed from the sub-stream without affecting the decoding of the remaining pictures in the sub-bitstream.

It was commented that this would make it more difficult for a "middle box" to identify pictures that can be dropped. Unless the highest temporal ID in the bitstream is known, the non-reference pictures cannot be dropped.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0112 AHG9: Various comments on HEVC draft 7 [Y.-K. Wang (Qualcomm)]

This document proposes the following:

- Removal of nal_ref_flag, and push the saved bit to reserved_one_5bits to make it to reserved_one_6bits; this proposal is suggested to be ignored if the proposal in JCTVC-J0113 is adopted

- Change of temporal_id to temporal_id_plus1 (The DPB size in level definitions only support hierarchical coding structures with GOP size up to 16 (see document JCTVC-J0111 for analyses of DPB size requirements for different GOP sizes), i.e., typically up to 5 temporal layers are supported.) However, it was commented that there may be a desire in the future to use deeper temporal nesting.

- Change of the value 0 of nal_unit_type from "Unspecified" to "Reserved".

- An alleged editorial fix to the semantics of the extension syntax elements in VPS, SPS, PPS, APS and slice data. This was not agreed. However, it was agreed that some further clarification of the intended tolerance of decoders for reserved values would be desirable.

- Extending of the extension mechanism to all types of NAL units. The participants did not support this part of the proposal.

- To support SEI NAL units that may follow the first VCL NAL unit in the same access unit

Additional discussion of this is needed in the JCT-VC meeting.

## Picture order count (POC) (3)

JCTVC-J0084 AHG9: Restrict Picture Order Count to 40-bit [M. Zhou (TI)]

At the 9th JCTVC meeting in Geneva, the dynamic range of picture order count (PicOrderCntVal) was increased from 32-bit to 64-bit. 64-bit PicOrderCntVal can support continuous 120 fps video recording for up to 4874.5 million years. However, on the decoder side there is certain complexity associated with carriage of 64-bit PicOrderCntVal. It is therefore recommended to restrict PicOrderCntVal to 40 bits, which can already support continuous 120 fps video recording for up to 290.5 years.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

At the previous meeting, we thought there was no real impact to the increase of the specified range.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0110 AHG9: On POC [R. L. Joshi, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

(AHG discussion of this was chaired by G. Sullivan and J. Boyce)

This document proposes the following:

- An editorial change to the definition of picture order count to discuss specific cases

- Removal of the function DifPicOrderCnt( ) or consistently using it in the specification, particularly for derivation processes for motion vectors and reference indices

- Removal of one of the POC-related constraints in Annex C.4

- An SEI message conveying additional POC information for intra pictures to enable derivation of output order for intra-picture-only trick mode playback

(Cross check not yet provided.)

Discussion of each item:

Topic 1. Editorial only. Something needs to be changed. Editors can consider this (and where this should go).

Topic 2. Notes that the POC difference computation function is not always used, and thus the constraint on POC difference is not applied everywhere that POC differences are computed. It may be desirable to impose the constraint when differences are computed for motion vector derivation, but perhaps not for RPS difference values (e.g. for an LTRP in the RPS). For further study.

Topic 3. About prevRefPic, it was commented that there seems to be an editing error, as our intent was to include prevRefPic in the set of pictures for which max and min POC counts are computed and the difference range constraint is imposed, but it was not put into the text that way. J. Boyce assessed the consensus that this was agreed and should be corrected.

The contribution suggested to include only prevRefPic and the current picture, and not to also include pictures in the DPB that are marked as used for short-term reference or waiting for output. J. Boyce assessed that there was no consensus to make this change.

A general comment was made that bitstream constraints are useful for early detection of bitstream errors.

Topic 4. Proposed new SEI message for intra pictures.

It was commented that the encoder could just send more POC LSBs to ensure coherence. It was responded that the presence of this SEI would provide an assurance that the issue was taken care of.

It was asked whether there could be a splicing problem with this – e.g., when discarding some GOPs so that a new bitstream starts at a CRA. It was suggested that sending the POC MSB difference rather than some POC MSB's value might be better.

It was asked how to detect an "intra picture".

It was commented that all RAP pictures are already in decoding order in the bitstream, so this may not be needed.

It was commented that non-RAP intra pictures would not be expected to ordinarily be used.

The AHG recommended this to be for further study with no action to be taken at this meeting.

JCTVC-J0248 On prevRefPic definition [J. Samuelsson, R. Sjöberg (Ericsson)]

This contribution proposes to revert the definition of prevRefPic to always be a picture in temporal layer 0, for the sake of simplicity and to correct a problem with TLA pictures having POC values that depend on the presence of prior inappropriate pictures.

Instead of using the previous reference picture that has a temporal_id equal to or less than the temporal_id of the current picture for prevRefPic, this document proposes to use the previous reference picture that has a temporal_id equal to 0.

(Cross check not yet provided.)

It was agreed that the TLA problem exists and needs to be corrected.

The AHG recommended adoption.

## Slices (6)

JCTVC-J0083 AHG9: On slice header parsing overhead reduction [M. Zhou, M. Mody (TI)]

In the current HM7.0 design there is reportedly a large difference (over 100x) in slice header parsing overhead between the typical and worst case. It was asserted that the worst case slice header parsing can be beyond capability of real-time decoding in which slice header parsing is implemented in software. Weighted prediction tables and reference picture parameter sets (RPS) are parsing intensive parts of slice header. To reduce the worst case slice header parsing overhead, this contribution proposes the following two changes: 1) move RPS from slice header to APS, 2) reduce weighted prediction table cycle overhead by either constraining the size of the reference picture lists (option 1), or enabling signalling of default weighted prediction tables in APS and limiting the number of slice headers per picture which can override weighted prediction tables signalled in APS (option 2). The proposed methods can reportedly reduce the worst case slice parsing overhead by roughly 2x to 3x based on cycle estimate on ARM9.

It was remarked that slice header sharing (e.g. J0109) is related.

(Cross check not yet provided.)

It was commented that since the RPS is the same in all SHs, the decoder can just skip over those bits after parsing them from the first SH.

The weighted prediction syntax aspect needs additional discussion (e.g. "option 1").

JCTVC-J0109 AHG9: Header parameter set (HPS) [Y. Chen, Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia)]

(AHG discussion of this was chaired by G. Sullivan and J. Boyce)

At the previous JCT-VC meeting, the proposal on slice header prediction in JCTVC-I0070 was discussed. It was noted in the meeting notes that using some kind of parameter set to enable slice header prediction seemed promising. Following such a direction, this document proposes a slice header prediction mechanism based on a so-called header parameter set (HPS), with two slightly different alternative approaches. In the first approach (single-AU HPS), an HPS would be required to be used only within one access unit. In the second approach (multi-AU HPS), an HPS may be used by multiple access units.

It is asserted that the proposed mechanism avoids the drawbacks in the JCTVC-I0070 design, the single-AU HPS approach is asserted to provide similar coding gains as reported in JCTVC-I0070, and the multi-AU HPS approach is asserted to provide opportunities for coding gains with increased complexity.

S. Wenger expressed his plan to submit a cross-check.

This was designed in a similar manner in concept to the previously-proposed APS partial update – multiple HPS IDs are proposed to be carried – one for each of several categories of syntax elements such as RPS selection, prediction weight selection, etc. A flag is proposed for single-slice encoding without using the scheme.

It was commented that the carrying of the multiple IDs seemed a bit complicated.

It was commented that the APS could perhaps be used for this.

The motivation is to provide coding efficiency improvement for multi-slice sharing of header data. There was discussion of loss resilience, but this did not seem to be a significant motivation.

With 6x6 CTB tiles and one slice per tile, the coding efficiency benefit was reportedly approximately 1.5% when reported in a prior contribution (I0070).

Aside from coding efficiency improvement, extensibility for SVC & 3D usage was suggested as a motivation.

It was commented that getting a better understanding of the coding efficiency impact would be needed.

Additional discussion needed in JCT-VC meeting.

JCTVC-J0216 AHG 9: Signalling slice index to detect lost slice earlier [Hendry, B. Jeon (LG)]

(AHG discussion of this was chaired by Y.-K. Wang and G. Sullivan)

This contribution discusses the possibility of detecting lost slices earlier when multiple slices are used per picture. The following changes are proposed:

- Replacing the current syntax element first_slice_in_pic_flag (coded with u(1)) with slice_idx_in_pic (coded with ue(v)).

- Adding a flag last_slice_in_pic_flag (coded with u(1)).

Coding efficiency results without loss reportedly show that the proposed modification affects BD-rate on average 0.1%Y, 0.1%U, 0.1%V for all intra and random access cases (AI-HE10, AI-Main, RAHE10, RA-Main), and 0%Y, 0%U, 0%V for low delay B cases.

It was asked how it would be useful to know, e.g., that the 2nd and 3rd slices were lost (without knowing which CTBs were in those slices until parsing the 1st slice).

It was commented that if a back-channel was available, the slice ID number might be useful to send.

It was commented that the system layer would ordinarily provide a packet counter functionality.

No action on this recommended by the AHG.

(Cross check not yet provided.)

JCTVC-J0416 AHG 9: Cross-check of J0216 - Signalling slice index to detect lost slice earlier [A. K. Ramasubramonian (Qualcomm)] [late] [miss]

JCTVC-J0217 On dependent slices [T. Lee, J. Park (Samsung)]

(AHG discussion of this was chaired by Y.-K. Wang and G. Sullivan)

In the previous meeting, dependent slice support was adopted. According to the specification, a dependent slice cannot be used with entropy slice while it can be used with either WPP or tile within a set of pictures. Since entropy slice is regarded as similar tools to WPP or tile to enable parallel processing, it is proposed to enable dependent slice with entropy slice within a set of pictures for consistency. Or alternatively, it is proposed to restrict the usage of dependent slices to be used only if WPP is used.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was commented that entropy slices and dependent slices are somewhat overlapping in functionality, which is why their use is mutually exclusive.

Sub-picture ultra-low-delay was mentioned as a potential motivation for dependent slices (e.g. in combination with tiles or in non-WPP usage). Document J0264 was mentioned as providing further information.

As a syntax cleanup only, conditional coding of dependent_slice_enabled_flag was proposed in the PPS did seem to make sense, and was recommended for adoption by the AHG. No other action on this was recommended by the AHG.

JCTVC-J0255 AHG9: Slice Prefix for sub-picture and slice level HLS signalling [T. Schierl, V. George, R. Skupin, D. Marpe (HHI)]

(AHG discussion of this was chaired by Y.-K. Wang and G. Sullivan)

In this document it was proposed to apply a functionality like SEI messages as well as additional high level syntax items, beyond the ones included in the NAL unit header, on a per slice level basis. Such messages were proposed as a "slice prefix NAL unit". Syntax and semantics of the slice prefix and slice-level/sub-picture SEI messages, use cases for low delay/sub-picture CPB operations, tile signalling and region of interest (ROI) signalling were described.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

Several potential sub-picture-level SEI messages were described.

It was commented that during the design of AVC the possibility was discussed to have slice-specific SEI messages or otherwise not require SEI messages to be at the beginning of the picture, but this was not done (for simplicity) because there was no clear need for it at the time.

It was commented that even today we still have reserved NUTs in AVC that could be used for such a purpose.

It was commented that we could just allow SEI NUT to follow the first VCL NALU of the picture, rather than using a different NUT for the slice level SEI than for the picture-level SEI.

Some participants commented that ultra-low-delay decode could benefit from definition of a timing SEI specification.

A participant suggested to also enable some SEI type of NALU that could follow after the relevant VCL NALU or the AU rather than precede it – e.g., for checksum data.

The general concept was supported by most participants in the AHG, but further discussion by the JCT-VC seemed needed.

# Further recommendations

Besides the recommendations made during the face-to-face AHG meeting, the high-level syntax ad hoc group further recommends:

- To review all related input contributions

- To coordinate with other closely-related AHGs on commonly related contributions, including

  - AHG4: High-level parallelism

  - AHG10: Hooks for scalable coding

  - AHG13: Reference picture buffering and list construction