| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0013 |
| --- | --- |

| Title: | JCT-VC AHG report: Reference picture buffering and list construction (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg KI/EAB/TVV/V 164 80 Stockholm Sweden Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121 USA Hendry #221 Yangjae-dong, Seocho-gu Seoul 137-130 Korea TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +46761153563 rickard sjoberg@ericsson com |
| Source: | AHG 13 |  |  |

_____________________________

# Abstract

This report summarizes the reference picture buffering and list construction ad-hoc activities between the 9th and the 10th JCT-VC meeting, and lists the related input documents to this meeting.

# Mandates

Table 1	- AHG13 mandates

| AHG Title and Email Reflector | Chairs |
| --- | --- |
| Reference picture buffering and list construction (AHG21) (jct-vc@lists.rwth-aachen.de) - Provide source code that enables HM encoding of all test cases described in JCTVC-H0725 and produce anchor data. - Identify and work to resolve issues relating to the draft text description of reference picture handling and list construction, and the associated HM software functionality. - Study possible improvements related to reference picture buffering and list construction. - Study the loss resilience properties of reference picture handling and its support in the HM software (in coordination with the loss resilience AHG). | R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, Y. -K. Wang (vice chairs) |

# Test recommendation for reference picture buffering proposals

## Test recommendation text

The test recommendation for reference picture buffering and list construction proposals was discussed on the reflector. It was decided to include reference picture duplication into test case 2.8 and remove all references to the combined list as that was taken out from the draft standard at the previous JCT-VC meeting. The document was sent out for review on June 18 on the main reflector. No comments were received and that document was uploaded as JCTVC-J0513.

Table 2	- JCTVC-J0513 test cases

| Test case | Description of picture structure |
| --- | --- |
| 2.1 | The JCT-VC 7B random access common condition picture structure |
| 2.2 | Hierarchical 7B with 1 reference picture per list |
| 2.3 | Hierarchical 7B with 2 reference picture per list |
| 2.4 | As 2.3 but additionally using multiple reference pictures in temporal layer 0 |
| 2.5 | Hierarchical 63B with 1 reference picture per list |
| 2.6 | Hierarchical 7B without CRA pictures and long-term pictures |
| 2.7 | Hierarchical 7B with 2 reference picture per list. Not nested. |
| 2.8 | As 2.1 but with reference picture list reordering and duplication |
| 3.1 | The JCT-VC low delay common condition picture structure |
| 3.2 | 1 reference picture low-delay structure with temporal scalability |
| 3.3 | Low-delay with one short-term and two long-term pictures |
| 3.4 | Low-delay using 5 picture pseudo-random adaptive reference picture selection. |
| 3.5 | Low-delay with reference picture list reordering for the first slice in each picture |

## Source code

Anchor source code supporting all test cases was made available on June 19 in the HM-7.0-dev-ahg13 branch. A config file for each test case is available in HM-7.0-dev-ahg13/cfg/JCTVC- I0608/. Most test cases are supported by only config file changes but cases 2.6, 3.3 and 3.4 contain source code changes as well.

Due to lack of time anchors were not generated.

The HM-7.0-dev-ahg13 source code contains bit count functionality. It reports the number of bits spent on reference picture set and reference picture lists in a bit stream. This includes SPS, PPS and slice header syntax. The following slice header syntax elements are included in the count:

IDR

- rap_pic_id

- no_output_of_prior_pics_flag

POC

- pic_order_cnt_lsb

RPS

- short_term_ref_pic_set_sps_flag

- parseShortTermRefPicSet()

- short_term_ref_pic_set_idx

- num_long_term_pics

- delta_poc_lsb_lt

- delta_poc_msb_present_flag

- delta_poc_msb_cycle_lt_minus1

- used_by_curr_pic_lt_flag

num_ref

- num_ref_idx_active_override_flag

- num_ref_idx_l0_active_minus1

- num_ref_idx_l1_active_minus1

modifications

- ref_pic_list_modification_flag_l0

- list_entry_l0

- ref_pic_list_modification_flag_l1

- list_entry_l1

The RPS cost below shows the average percentage of bits that are spent on RPS related syntax. The numbers were calculated by first running each test case on the common condition sequences. The RA sequence set was used for the 2.x test cases and the LD sequence set was used for the 3.x test cases. For each sequence, fixed QPs 23, 27, 32, 37 was used. Note that 1500 byte slices were turned on for the 3.x test cases. The HM-7.0-dev-ahg13 decoder reports "RPS related bits in total: ", this number was divided by the bitstream file size and averaged for all bitstreams in each test case.

Table 3	- RPS bit-cost for the JCTVC-J0513 test cases

| Case | Description of picture structure | QP22 | QP27 | QP32 | QP37 | Avg |
| --- | --- | --- | --- | --- | --- | --- |
| 2.1 | The JCT-VC 7B RA common condition picture structure | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.2T1 | Hierarchical 7B with 1 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.2T1 | Hierarchical 7B with 1 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.3T3 | Hierarchical 7B with 2 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.3T4 | Hierarchical 7B with 2 reference picture per list | 0.0 | 0.0 | 0.1 | 0.2 | 0.1 |
| 2.4 | As 2.3, additionally using multiple ref pictures in t_id 0 | 0.0 | 0.1 | 0.1 | 0.3 | 0.1 |
| 2.5 | Hierarchical 63B with 1 reference picture per list | 0.1 | 0.2 | 0.4 | 0.7 | 0.4 |
| 2.6 | Hierarchical 7B without CRA pictures and long-term pictures | 0.0 | 0.1 | 0.2 | 0.3 | 0.1 |
| 2.7 | Hierarchical 7B with 2 reference picture per list. Not nested. | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 2.8 | As 2.1 but with reference picture list reordering and duplication | 0.0 | 0.1 | 0.1 | 0.2 | 0.1 |
| 3.1 | The JCT-VC low delay common condition picture structure | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 |
| 3.2 | 1 reference picture low-delay structure with temporal scalability | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 |
| 3.3a | LD with 1 short-term and 2 long-term pictures (200ms RTT) | 0.2 | 0.2 | 0.2 | 0.4 | 0.2 |
| 3.3b | LD with 1 short-term and 2 long-term pictures (1000ms RTT) | 0.2 | 0.2 | 0.3 | 0.5 | 0.3 |
| 3.4 | Low-delay using 5 pseudo-random adaptive reference pictures | 0.2 | 0.2 | 0.3 | 0.4 | 0.3 |
| 3.5 | Low-delay with picture list reordering for the first slice in each picture | 0.1 | 0.2 | 0.3 | 0.5 | 0.3 |

# List of AHG related input documents

The following contributions related to this AHG were submitted to the current JCT-VC meeting:

JCTVC-J0115 [A. K. Ramasubramonian, Y.-K. Wang, R. L. Joshi, Y. Chen (Qualcomm)] "AHG13: Signalling of long-term reference pictures in the slice header"

JCTVC-J0116 [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm),C. S. Lim (Panasonic),S. Deshpande (Sharp),Hendry (LG Electronics)] "AHG13: Signalling of long-term reference pictures in the SPS"

JCTVC-J0117 [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)] "AHG13: On inter-RPS prediction"

JCTVC-J0118 [A. K. Ramasubramonian, R. L. Joshi, Y.-K. Wang (Qualcomm)] "AHG13: On signalling of MSB cycle for long-term reference pictures"

JCTVC-J0119 [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)] "AHG13: On reference picture list modification"

JCTVC-J0164 [Hendry, B. Jeon (LG)] "AHG 13: Simplified signalling of MSB cycle"

JCTVC-J0182 [Hendry, Y. Jeon, S. Park, B. Jeon (LG)] "AHG 9 / AHG 13: On identical reference picture lists"

JCTVC-J0185 [S Lu, K Sato (Sony)] "AHG13: On short_term_ref_pic_set"

JCTVC-J0210 [A.Fujibayashi, T.K.Tan (NTT DOCOMO)] "MaxDpbSize setting based on the picture size"

JCTVC-J0234 [J. Ström, R. Sjöberg, J. Samuelsson (Ericsson)] "Inter-RPS complexity reduction"

JCTVC-J0240 [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)] "Consistent coding of motion information for B slices with identical reference picture lists"

JCTVC-J0345 [G. J. Sullivan, S. Kanumuri (Microsoft)] "Editorial modifications to HEVC text specification relating to reference picture sets and random access points"

# Recommendation

The Reference picture buffering and list construction AHG recommends for the JCT-VC to review all picture buffer management and list construction related input documents.