| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0030_r1 WG11 Number: m25333 |
| --- | --- |

| Title: | CE1: Cross-verification of Intra transform mode dependency simplifications (JCTVC-J0021) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of the common-condition QP cases of CE1 on Intra transform mode dependency simplifications. The software was examined, and simulations specified in the CE description were performed. Simulations for additional configurations were also performed. The simulation results performed for this cross-verification match those provided by the proponents. The software changes are consistent with the described simplifications. Results from additional simulations are reported to be consistent with those of the required simulations. An informal subjective evaluation of some decoded sequences was also performed.

# Introduction and examination of software

Core Experiment 1 [1] defines two simplifications for applying the DST on 4x4 Luma Intra Transform Blocks. Simplification 1 applies the DST both horizontally and vertically, irrespective of the Intra prediction mode. Simplification 2 similarly applies the DST, except that the DCT is applied both horizontally and vertically when the DC Intra prediction mode is used.

The changes to the software are in the forward and inverse transform functions only. For cases when the DST would be used, instead of choosing the horizontal and vertical transforms separately based upon Intra prediction mode, the DST is always used for Simplification 1. For Simplification 2, the DST is always used except when the Intra prediction mode is 1, when the DCT is used. For Simplification 2, the software compares the mode to the value 1. If this simplification is adopted into the software, we recommend that the mode be compared to DC_IDX instead.

# Quantitative Performance

The proposed tools are compared to a reference using HM-7.0.

## Simplification 1

The DST is used both horizontally and vertically on 4x4 Luma Intra Transform Blocks, for all Intra prediction modes.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class C | 0.2% | -0.2% | -0.2% | 0.2% | -0.1% | -0.2% |
| Class D | 0.2% | -0.1% | -0.2% | 0.2% | -0.1% | -0.1% |
| Class E | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Overall | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.2% |
|  | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |
| Class F | 0.5% | 0.1% | 0.1% | 0.4% | 0.0% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | -0.2% |
| Class D | 0.2% | 0.2% | 0.0% | 0.1% | 0.0% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class F | 0.4% | 0.2% | 0.2% | 0.4% | 0.3% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Simplification 2

The DST is used both horizontally and vertically on 4x4 Luma Intra Transform Blocks for all Intra prediction modes except DC. For DC mode, the DCT is used horizontally and vertically.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class D | 0.1% | 0.0% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.3% | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.0% | 0.2% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.2% |
| Class F | 0.3% | 0.1% | 0.2% | 0.2% | 0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Subjective evaluation

Although both simplifications yield similar BD-Rate changes for many sequences, the lack of differences in BD-Rate does not always mean that there are no subjective differences among the decoded sequences. For example, Kimono1 has a 0.0% BD-Rate change for both simplifications 1 and 2, but there are visible differences in the decoded video. Figures 1-3 demonstrate an example for frame 200 of Kimono1 coded with QP=32 using the AI-Main common test condition:

Figure 1. Decoded anchor, Kimono1, portion of frame 200, QP-32

Figure 2. Decoded simplification 1, Kimono1, portion of frame 200, QP-32

Figure 3. Decoded simplification 2, Kimono1, portion of frame 200, QP-32

Both simplifications exhibit artifacts that are not present in the anchor. Some of these differences are circled in yellow. When comparing both simplifications, simplification 1 may look better for the leftmost circled vertical edge, whereas simplification 2 may look better for the middle difference. The rightmost indicated difference highlights a luma/chroma difference for the corner using simplification 1, and simplification 2 produces a softened corner. Subjective viewing by additional CE participants and viewing by other interested JCT-VC participants at the upcoming meeting should help with the decision process. Given that differences in PSNR do not always reflect differences in subjective quality, and given that many of these artifacts may not be perceived when the video is viewed in real time, this cross-verifier recommends viewing a variety of decoded sequences, and not just those which exhibit larger BD-Rate differences.

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well. The modifications to the software are consistent with the simplifications defined in the CE. Some small differences in artifacts are visible. At the planned viewing session during the upcoming meeting, conclusions on the visibility of these artifacts can be drawn.

# References

- JCTVC-I1101, “Description of Core Experiment 1 (CE1): Intra transform mode dependency simplifications,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, CH, May 2012.