| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 July – 20 July 2012 | Document: JCTVC-J0031 |
| --- | --- |

| Title: | Unification of band and edge offsets with respect to sign for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal unifies band and edge offset syntax with respect to sign while having similar coding efficiency compared to HM-7.0 (0.0 % average BDR for common conditions). The modification consist of removing all 4 sign syntax element for band offset and instead having implicit derivation of sign similar as for edge offset.

# Introduction

There are currently two main differences between the syntax for band offset and edge offset. In addition to the syntax elements for offsets, band offsets also requires syntax elements for band position and 4 syntax elements for sign.

# Proposal

It is proposed to remove the sign syntax elements for band offset and derive band offsets with sign implicitly as for edge offsets. The first two offsets are positive and the other two offsets are negative as for edge offsets, see Figure 1. To keep the ability to adjust pixel values towards 0 or towards the maximum pixel value according to the bit depth as in HM-7.0 band position equal to 0 has respective offset either negative or equal to zero and band position equal to 30 has respective offset either positive or equal to zero. The syntax modification versus the HEVC draft [1] is indicated by yellow highlight and can be found in section 4 of the document.

Band position

Figure 1: Implicit derivation of sign for band offset

# Results

## Objective performance

The results indicate that the proposal has similar coding efficiency as HM-7.0 using common coding conditions [2]. Encoding time is measured on a linux cluster and is unreliable but is expected to be similar as for HM-7.0. Decoding time is similar as for HM-7.0.

Table 1 Proposal versus HM-7.0 with average excluding class F.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,2% | 0,3% | 0,0% | 0,0% | 0,2% |
| Class D | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% |
| Overall | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,1% | 0,3% | 0,0% | 0,0% | 0,1% |
| Enc Time[%] | 98% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | -0,2% |
| Class B | 0,0% | 0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
| Class C | 0,0% | 0,3% | 0,4% | 0,0% | 0,1% | 0,1% |
| Class D | 0,0% | 0,1% | 0,0% | 0,0% | 0,1% | 0,0% |
| Class E |  |  |  |  |  |  |
| Overall | 0,0% | 0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
|  | 0,0% | 0,1% | 0,1% | 0,0% | 0,0% | -0,1% |
| Class F | 0,1% | 0,2% | 0,4% | 0,0% | 0,0% | 0,1% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| Class C | 0,1% | 0,4% | 0,6% | 0,0% | -0,1% | 0,1% |
| Class D | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% | 0,0% |
| Class E | 0,1% | -0,1% | -0,3% | 0,1% | -0,4% | 0,1% |
| Overall | 0,1% | 0,1% | 0,1% | 0,0% | -0,1% | 0,0% |
|  | 0,1% | 0,1% | 0,1% | 0,0% | -0,1% | 0,1% |
| Class F | 0,3% | 0,0% | 0,7% | 0,2% | -0,1% | 1,0% |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,0% | 0,1% | -0,3% | 0,0% | 0,1% | -0,2% |
| Class C | 0,0% | 0,1% | 0,4% | 0,0% | -0,1% | 0,1% |
| Class D | 0,0% | 0,1% | 0,7% | 0,0% | -0,2% | -0,6% |
| Class E | 0,1% | -0,2% | 0,2% | 0,1% | 0,3% | 0,6% |
| Overall | 0,0% | 0,0% | 0,2% | 0,0% | 0,0% | -0,1% |
|  | 0,0% | 0,1% | 0,2% | 0,0% | 0,1% | 0,0% |
| Class F | 0,2% | 0,3% | 0,2% | 0,1% | 0,2% | -0,1% |
| Enc Time[%] | 116% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

## Visual inspection

The subjective quality is similar to HM-7.0.

# Modifications to draft 7

#### 7.3.4.1  Sample adaptive offset parameter syntax

| sao_param( rx, ry, cIdx ){ | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInSlice = CtbAddrInSlice > 0 |  |
| leftCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − 1 ] ] |  |
| if( leftCtbInSlice && leftCtbInTile ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInSlice = (CtbAddrTS – CtbAddrRStoTS[CtbAddrRS − PicWidthInCtbs]) <= CtbAddrInSlice |  |
| upCtbInTile = TileId[ CtbAddrTS ] = = TileId[ CtbAddrRStoTS[ CtbAddrRS − PicWidthInCtbs ] ] |  |
| if( upCtbInSlice && upCtbInTile ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) { |  |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.4.1 Sample adaptive offset parameter semantics

sao_merge_left_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, and sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the left coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the left coding tree block. When sao_merge_left_flag is not present, it is inferred to be equal to 0.

sao_merge_up_flag equal to 1 specifies that the syntax elements sao_type_idx, sao_band_position, and sao_offset_abs and sao_offset_sign are derived from the corresponding syntax elements of the above coding tree block; equal to 0 specifies that these syntax elements are not derived from the corresponding syntax elements of the above coding tree block. When sao_merge_up_flag is not present, it is inferred to be equal to 0.

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of sao_offset[ cIdx ][ rx ][ ry ][ i ] when sao_type_index is equal to 5.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows.

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ]

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is set equal 0.

The variable offsetSign is derived as follows.

- If sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5 and sao_band_position[ cIdx ][ rx ][ ry ] is equal to 0and i is greater than 1,offsetSign is set to −1.

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is equal to 5 and sao_band_position[ cIdx ][ rx ][ ry ] is equal to 30, offsetSign is set to 1

- Otherwise, if sao_type_idx[ cIdx ][ rx ][ ry ] is less to 5 and i is less than 2, offsetSign is set to 1.

- Otherwise, offsetSign is set to −1. equal to sao_offset_sign[ cIdx ][ rx ][ ry ][ i ].

#### 8.7.2 Coding tree block modification process

- Otherwise (saoTypeIdx is equal to 5), the following ordered steps apply:

2. The variable saoLeftClass is set equal to min(28, max(0,sao_band_position[ cIdx ][ rx ][ ry ]-1)).

Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| Syntax element | initialisationType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| --- | --- | --- | --- | --- | --- |
| sao_offset_sign | 0 | FL, cMax = 1 | na | na | na, (uses Decode Bypass) |
|  | 1 |  | na | na | na, (uses Decode Bypass) |
|  | 2 |  | na | na | na, (uses Decode Bypass) |

# Conclusions

Band and edge offset can be unified with respect to sign while keeping coding efficiency and subjective quality as of HM-7.0. The proposal removes syntax for sign, simplifies semantics and uses implicit derivation of sign similar as for edge offset. We propose to adopt the proposal for HEVC.

# References

- B. Bross, W-J. Han et al, “High efficiency video coding (HEVC) text specification draft 7”, JCTVC-I1003_d4, Geneva, Switzerland, May 2012

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, Geneva, Switzerland, May 2012

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).