| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0032 |
| --- | --- |

| Title: | An HEVC transcoder converting non-parallel bitstreams to/from WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon CLARE (Orange Labs), Félix HENRY (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution presents a transcoder that can convert an HEVC bitstream from non-parallel to WPP, and the converse. This is achieved by plugging the output of the parsing process of HM7.0 into the input of an entropy coder. The process is lossless (the decoded videos are identical) and reversible (bitstreams transcoded twice are bit-level identical). Furthermore, producing a WPP bitstream by transcoding a non parallel one rather than from the direct source produces +0.1% (I), +0.1% (RA) and +0.3% (LDB) performance degradation measure in BD-rate. The same behavior is observed when converting a WPP bitstream to a non-parallel. The software is provided to interested JCTVC experts.

# Introduction

Multi-core architectures are becoming more widely available on a variety of devices including mobile terminals, and these terminals are no longer only used for content display, but also for content production. Consequently, efficient parallelism of the HEVC video encoder and decoder is of paramount importance.

Furthermore, it becomes more and more frequent that compressed videos are produced by the average user instead of a professional entity. One can think about, for instance, the amount of video material that is shot with personal digital camcorders, mobile phones and tablets, and that gets uploaded to popular video websites such as youtube or dailymotion. Since there is no control over the encoding of the video, it may happen that the bitstreams produced are not parallel-friendly. It may then be desirable to convert them into parallel friendly bitstreams, so that they can be displayed on devices with multi-core capabilities.

The current document describes a transcoder that converts a non-parallel bitstream into a WPP bitstream [1].

# Transcoder software

The transcoder software consists of an HM7.0 parser that provides the parsed symbols to an HM7.0 entropy encoder.

The following modifications were performed in the HM decoder software:

- Each context is identified with an “index” – the index of the context in the table holding all contexts.  In this way, at a very low level, the context being used can be easily identified.

- A small “transcoder” object is created which manages encoding of bits to either a single (nonWPP) or multiple (WPP) output substreams.  It also manages the save/restore of contexts that is necessary when encoding to WPP output.

- This transcoder object is given to the decoder’s normal CABAC decoder.

- During an otherwise normal decode operation, the low level CABAC decoder decodes symbols as before.  After each symbol is decoded, that same symbol is then passed to the transcoder object which then encodes that symbol with the appropriate encoding context.

- At a higher level during the decode operation, the transcoder object is advised of save/restore points (ie, LCU positions at the start of each LCU line).  In this way the object can perform context save/store operations if it is producing WPP output.

- Also at a higher level, the decoder needs to re-encode some high level syntax – namely SPS, PPS and slice header content, in order to reflect the changed parallel nature of the newly encoded stream.  Some minor modifications were made to the decoder in order to store values in various existing objects so that a later “encode” operation operates on initialized and valid values.

- Other packets, such as APS and SEI messages, are passed through unchanged to the output stream.

# Performance

## Losslessness

We have tested the transcoder on a set of compressed videos from the test set. The compressed bitstreams are originally encoded either in non-parallel using common test conditions [2] or in WPP mode, and are then decoded.

The bitstreams are then transcoded to the “opposite” mode. The resulting bitstreams are decoded and compared to the first one. We observed that the decoded bitstreams are identical.

## Reversibility

The transcoder was used to convert a bitstream twice:

- Non-Parallel  WPP  non-parallel

- WPP  non-parallel  WPP

In both scenarios, the initial and last bitstreams are identical at the bit level.

## Performance

In these scenarios, we have compared the rate-distortion performance of a transcoded bitstream compared to a non-transcoded bitstream

First, the videos from the test set are encoded using common conditions with WPP on. This is used as the anchor. Then, the videos are encoded with WPP off, and then transcoded to WPP on. This is the measured performance, as shown in table 1. A rate-distortion penalty is expected, since the initial coding choices have been made with slightly different proababiliites than the ones used in the transcoded video. However, the loss is moderate: +0.1% in Intra, +0.1 in Random Access, and +0.3% in Low Delay B.

Table 1: Performance of non-parallelWPP transcoding (anchor is WPP)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | 0,1% | 0,0% | -0,2% | -0,2% |
| Class B | 0,1% | 0,0% | 0,0% | 0,0% | -0,1% | 0,1% |
| Class C | 0,1% | 0,0% | -0,1% | 0,1% | -0,1% | 0,0% |
| Class D | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% | 0,1% |
| Class E | 0,1% | 0,2% | 0,1% | 0,1% | 0,2% | 0,1% |
| Class F | 0,1% | 0,0% | -0,1% | 0,2% | 0,5% | 0,5% |
| Overall | 0,1% | 0,0% | 0,0% | 0,1% | 0,1% | 0,1% |
|  | 0,1% | 0,0% | 0,0% | 0,1% | 0,1% | 0,1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,1% | -0,3% | -0,4% | 0,1% | -1,0% | -0,8% |
| Class B | 0,1% | -0,2% | -0,1% | 0,2% | -0,3% | -0,2% |
| Class C | 0,1% | -0,2% | -0,1% | 0,1% | -0,3% | 0,1% |
| Class D | 0,1% | -0,1% | 0,2% | 0,1% | 0,1% | 0,4% |
| Class E |  |  |  |  |  |  |
| Class F | 0,1% | -0,3% | -0,2% | 0,2% | -0,1% | 0,3% |
| Overall | 0,1% | -0,2% | -0,1% | 0,1% | -0,3% | -0,1% |
|  | 0,1% | -0,2% | -0,1% | 0,1% | -0,3% | -0,1% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,2% | -0,5% | -0,3% | 0,2% | -0,7% | -0,9% |
| Class C | 0,2% | -0,3% | -0,4% | 0,3% | -0,4% | -0,7% |
| Class D | 0,1% | -0,5% | -0,3% | 0,2% | -0,5% | -0,2% |
| Class E | 0,7% | 0,1% | 0,4% | 0,3% | 0,7% | 1,3% |
| Class F | 0,2% | -0,3% | -0,5% | 0,2% | -0,8% | -1,6% |
| Overall | 0,3% | -0,3% | -0,3% | 0,2% | -0,4% | -0,5% |
|  | 0,3% | -0,3% | -0,3% | 0,2% | -0,5% | -0,6% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,3% | -0,3% | -0,3% | 0,2% | -0,6% | -0,5% |
| Class C | 0,2% | -0,3% | -0,1% | 0,3% | -0,4% | -0,6% |
| Class D | 0,1% | -0,3% | -0,6% | 0,1% | -0,2% | 0,1% |
| Class E | 0,5% | 0,7% | 0,0% | 0,3% | 0,7% | 1,1% |
| Class F | 0,4% | -0,2% | -0,2% | 0,2% | -0,6% | -0,2% |
| Overall | 0,3% | -0,1% | -0,3% | 0,2% | -0,3% | -0,1% |
|  | 0,3% | -0,1% | -0,2% | 0,2% | -0,3% | -0,1% |

Similarly, the videos from the test set are encoded using common conditions with WPP off. This is used as the anchor. Then, the videos are encoded with WPP on, and then transcoded to WPP off. This is the measured performance, as shown in table 2. The loss is moderate: +0.1% in Intra, +0.2 in Random Access, and +0.3% in Low Delay B

Table 1: Performance of WPP non-parallel transcoding (anchor is reference)

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% | 0,2% |
| Class B | 0,1% | 0,2% | 0,2% | 0,1% | 0,2% | 0,1% |
| Class C | 0,1% | 0,2% | 0,3% | 0,1% | 0,2% | 0,1% |
| Class D | 0,1% | 0,0% | 0,1% | 0,1% | 0,1% | 0,1% |
| Class E | 0,1% | 0,0% | 0,2% | 0,2% | 0,0% | 0,2% |
| Class F | 0,1% | 0,1% | 0,2% | 0,0% | -0,3% | -0,3% |
| Overall | 0,1% | 0,1% | 0,2% | 0,1% | 0,1% | 0,1% |
|  | 0,1% | 0,1% | 0,2% | 0,1% | 0,1% | 0,1% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0,2% | 0,6% | 0,8% | 0,2% | 1,3% | 1,1% |
| Class B | 0,3% | 0,6% | 0,5% | 0,2% | 0,6% | 0,6% |
| Class C | 0,2% | 0,6% | 0,5% | 0,2% | 0,7% | 0,3% |
| Class D | 0,2% | 0,3% | 0,0% | 0,1% | 0,2% | -0,2% |
| Class E |  |  |  |  |  |  |
| Class F | 0,3% | 0,6% | 0,5% | 0,2% | 0,5% | 0,1% |
| Overall | 0,2% | 0,5% | 0,5% | 0,2% | 0,7% | 0,4% |
|  | 0,3% | 0,5% | 0,5% | 0,2% | 0,6% | 0,4% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,4% | 1,0% | 0,8% | 0,3% | 1,3% | 1,5% |
| Class C | 0,3% | 0,8% | 0,9% | 0,2% | 0,9% | 1,2% |
| Class D | 0,2% | 0,8% | 0,6% | 0,1% | 0,9% | 0,5% |
| Class E | 0,1% | 0,7% | 0,5% | 0,4% | 0,1% | -0,5% |
| Class F | 0,3% | 0,8% | 1,0% | 0,4% | 1,4% | 2,3% |
| Overall | 0,3% | 0,9% | 0,8% | 0,3% | 0,9% | 1,1% |
|  | 0,3% | 0,9% | 0,8% | 0,3% | 1,0% | 1,1% |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0,3% | 0,9% | 0,8% | 0,3% | 1,1% | 1,1% |
| Class C | 0,2% | 0,7% | 0,6% | 0,2% | 0,8% | 1,1% |
| Class D | 0,2% | 0,6% | 1,0% | 0,2% | 0,5% | 0,2% |
| Class E | 0,3% | 0,1% | 0,8% | 0,4% | 0,0% | -0,4% |
| Class F | 0,2% | 0,7% | 0,7% | 0,4% | 1,1% | 0,8% |
| Overall | 0,2% | 0,6% | 0,8% | 0,3% | 0,8% | 0,6% |
|  | 0,2% | 0,6% | 0,8% | 0,3% | 0,8% | 0,7% |

# Complexity

The table below shows the additional runtime introduced in the decoder as a percentage when it performs a transcode operation from a non-parallel reference to a WPP stream.  The decode and transcode operations in these cases were both run with –SEIpictureDigest=0 to avoid the MD5 calculation overhead.

|  | AIMAIN | RAMAIN | LDBMAIN | LDPMAIN |
| --- | --- | --- | --- | --- |
| Class A | 104% | 107% |  |  |
| Class B | 105% | 101% | 97% | 97% |
| Class C | 105% | 99% | 101% | 102% |
| Class D | 107% | 103% | 103% | 104% |
| Class E | 103% |  | 96% | 96% |
| Class F | 103% | 97% | 99% | 99% |
| Dec Time[%] | 105% | 102% | 99% | 99% |

Table 3: decoder runtime of non-parallelWPP transcoding

# Transcoder software release

The transcoder software can be provided to JCTVC experts on email request. Please send the requests to felix.henry@orange.com.

# Bibliography

[1] JCTVC-F274/m20694 “Wavefront Parallel Processing for HEVC Encoding and Decoding” C. Gordon, F. Henry, S. Pateux, 6th JCTVC meeting, Torino, july 2011

[2] JCTVC-I1100/m25274, ”Common HM test conditions and software reference configurations”, F. Bossen, 9th JCTVC meeting, Geneva, April 2012.

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).