| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0033 |
| --- | --- |

| Title: | WPP entry points simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon CLARE (Orange Labs), Félix HENRY (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution presents a simplified method to indicate entry point offsets for WPP substreams. Due to historical reasons in HM6.0 and earlier versions, combining WPP and slices sometimes produced empty substreams with corresponding zero-length offsets, due to the interleaved aspects of WPP substreams. Now that WPP substreams correspond to one line of CTBs, it becomes possible to modify the design of entry points so that there are no empty substreams and zero offsets anymore. The proposed modification produces a rate distortion performance of -3.4%, -3.3%, -2.4%, -2.4% (AI, RA, LB, LP) when tested with 1500 byte slices, against HM7.0.

# Introduction

In the previous design of WPP [1], in HM6.0 and earlier versions, the number of WPP substreams could be specified at encoding time. One WPP substream could correspond to a set of non-consecutive CTB lines. For instance, when using 4 substreams, the first line was assigned to substream 0, the second line to substream 1, the third line to substream 2, the fourth line to substream 3, the fifth line to substream 0, and so on. This followed a pattern where the ith line in the frame is assigned to substream number i%N, where N is the number of substreams.

Due to this mapping, when using multiple slices in a picture, it sometimes occurred that some empty substreams were created (containing only initialization bits and byte alignement), and corresponding minimal-length offsets were written in the slice header. For instance, in the above example where 4 substreams are used, it was possible to have a slice that intercepts one CTB line corresponding to substream number 3 and the next CTB line corresponding to substream number 0. In this case, the slice data would have contained substream 0, substream1 (empty), substream 2 (empty) , and substream 3. The slice header would have contained entry point offset 0, 1 (minimal length), 2 (minimal length), and 3.

During the 9th JCTVC meeting in Geneva, a modification of WPP was adopted [3], where each CTB line corresponds to one substream. Now, the number of WPP substreams N is equal to the number of CTB lines in the picture, and there is no more interleaving. This modification was adopted into HM7.0, but the entry points design was not modified. As a consequence, a slice starting at the ith CTB line will contain (i-1) leading empty substreams, and their corresponding zero length entry points will be written in the slice header.

The current proposal simplifies the design by creating exactly the number of substreams corresponding to the number of CTB lines in the slice and writing only these offsets in the slice header.

# Proposed design

In the proposed design, the slice data contains a number of WPP substreams equal to the number of CTB lines it intercepts. The mapping of CTB lines to substreams is not done using the CTB’s line number in the frame but, rather, the CTB’s line number in the frame minus the line number of the first substream present in the slice. Thus, substreams in a slice are always numbered 0,1,..,M-1 where M is the number of CTB lines in the slice.

The entry points written in the slice header are exactly those corresponding to the M WPP substreams present in the slice data.

# Performance

We used HM-7.0-dev release 2445, and common condition [2] with WPP activated and 1500-byte-size-limited slices. The reference uses the existing design for substream creation and offset entry points. The measured performance is the proposal. On can notice that a substantial reduction in rate-distortion perofmance is achieved. This is due to the fact that, for each slice starting at the i-th line of CTBs, the reference writes i-1 empty substreams and their corresponding minimal length entry points before writing the useful substreams and their respective entry points.

|  |  | All Intra Main |  |  |
| --- | --- | --- | --- | --- |
|  | Y |  | U | V |
| Class A | -6,3% |  | -6,3% | -6,4% |
| Class B | -4,5% |  | -4,5% | -4,5% |
| Class C | -2,2% |  | -2,1% | -2,1% |
| Class D | -1,0% |  | -1,0% | -1,0% |
| Class E | -3,4% |  | -3,4% | -3,4% |
| Class F | -3,0% |  | -3,0% | -3,0% |
| Overall | -3,4% |  | -3,4% | -3,4% |
|  | -3,4% |  | -3,4% | -3,4% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Random Access Main |  |  |  |
|  | Y |  | U | V |
| Class A | -6,3% |  | -6,3% | -6,3% |
| Class B | -4,4% |  | -4,4% | -4,4% |
| Class C | -1,9% |  | -1,9% | -1,9% |
| Class D | -0,7% |  | -0,7% | -0,7% |
| Class E |  |  |  |  |
| Class F | -2,8% |  | -2,8% | -2,8% |
| Overall | -3,3% |  | -3,3% | -3,3% |
|  | -3,3% |  | -3,3% | -3,3% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low delay B Main |  |  |  |
|  | Y |  | U | V |
| Class A |  |  |  |  |
| Class B | -4,5% |  | -4,6% | -4,6% |
| Class C | -1,8% |  | -1,9% | -1,9% |
| Class D | -0,6% |  | -0,6% | -0,6% |
| Class E | -1,6% |  | -1,8% | -1,8% |
| Class F | -2,6% |  | -2,7% | -2,7% |
| Overall | -2,4% |  | -2,4% | -2,4% |
|  | -2,4% |  | -2,4% | -2,4% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low delay P Main |  |  |  |
|  | Y |  | U | V |
| Class A |  |  |  |  |
| Class B | -4,5% |  | -4,6% | -4,6% |
| Class C | -1,8% |  | -1,9% | -1,9% |
| Class D | -0,6% |  | -0,6% | -0,6% |
| Class E | -1,7% |  | -1,8% | -1,8% |
| Class F | -2,6% |  | -2,7% | -2,7% |
| Overall | -2,4% |  | -2,5% | -2,5% |
|  | -2,4% |  | -2,5% | -2,4% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

Table 1: Performance of proposed modification

# Conclusion

The entry points design had become suboptimal because of the a combination of the initial WPP proposal based on substreams representing interleaved lines, and the decision to have one substream per LCU line adopted during previous JCTVC meeting.

The proposed modification is a simplification of substreams and entry point offset construction, where the substreams are indexed in order of appearance of their CTB row in the slice.

The proposed modification does not require any syntax change of the draft text, only a modification of the semantics.

# Bibliography

[1] JCTVC-F274 “Wavefront Parallel Processing for HEVC Encoding and Decoding” C. Gordon, F. Henry, S. Pateux, 6th JCTVC meeting, Torino, july 2011

[2] JCTVC-I1100, ”Common HM test conditions and software reference configurations”, F. Bossen, 9th JCTVC meeting, Geneva, April 2012.

[3] JCTVC-I0360, “AHG4: Wavefront parallel processing simplification”, Y.-K. Wang, M. Coban (Qualcomm), F. Henry (Orange Labs, 9th JCTVC meeting, Geneva, April 2012.

# Draft text modifications

It appeared from technical discussions during the meeting that the current text specification is correct, and the HM7 software is wrong, because it assigns a value to num_entry_point_offsets that is not allowed.

When WPP is activated (tiles_or_entropy_coding_sync_idc is equal to 2) the software sets num_entry_point_offsets to the index of the CTB row in the picture minus one. According to the current text, num_entry_point_offsets should be set to the number of LCU rows contained in the slice minus one.

It is therefore suggested that the software is fixed to match the current text specification. The authors of this contribution will file a bug report and provide a software patch.

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).