| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0036 |
| --- | --- |

| Title: | ALF coefficient coding with a single k-table |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Madhukar Budagavi | Tel: Email: Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com +1-214-480-1861 madhukar@ti.com |
| Source: | Nokia, Texas Instruments |  |  |

_____________________________

# Abstract

The ALF coefficient coding in HEVC is currently done using Exp-Golomb codewords with different k-variables. For chroma, all the coefficients are coded with a k-variable equal to 0 (EG(0)). For luma coefficients, a coefficient position dependent k variable is used (EG(k)). This contribution proposes to remove position dependent coding of coefficients and instead use EG(0) for all the coefficients. Two versions of signed EG(0) VLC were tested – Unsigned EG(0)+separate sign and se(v). The results show that there is 0.0%-0.1% change in coding efficiency. Since position dependent coding of coefficients is not needed anymore and se(v) can be used to code both luma and chroma coefficients, it is claimed that this proposal simplifies ALF coefficient coding.

# Introduction

The ALF coefficient coding in HEVC is currently done using Exp-Golomb codewords with different k-variables. For chroma, all the coefficients are coded with a 0 k-variable (EG(0)). For luma coefficients, a coefficient position dependent k variable is used (EG(k)). This contribution proposes to remove position dependent coding of coefficients and instead use EG(0) for all the coefficients. Two versions of signed EG(0) VLC were tested – Unsigned EG(0)+separate sign and se(v). The results show that there is 0.0%-0.1% change in coding efficiency. It should be noted that the unsigned EG(0)+separate sign part of the proposal is identical to an earlier proposal by TI (EG(0) part of JCTVC-I0346).

The change in the text is as follows:

Unsigned EG(0) + separate sign

alf_filt_coeff[ cIdx ][ i ][ j ] specifies the j-th filter coefficient of i-th filter used in the adaptive loop filtering process for the colour component cIdx.

-

Using se(v)
In addition to above change, following changes are needed

| alf_picture_info( cIdx ) { | Descriptor |
| --- | --- |
| if( cIdx = = 0 ) { |  |
| alf_no_filters_idc | ue(v) |
| if( alf_no_filters_idc = = 1 ) |  |
| alf_start_second_filter | ue(v) |
| else if( alf_no_filters_idc > 1 ) |  |
| for( i=0; i<15; i++ ) |  |
| alf_filter_pattern_flag[ i ] | u(1) |
| for( i = 0; i < AlfNumFilters; i++ ) |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ i ][ j ] | se(v) |
| } else |  |
| for( j = 0; j < 10; j++ ) |  |
| alf_filt_coeff[ cIdx ][ 0 ][ j ] | se(v) |
| } |  |

### -

### -

### -

# Results

On average the proposal has 0.0%-0.1% coding efficiency change. The individual test results are shown below.

## Unsigned EG(0) + separate sign

|  |  | All Intra HE10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |
| Class A |  | 0.0% |  | 0.0% |  | 0.0% |
| Class B |  | 0.0% |  | 0.0% |  | 0.0% |
| Class C |  | 0.0% |  | 0.0% |  | 0.0% |
| Class D |  | 0.0% |  | 0.0% |  | 0.0% |
| Class E |  | 0.1% |  | 0.0% |  | 0.0% |
| Overall |  | 0.0% |  | 0.0% |  | 0.0% |
|  |  | 0.0% |  | 0.0% |  | 0.0% |
| Class F |  | 0.0% |  | 0.0% |  | 0.0% |
|  | Random Access HE10 |  |  |  |  |  |
|  | Y |  | U |  | V |  |
| Class A | 0.1% |  | -0.1% |  | 0.0% |  |
| Class B | 0.1% |  | 0.0% |  | 0.0% |  |
| Class C | 0.1% |  | 0.0% |  | -0.1% |  |
| Class D | 0.0% |  | 0.0% |  | -0.1% |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% |  | 0.0% |  | 0.0% |  |
|  | 0.1% |  | 0.0% |  | 0.0% |  |
| Class F | -0.1% |  | 0.0% |  | 0.0% |  |

|  | Low delay B HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | -0.5% | -0.2% |
| Class E | 0.1% | 0.1% | 0.3% |
| Overall | 0.0% | -0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.1% | 0.8% |

## se(v) – Signed Exp-Golobm coding

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |
|  | Random Access HE10 |  |  |
|  | Y | U | V |
| Class A | 0.1% | -0.1% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | -0.1% |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |
|  | Low delay B HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.0% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | -0.3% |
| Class E | 0.1% | -0.4% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | 0.2% |
| Class F | 0.0% | 0.1% | 1.1% |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |
|  | Low delay P HE10 |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.1% | -0.1% |
| Class C | 0.1% | -0.2% | 0.0% |
| Class D | 0.0% | -0.1% | -0.3% |
| Class E | 0.3% | 0.2% | 0.3% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% |
| Class F | -0.3% | 0.3% | -0.2% |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).