| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0037 |
| --- | --- |

| Title: | On still picture profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema Miska Hannuksela | Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution requests to define a Still Picture profile for HEVC version 1. The proposal is based on a similar proposal at the 9th JCT-VC meeting providing further detail on the requested functionality and the associated specification text.

# Introduction

Following principles are followed for the Still Picture profile:

- Identical coding tools as in Main profile: We would like to make sure that the hardware/software decoders that implement the HEVC Main profile could decode the Still Picture profile bitstreams. We propose to enable this by

i) defining identical coding tool sets for Still Picture profile and Main profile, and

ii) choosing proper values of profile_idc for main and still picture profiles.

- A bitstream contains a single picture

- Mandating usage of tile entry point offsets when tiles are utilized: Tiles provide “random access within the picture” functionality, which is very useful for decoding very large pictures (A similar functionality is also implemented in JPEG-XR). If entry points are not signalled, it is difficult for the decoder to know what portion of the bitstream needs to be decoded for reconstructing a particular tile. It is known that entry points increases the latency in the encoder, but for coding still pictures we believe that is not a problem. Another alternative for solving this problem could be:

  - Each tile contains a single slice, where the slice doesn’t contain any LCU from other tiles: This alternative would suffer from loss of coding efficiency due to slice header overhead. Slice header prediction mechanisms might reduce the impact.

- Levels: Only the picture size related limitation apply as defined in Table A-1. Restrictions on pixel rate, CPB size, DPB size and max bitrate are not relevant as there is a single still picture to be coded. With regards to minimum compression ratio (or MinCR), we believe a restriction on number of bits (or bins) for LCU would suffice and no additional restrictions is necessary. For example, the restriction provided by Minhua Zhou to the jct-vc reflector (imposing 512 bytes for a 16x16 region) corresponds to a MinCR of 0.75.

- Decoder conformance: Still Picture Profile decoders need not be output timing conformant.

# Text

The changes over Main profile description in JCTVC-I003_d4 are highlighted by “Track changes” to highlight the small changes needed:

A.3.3. Still Picture profile

Bitstreams conforming to the Still Picture profile shall obey the following constraints:

- A bitstream shall contain exactly one coded picture

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have inter_4x4_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 0 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 1, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- When tiles_or_entropy_coding_sync_idc is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

[Ed. (KM): Confirm that this is the best formulation]

- When tiles_or_entropy_coding_sync_idc is equal to 1, the num_entry_point_offsets shall be equal to ( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 ) − 1.

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Still Picture profile is indicated by profile_idc being equal to XX.

Decoders conforming to the Still Picture profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to XX and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level. Decoders conforming to the Still Picture profile may or may not be obey output timing conformance as specified in Annex C.

A.4.1 General

The following is specified for expressing the constraints in this annex.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0.

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable fR be set to 1  300.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

- pic_width_in_luma_samples * pic_height_in_luma_samples <= MaxLumaFS, where MaxLumaFS is specified in Error: Reference source not found

- pic_width_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- pic_height_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

Bitstreams conforming to a profile at a specified level and having more than one access unit shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Error: Reference source not found that applies to picture n − 1, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Error: Reference source not found for picture n, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples of picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] <= MaxDpbSize, where MaxDpbSize is specified in Error: Reference source not found.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).