| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11-20 July 2012 | Document: JCTVC-J0038 |
| --- | --- |

| Title: | Exif data SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Jani Lainema Miska Hannuksela | Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a way to associate EXIF information for each picture of the video sequence as an SEI message. It is argued that the feature is especially useful for still pictures but could have use for video content as well.

# Introduction

Exchangeable image file format (EXIF) is a standard commonly used by cameras, smartphones, etc. to record the camera settings (shutter speed, ISO sensitivity, etc.) and scene information associated with the picture. Almost all the digital cameras and smartphones are using EXIF data to exchange scene and camera related information associated with the picture. Therefore it is important to be able to embed EXIF data to the HEVC bitstream when HEVC is used to code still pictures.

To enable this functionality, we propose a new SEI message. The SEI message is quite straightforward and is basically used to embed EXIF information within the SEI syntax. Another alternative to SEI message is to enable this functionality at a higher level, such as in the HEVC file format (ISO/IEC 14496-15:2012 Amd.2). We believe this is not very desirable, mostly because not all applications are expected to use the HEVC file format.

For the HEVC specification, we propose to refer to JEITA standard instead of replicating the syntax and semantics:

- CIPA DC-008-2010 Exchangeable image file format for digital still cameras: Exif version 2.3

Referring to EXIF standard could have one drawback. Since EXIF was standardized much earlier than HEVC, some of its features might not compatible with HEVC. For example, in section 4.5.1 the image data supported by EXIF could either be RGB uncompressed, YCbCr uncompressed or JPEG Compressed (i.e. HEVC is not mentioned).

We believe this is not an issue as the SEI message is not required by the decoding process. Also, an HEVC decoder utilizing this SEI message is aware that the corresponding EXIF data is associated with an HEVC coded picture and could interpret the specific EXIF information accordingly. We believe the exact interpretation of EXIF data should not be specified in the HEVC specification text.

# Text

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else if( payloadType = = 131) |  |
| exif_data( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

EXIF data SEI message syntax

| exif_data( payloadSize ) { | Descriptor |
| --- | --- |
| exif_poc_delta | u(32) |
| for( i = 0; i < payloadSize – 4; i++ ) |  |
| exif_data[ i ] | u(8) |
| } |  |

EXIF data SEI message semantics

- The EXIF data SEI message includes Exif Attribute Information for one or more pictures.

- exif_poc_delta specifies that the EXIF data provided with the SEI message is valid for pictures picX, for which PicOrderCnt( picX ) returns a value in the range of PicOrderCntVal to PicOrderCntVal + exif_poc_delta, inclusive.

- exif_data[ i ] contains Exif Attribute Information (“APP1”) as is specified in CIPA DC-008-2010  Exchangeable image file format for digital still cameras: Exif version 2.3.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).