| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0039 |
| --- | --- |

| Title: | AHG4/AHG9: Syntax for restricting slices and tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chia-Yang Tsai , Yu-Wen Huang, Yung-Chang Chang, Chih-Ming Wang, Chia-Yun Cheng, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chiayang.tsai, yuwen.huang, yc.zhang, cm.wang, subrina.cheng, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, both slice and tile coding structures can coexist with normative constraints on the encoder side. The encoder constraints require that all slices within a tile shall be complete and all tiles within a slice shall be complete. However, a practical decoder may still have to deal with a bitstream that unintentionally violates the constraints. In this contribution, the conditions to signal end_of_slice_flag are changed to better guide an encoder to follow the constraints. With such changes, an unintentional violation of the constraints can be avoided. The proposal is as follows: When tiles are enabled, for slices contained in a tile, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of the tile is not explicitly coded and is inferred as true if the slice starts in the middle of the tile; for tiles contained in a slice, the end_of_slice_flag syntax elements are coded as usual in the first tile, but in each of the rest tiles, only the end_of_slice_flag of the last slice granularity unit of the tile is explicitly coded while those non-last slice granularity units are not explicitly coded and are inferred as false. Simulation results reportedly show no bit rate increase and unchanged run time for the proposed syntax changes in comparison with HM-7.0-dev.

# Introduction

In HM-7.0, independent tiles are supported. Inside each tile, largest coding units (LCUs) are raster scanned. Inside each picture, tiles are raster scanned. Tile boundaries must be LCU boundaries. Moreover, both slice and tile coding structures can coexist. Tiles can be regarded as a change of LCU scan order, and slices have to follow the modified LCU scan order. In the 8th JCT-VC meeting in San Jose, on discussing JCTVC-H0463 [1], in order to simplify practical implementations, the following constraints were adopted: all slices within a tile shall be complete, all tiles within a slice shall be complete, and having both characteristics mixed within a picture is possible. Figure 1, Figure 2, Figure 3 and Figure 4 show several examples. In these figures, each block stands for one LCU; thick grey lines and thick black lines are tile and slice boundaries, respectively. In Figure 1, slice 0 and slice 1 are contained in tile 0, slice 2 and slice 3 are contained in tile 1, and slice 4 and slice 5 are contained in tile 2. All slices within a tile are complete, and nothing is illegal. In Figure 2, tile 0 and tile 1 are contained in slice 0, and tile 1 is contained in slice 1. All tiles within a slice are complete, and nothing is illegal. In Figure 3, however, slice 1 in tile 0 is not complete since it is also covered by tile 1. Hence, slice 1 is illegal. In Figure 4, tile 1 in slice 0 is not complete since it is also covered by slice 1. Similarly, tile 1 in slice 1 is not complete since it is also covered by slice 0. Hence, neither slice 0 nor slice 1 is legal.

Figure 1.  All slices within a tile are complete.

Figure 2.  All tiles within a slice are complete.

Figure 3.  Slice 1 is not complete in a tile and is illegal.

Figure 4.  Tile 0 and tile 1 are not complete in a slice and are illegal.

The above-mentioned restrictions simplify the coexistence of tiles and slices. Nevertheless, they do not impose any normative change on the bitstream syntax. Consequently, an encoder could inadvertently generate a bitstream that conforms to the syntax but violates the restrictions. In this contribution, we propose to embed the restrictions into the bitstream syntax in order to avoid an inadvertent violation of the restrictions by an encoder. The proposed method is implemented and verified on HM-7.0-dev, revision 2458. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

The above-mentioned restrictions are restated as follows: all slices within a tile shall be complete, and all tiles within a slice shall be complete. To meet the restrictions under different relations of tiles and slices, each slice can be categorized into two kinds: a slice starting at the first LCU of a tile or a slice starting in the middle of a tile. When a slice starts in the middle of a tile, it can only end within the tile, which means the slice cannot cross the tile boundary. Therefore, slices starting in the middle of a tile and containing the last LCU of the tile shall end at the last LCU of the tile, which means the end_of_slice_flag of the last LCU of the tile should be always true for these cases. To make it a normative restriction on the decoder side, the end_of_slice_flag of the last LCU of the tile is proposed to be skipped and derived as true in this situation. Figure 5 shows an example. In Figure 5, slice 1, slice 3, and slice 5 start in the middle of tile 0, tile 1, and tile 2, respectively, and contain the last LCU of the tile. Thus, each end_of_slice_flag of the last LCU of these tiles is not explicitly coded and is inferred as true. Please note that in this example the slice granularity unit is LCU.

Figure 5.  An example of the proposed method for LCU-aligned slices and tiles

When a slice starts at the first LCU of a tile, there are two possibilities for the end of the slice. One possibility is to end before the last LCU of the tile, and the slice is contained completely within a tile. The end_of_slice_flag will be coded as usual in this case. The other possibility is to end only at the last LCU of a following tile. Thus, for following tiles, only the end_of_slice_flag of the last LCU of the tile is explicitly coded while those of non-last LCUs of the tile should be always set to false. In this way, multiple tiles are contained completely in one slice. To make it a normative constraint on the decoder side, the end_of_slice_flag of a non-last LCU of a following tile is proposed to be skipped and should be always inferred as false.

Figure 5 shows an example for the first possibility, where slice 0, slice 2 and slice 4 start at the first LCU of tile 0, tile 1, and tile 2, respectively, and all end before the last LCU of the tile. Therefore, the end_of_slice_flag syntax elements of slice 0, slice 2, and slice 4 will be coded as usual. Figure 6 shows an example for the second possibility. In Figure 6, slice 0 starts at the first LCU of tile 0. Since it does not end at the last LCU of tile 0, it can only end at the last LCU of one of the following tiles. In this example, only one end_of_slice_flag at the last LCU of tile 1 is explicitly coded (with the value equal to false), and only one end_of_slice_flag at the last LCU of tile 2 is explicitly coded (with the value equal to true).

Figure 6.  An example of the proposed method for LCU-aligned slices and tiles

When fine-grained slices are used, the same concept with a slightly different description can be applied as follows. For slices starting in the middle of a tile and containing the last CU of the tile, they should end at the last CU of the tile, which means the end_of_slice_flag of the last CU of the tile is not explicitly coded and is inferred as true. Figure 7 shows an example. In Figure 7, slice 1, slice 3 and slice 5 start in the middle of tile 0, tile 1, and tile 2, respectively, and contain the last CU of the tile. Therefore, the end_of_slice_flag of the last CU of the tile is not be explicitly coded and is inferred as true. For slices starting in the beginning of a tile, it shall end before the tile (e.g. slice 0, slice 2, and slice 4 in Figure 7, where end_of_slice_flag syntax elements are coded as usual) or at the last CU of a following tile (e.g. Figure 6, where end_of_slice_flag syntax elements are coded as usual for the first tile and are only explicitly coded at the last CU for following tiles).

Figure 7.  Example of proposed method for fine-grained slices and tiles

With the proposed syntax changes, encoders are guided to generate bitstreams conforming to the restrictions on the coexistence of tiles and slices. Decoders can be relieved from possible burden that has to deal with the illegal bitstreams that violate the restrictions inadvertently.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [2]. Main-AI/RA/LB conditions were tested. The software is based on HM-7.0-dev, revision 2458. Two configuration settings of encoder are used as described below.

The first one is to test tiles containing complete slices. In this case, UniformSpacingIdc is set to 1. NumTileColumnsMinus1 and NumTileRowsMinus1 are set to 1 and 0, respectively, to enable two tile columns per picture. Moreover, SliceMode is set to 1 to enforce maximum number of LCUs in a slice. SliceArgument is set to 4 to represent maximum LCUs per slice.

The second configuration is to test slices containing complete tiles. In this case, UniformSpacingIdc is set to 1. NumTileColumnsMinus1 and NumTileRowsMinus1 are both set to 2 to enable nine (3x3) tiles per picture. Moreover, SliceMode is set to 3 to enforce maximum number of tiles in a slice. SliceArgument is set to 4 to represent maximum tiles per slice.

Simulation results are shown in Table 1 and Table 2 for two configurations, respectively. The anchor uses the original syntax in the current HEVC, while the test uses the proposed end_of_slice_flag syntax change. It can be seen that the proposed method with both two configurations does not cause any noticeable change in terms of BD-rates and run times.

Table 1.  Results of the proposed method for the first configuration

Table 2.  Results of the proposed method for the second configuration

# Conclusions

In this contribution, normative syntax changes for the restrictions on the coexistence of tiles and slices are proposed as follows. For slices contained in a tile, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of the tile is not explicitly coded and is inferred as true if the slice starts in the middle of a tile. For tiles contained in a slice, in the first tile the end_of_slice_flag syntax elements are coded as usual, but in each of the rest tiles contained in this slice only the end_of_slice_flag of the last slice granularity unit is explicitly coded while those of non-last slice granularity units are not explicitly coded and are inferred as false. With the proposed syntax changes, encoders are guided to generate bitstreams conforming to the restrictions on the coexistence of tiles and slices. Decoders can be relieved from the possible burden that has to deal with the illegal bitstreams that violate the restrictions inadvently. Simulation results reportedly show the proposed method has no impact on coding efficiency or run time.

# References

- W. Wan, T. Hellman, “Comments on specifying tile support in HEVC Profiles and Levels,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0463, February, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).