### Coding tree block raster and tile scanning conversation process

Inputs of this process are:

–	variables picHeightInCtbs and picWidthInCtbs specifying the height and the width of a picture in units of CTBs.

Outputs of this process are:

–	a vector colWidth[ i ] specifying the width of the i-th tile column in units of CTBs with the column i ranging from 0 to num_tile_columns_minus1, inclusive,

–	a vector ctbAddrRStoTS[ ctbAddrRS ] specifying the conversation from a CTB address in raster scan order to a CTB address in tile scan order with the index ctbAddrRS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive,

–	a vector ctbAddrTStoRS[ ctbAddrTS ] specifying the conversation from a CTB address in tile scan order to a CTB address in raster scan order with the index ctbAddrTS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive,

–	a vector tileId[ ctbAddrTS ] specifying the conversation from a CTB address in tile scan order to a tile id with ctbAddrTS ranging from 0 to (picHeightInCtbs * picWidthInCtbs) − 1, inclusive.

–	a vector tileFirstCtbAddr[ i ] specifying the first CTB address in the i-th tile with the tile index i ranging from 0 to (( num_tile_columns_minus1 + 1 ) * ( num_tile_rows_minus1 + 1 )) − 1, inclusive.

The vector colWidth is derived as follows:

for( i = 0; i <= num_tile_columns_minus1; i++ ) {
	if( uniform_spacing_flag )
		colWidth[ i ] = ( ( i + 1 ) * picWidthInCtbs ) / ( num_tile_columns_minus1 + 1 ) – 
			( i * picWidthInCtbs ) / ( num_tile_columns_minus1 + 1 )
	else
		colWidth[ i ] = column_width[ i ]
}

The values of rowHeight[ j ], specifying the height of the j-th tile row in units of CTBs, are derived as follows::

for( j = 0; j <= num_tile_rows_minus1; j++ )
	if( uniform_spacing_flag )
		rowHeight[ j ]  = ( ( j + 1 ) * picHeightInCtbs ) / ( num_tile_rows_minus1 + 1 ) – 
					( j * picHeightInCtbs ) / ( num_tile_rows_minus1 + 1) 
	else
		rowHeight[ j ] = row_height[ j ]

The values of colBd[ i ], specifying the location of the left column boundary of the i-th tile column in units of coding tree blocks, are derived as follows:

for( colBd[ 0 ] = 0, i = 0; i <= num_tile_columns_minus1; i++ )
		colBd[ i + 1 ] = colBd[ i ] + colWidth[ i ]

The values of rowBd[ j ], specifying the location of the top row boundary of the j-th tile row in units of coding tree blocks, are derived as follows:

for( rowBd[ 0 ] = 0, j = 0; j <= num_tile_rows_minus1; j++ )
		rowBd[ j + 1 ] = rowBd[ j ] + rowHeight[ j ]

The vector ctbAddrRStoTS is derived as follows:

for( ctbAddrRS = 0; ctbAddrRS < picHeightInCtbs * picWidthInCtbs; ctbAddrRS++ ) {
		tbX = ctbAddrRS % picWidthInCtbs
		tbY = ctbAddrRS / picWidthInCtbs
		for( i = 0; i <= num_tile_columns_minus1; i++ )
			if( tbX >= colBd[ i ] )
				tileX = i
		for( j = 0; j <= num_tile_rows_minus1; j++ )
			if( tbY >= rowBd[ j ] )
				tileY = j
		ctbAddrRStoTS[ ctbAddrRS ] = 0
		for( i = 0; i < tileX; i++ )
			ctbAddrRStoTS[ ctbAddrRS ] += rowHeight[ tileY ] * colWidth[ i ]
		for( j = 0; j < tileY; j++ )
			ctbAddrRStoTS[ ctbAddrRS ] += picWidthInCtbs * rowHeight[ j ]
		ctbAddrRStoTS[ ctbAddrRS ] += ( tbY − rowBd[ tileY ] ) * colWidth[ tileX ] + tbX − colBd[ tileX ]
	}

The vector ctbAddrTStoRS is derived as follows:

for( ctbAddrRS = 0; ctbAddrRS < picHeightInCtbs * picWidthInCtbs; ctbAddrRS++ )
		ctbAddrTStoRS[ ctbAddrRStoTS[ ctbAddrRS ] ] = ctbAddrRS

The vector tileId is derived as follows:

for( j = 0, tIdx = 0; j <= num_tile_rows_minus1; j++ )
		for( i = 0; i <= num_tile_columns_minus1; i++, tIdx++ )
			for( y = rowBd[ j ]; y < rowBd[ j + 1 ]; y++ )
				for( x = colBd[ i ]; x < colBd[ i + 1 ]; x++ )
						tileId[ ctbAddrRStoTS[ y*picWidthInCtbs + x ] ] = tIdx

The vector tileFirstCtbAddr is derived as follows:

for( j = 0, tIdx = 0; j <= num_tile_rows_minus1; j++ )
		for( i = 0; i <= num_tile_columns_minus1; i++, tIdx++ )

tileFirstCtbAddr[tIdx] = rowBd[tIdx] * PicWidthInCtbs + colBd[tIdx]

### Slice data syntax

| slice_data( ) { | Descriptor |
| --- | --- |
| CtbAddrRS = SliceCtbAddrRS |  |
| CtbAddrTS = CtbAddrRStoTS[ CtbAddrRS ] |  |
| moreDataFlag = 1 |  |
| isStartAtFristCuFlag = 0 |  |
| isCrossTileFlag = 0 |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlagIdx = −1 |  |
| do { |  |
| … |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| … |  |
| } while( moreDataFlag ) |  |

### Coding tree syntax

| coding_tree(x0,y0,log2CbSize,ctDepth) { | Descriptor |
| --- | --- |
| … |  |
| if( (!( ( x0 + (1 << log2CbSize )) % (Log2MaxCbSize >> slice_granularity) ) ¦¦ ( x0 + (1 << log2CbSize ) == pic_width_in_luma_samples)) && (!( ( y0 + (1 << log2CbSize )) % (Log2MaxCbSize >> slice_granularity) ) ¦¦ ( y0 + (1 << log2CbSize ) == pic_height_in_luma_samples)) && NumPCMBlock = = 0 ) { |  |
| if( !skipEosInferredTrue && !skipEosInferredFalse ) |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| … |  |
| } |  |

### Slice data semantics

end_of_slice_flag equal to 0 specifies that another coding block is following in the slice. end_of_slice_flag equal to 1 specifies the end of the slice and that no further coding block follows. When skipEosInferredTrue is equal to 1, end_of_slice_flag is inferred to 1. When skipEosInferredFalse is equal to 1, end_of_slice_flag is inferred to 0.

- If all of the following conditions are true, isLowerRightCornerFlag is set equal to 1,

- ( x0 + (1 << log2CbSize ) ) is equal to ( colBd[tileId[ CtbAddrTS ]%( num_tile_columns_minus1+1)+1] * (1 << Log2MaxCbSize) ) or ( x0 + (1 << log2CbSize ) ) is equal to pic_width_in_luma_samples

- ( y0 + (1 << log2CbSize ) ) is equal to ( rowBd[tileId[ CtbAddrTS ]/( num_tile_ columns _minus1+1)+1] * (1 << Log2MaxCbSize) ) or ( y0 + (1 << log2CbSize ) ) is equal to pic_height_in_luma_samples

Otherwise, isLowerRightCornerFlag is set equal to 0.

- If all of the following conditions are true, isStartAtFristCuFlag is set equal to 1,

- isCrossTileFlag is equal to 0

- SliceCbAddrZS is equal to ( tileFirstCtbAddr[tileId[ CtbAddrTS ]] * (1 << log2_diff_max_min_coding_block_size) * (1 << log2_diff_max_min_coding_block_size))

Otherwise, isStartAtFristCUflag is set equal to 0.

- If all of the following conditions are true, isCrossTileFlag is set equal to 1,

- isCrossTileFlag is equal to 0

- isLowerRightCornerFlag is equal to 1

Otherwise, isCrossTileFlag is set equal to 0.

- If all of the following conditions are true, skipEosInferredTrue is set equal to 1,

- tiles_or_entropy_coding_sync_idc is equal to 1

- SliceCbAddrZS is larger than ( tileFirstCtbAddr[tileId[ CtbAddrTS ]] * (1 << log2_diff_max_min_coding_block_size) * (1 << log2_diff_max_min_coding_block_size))

- isLowerRightCornerFlag is equal to 1

- Otherwise, skipEosInferredTrue is set equal to 0.

- If all of the following conditions are true, skipEosInferredFalse is set equal to 1,

- tiles_or_entropy_coding_sync_idc is equal to 1

- isStartAtFristCuFlag is equal to 1

- isCrossTileFlag is equal to 1

- isLowerRightCornerFlag is equal to 0

- Otherwise, skipEosInferredFalse is set equal to 0.