| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0040 |
| --- | --- |

| Title: | AHG4/AHG9: Syntax for restricting slices and WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Chia-Yang Tsai , Yu-Wen Huang, Yung-Chang Chang, Chih-Ming Wang, Chia-Yun Cheng, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {cw.hsu, chiayang.tsai, yuwen.huang, yc.zhang, cm.wang, subrina.cheng, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, both slice and WPP coding structures can coexist with a normative constraint on the encoder side. The encoder constraint requires that a slice must end in a LCU row if the slice starts in the middle of the LCU row. However, a practical decoder may still have to deal with a bitstream that unintentionally violates the constraint. In this contribution, the conditions to signal end_of_slice_flag are changed to better guide an encoder to follow the constraint. The proposal is as follows: When WPP is enabled, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of a LCU row is not explicitly coded and is inferred as true if the slice starts in the middle of the LCU row. Simulation results reportedly show no bit rate increases and unchanged run time for the proposed syntax changes in comparison with HM-7.0-dev.

# Introduction

In HM-7.0, wavefront parallel processing (WPP) is supported, where each row of largest coding units (LCUs) is in a substream, and multiple rows of LCUs can be processed in parallel by multiple encoding or decoding threads. In order to limit the degradation of coding efficiency, a wavefront pattern of processing order ensures that dependencies on spatial neighbors are not changed. Moreover, the CABAC states at the start of each LCU row are initialized based on those of the second LCU from the upper LCU row, the CABAC engine is flushed at the end of each LCU row, and byte alignment is enforced at the end of each substream. The entry points of WPP substreams are signaled as byte offsets in the slice header of the slice that contains the wavefront.

In the current HEVC, both slice and WPP coding structures can coexist. In the 9th JCT-VC meeting in Geneva, JCTVC-I0361 [1] was adopted to limit the coexistence of WPP and slices, that is, if a slice starts in the middle of an LCU row, it must end in this LCU row. Figure 1 shows an example. In Figure 1, there are six LCU rows in a picture, and each LCU row forms a wavefront substream. Moreover, the picture is also partitioned into six slices. According to the restriction on the coexistence of WPP and slices, there is no constraint on slices starting at the beginning of an LCU row, which means that these slices can end at any LCU in the picture and even cross multiple WPP substreams. Therefore, slice 0, slice 3, and slice 5 are all legal. However, as to slices starting in the middle of an LCU row, it must end no later than the end of the LCU row, which means that these slices cannot cross multiple WPP substreams. Thus, only slice 1 and slice 2 are legal, while slice 4 should be prohibited.

Figure 1.  An example of coexistence of WPP and slices, where slice 4 is illegal

The above-mentioned restriction simplifies the coexistence of WPP and slices. Nevertheless, it does not impose any normative change on the bistream syntax. Consequently, an encoder could still generate a bistream that conforms to the syntax but violates the restriction. In this contribution, we propose to embed the restriction into the bitstream syntax to avoid an inadvertent violation of the restriction by an encoder. The proposed method is implemented and verified on top of HM-7.0-dev, revision 2458. The rest of this document is organized as follows. Details of the proposed method will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

# Proposed method

The above-mentioned restriction is restated as follows: If a slice starts in the middle of an LCU row, it must end no later than the end of the LCU row. For a slice starting in the middle of an LCU row, there are only two possibilities for the end of the slice. One is to end before the last LCU of the LCU row, and the other is to end exactly at the last LCU of the LCU row, which implies no crossing LCU row is allowed. In order to meet the restriction, those slices starting in the middle of an LCU row and containing the last LCU of the LCU row shall end at the last LCU of the LCU row, which means the end_of_slice_flag syntax element of that last LCU of the LCU row should be always set to true. To make it a normative restriction on the decoder side, the end_of_slice_flag syntax element of the last LCU is proposed to be skipped and should be always derived as true. Figure 2 shows an example. In Figure 2, slice 1 starts in the middle of the LCU row and does not contain the last LCU of the LCU row, so the end_of_slice_flag will be coded as usual. However, slice 2 and slice 4 start in the middle of the LCU row and contain the last LCU of the LCU row, so the end_of_slice_flag of the last LCU of the LCU row will not be explicitly coded and is inferred as true.

Figure 2.  Proposed method for LCU-aligned slices and WPP

When fine-grained slices are used, the same concept with a slightly different description can be applied as follows. For those slices starting in the middle of the LCU row and containing the last CU of the LCU row, the end_of_slice_flag of the last CU of the LCU row will not be coded and is inferred as true. Figure 3 shows an example. In Figure 3, slice 2 and slice 4 start in the middle of the LCU row and contain the last CU of the LCU row. Thus, the end_of_slice_flag of the last CU of each of their LCU rows will not be explicitly coded and is inferred as true.

Figure 3.  Proposed method for fine-grained slices and WPP

By using the proposed syntax change, as long as an encoder follows the syntax, the generated bistreams will follow the restriction.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [2]. Main-AI/RA/LB conditions were tested. The software is based on HM-7.0-dev, revision 2458. The configuration settings of encoder are as follow. WaveFrontSynchro and WaveFrontFlush are both set to 1 to enable WPP. SliceMode is set to 1 to enforce maximum number of LCUs in a slice. SliceArgument is set to 4 to represent maximum LCUs per slice. Simulation results are shown in Table 1. The anchor uses the original syntax in the current HEVC, while the test uses the proposed end_of_slice_flag syntax change. It can be seen that the proposed method does not cause any noticeable change in terms of BD-rates and run times.

Table 1.  Results of the proposed method

# Conclusions

In this contribution, a normative syntax change for the restriction on the coexistence of WPP and slices is proposed as follows. The end_of_slice_flag of the last slice granularity unit (LCU or CU) of the LCU row is not explicitly coded and inferred as true if the slice starts in the middle of an LCU row and WPP is enabled. By doing this, an inadvertent violation of the restriction by an encoder can be avoided. It can relieve a decoder from having to deal with illegal bitstreams that violate the restriction. Simulation results reportedly show the proposed method has no impact on coding efficiency or run time.

# References

- M. Coban, Y.-K. Wang, “AHG4: Restriction on coexistence of WPP and slices,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0361, April, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).