| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0041 |
| --- | --- |

| Title: | AHG5/AHG6: On reducing context models for SAO merge syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Elena Alshina +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com Shawmin Lei +886-3-5670766 ext. 25555 {chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Samsung Electronics Ltd. and MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution the number of context models used for SAO merge-left and merge-up flags is reduced from four to one by sharing one context model for the two syntax elements among the three color components. It is reported that the coding efficiency change is 0.0% (Y), -0.1% (Cb), and -0.2% (Cr) in terms of BD-rate.

# Proposed method

In total eight context models are used for SAO parameters coding in HM-7.0: two for offset magnitudes, two for SAO types, three (one per color component) for SAO merge-left flag, and one for SAO merge-up flag. In this contribution we suggest to share one context model for SAO merge-left flag and SAO merge-up flag. In this way, the number of context models used for SAO merge syntax is reduced from four to one. CABAC initialization process was not changes during our test.

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [1]. The software is implemented on top of HM-7.0 and attached to this contribution. Test results are summarized in Table 1. In average there is small gain for all color components. BD-rates are 0.0%, -0.1%, and -0.2% for Y, Cb, and Cr, respectively.

We would like to thank QUALCOMM for verification of these test data.

Table 1.  Results of the proposed method

Additionally we have tested variant when two context models are used: one for merge-left flag (without dependency on color component) and one for merge-up flag. This test was also done without CABAC initialization change. The performance is almost the same, but the number of context models is higher. As a result, we do not recommend this variant further for consideration.

Since merge-left and merge-up flags are encoded for each LCU, we also have studied the coding efficiency impact of the suggested change in test with LCU size 32x32 and LCU size 16x16. As it is summarized in Table 2, there is no harm for SAO coding efficiency from the suggested changes even for smaller LCU sizes.

Table 2.  Summary of BD-rates of the proposed method for different LCU sizes

| LCU size | Y BD-rate,% | Cb BD-rate,% | Cr BD-rate,% |
| --- | --- | --- | --- |
| 64x64 | -0.04% | -0.13% | -0.17% |
| 32x32 | -0.04% | 0.01% | -0.09% |
| 16x16 | -0.01% | 0.00% | 0.09% |

# Conclusions

Based on reported results:

- 0.0% (Y), -0.1%(Cb), -0.2%(Cr) BD-rate change

- 3 context model reduction in HEVC design

Samsung and MediaTek propose using only one context model for both SAO merge-left and SAO merge-up flags in HEVC DIS text and next version of HM reference software.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).