#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| pic_width_in_luma_samples | ue(v) |
| num_tile_columns_minus1 | tu(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| if( num_tile_columns_minus1 > 0 ) { |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| } |  |
| for( i = 0; i < num_tile_columns_minus1 – 1; i++ ) |  |
| column_width_minus_min_ctb_num[ i ] | ue(v) |
| if( num_tile_columns_minus1 > 0) |  |
| second_last_column_width_minus_min_ctb_num | tu(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| … |  |
| } |  |

#### Picture parameter set RBSP semantics

…

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture and coded by truncated unary codeword with the maximum value maxNumTileColMinus1:

maxNumTileColMinus1 = ( pic_width_in_luma_samples / 384) -1

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture.

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

uniform_spacing_flag equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width_minus_min_ctb_num[ i ] and row_height[ i ]. When uniform_spacing_flag is equal to 0 and num_tile_columns_minu1 is greater than 0, minCtbNumTileSize is derived as follow:

minCtbNumTileSize =
 	Ceil(384/(1<< ( log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)))

column_width_minus_min_ctb_num[ i ] specifies the width of the i-th tile column minus minCtbNumTileSize in units of coding tree blocks. The width of the i-th tile column in units of CTBs is equal to column_width_minus_min_ctb_num[i] plus minCtbNumTileSize.

second_last_column_width_minus_min_ctb_num specifies the width of the second last tile column minus minCtbNumTileSize in units of coding tree blocks and coded by truncated unary codeword with the maximum value maxSecondLastColumnWidth:

acculmulatedTileWidth = 0

for( i = 0; i < num_tile_columns_minus1 – 1; i++ ) {

acculmulatedTileWidth +=  (column_width_minus_min_ctb_num[ i ] + minCtbNumTileSize)

}

maxSecondLastColumnWidth = (pic_width_in_luma_samples – (acculmulatedTileWidth << (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)) – 384) >> (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)

The width of the second last tile column in units of CTBs is equal to second_last_column_width_minus_min_ctb_num plus minCtbNumTileSize.

row_height[ i ] specifies the height of the i-th tile row in units of coding tree blocks.

The list colWidth[ i ] specifies the width of the i-th tile column in units of CTBs with the column i ranging from 0 to num_tile_columns_minus1, inclusive.

…