| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0042 |
| --- | --- |

| Title: | AHG4/AHG9: Syntax modifications for tile width constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai , Chih-Wei Hsu, Yu-Wen Huang, Yung-Chang Chang, Chih-Ming Wang, Chia-Yun Cheng, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, cw.hsu, yuwen.huang, yc.zhang, cm.wang, subrina.cheng, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC, an encoder shall obey a normative constraint on tile width. The encoder constraint requires that tile width shall be equal to or greater than 384 pixels. However, a practical decoder may still have to deal with a bitstream that unintentionally violates this constraint. It is proposed to modify the syntax elements of tile width and number of tile columns to better guide an encoder to follow the constraint. With such changes, only legal tile widths have corresponding codewords to code them. Thus, an unintentional violation of the constraint can be avoided.

# Introduction

In HEVC text specification draft 7 [1], tile-related syntax elements are specified in PPS, as shown in Table 1. There is a constraint on tile width: Tile width shall be equal to or greater than 384 pixels. When tiles are enabled, a legal bitstream must obey the constraint on tile width. However, even if the tile constraint on semantics applies, a decoder still may receive an illegal bitstream if an encoder does not obey the constraint. For a practical decoder product, overdesign is required to deal with these exceptions. In the worst case of an illegal bitstream, one picture may contain tiles as many as LCUs. For example, there are total 129600 LCUs for 7680x4320 resolution video with LCU size equal to 16x16, and thus an overdesigned decoder needs to allocate memory to store the tile partitioning information of 129600 tiles in the worst case. This overdesign requires non-trivial memory storage for a practical decoder product. Therefore, we propose syntax changes such that only legal tile widths or numbers of tile columns can be signaled, which force encoders to follow the constraint.

Table 1.  Tile-related syntax elements in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| ………. |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| ……… |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposed method

According to the tile constraint which restricts the minimum tile width equal to 384 pixels, we propose syntax modifications for tile width and number of tile columns to prevent encoders from generating illegal bitstreams.

## Syntax modifications for tile width

When the tile partitioning method is not uniform, i.e., uniform_spacing_flag is equal to false, the size of each tile has to be explicitly coded in units of LCU (namely, CTB). In order to ensure tile width is at least 384 pixels, as shown in Table 2, it is proposed to represent the tile width by the syntax element, column_width_minus_min_ctb_num, where the “min_ctb_num” is equal to “384 divided by CtbSize.” In HEVC text specification draft 7, please note that the CtbSize is derived using two syntax elements, log2_min_coding_block_size_minus3 and log2_diff_max_min_coding_block_size, coded in SPS. To simplify the decoding flow, it is desirable to derive real tile sizes immediately during parsing tile size syntax elements in PPS, so the CtbSize has to be known in advance. If we want to derive real tile sizes during parsing PPS without referencing SPS data, the two CtbSize-related syntax elements have to be re-sent in PPS when tiles are enabled and tile partitioning method is not uniform; otherwise, the two CtbSize-related syntax elements in PPS can be removed.

Table 2.  Proposed syntax modifications for restricting tile sizes and numbers

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| ………. |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| pic_width_in_luma_samples | ue(v) |
| num_tile_columns_minus1 | tu(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| if( num_tile_columns_minus1 > 0) { |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| } |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus_min_ctb_num[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height [ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| ……… |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Syntax modifications for number of tile columns

When the tile partitioning method is uniform, i.e., uniform_spacing_flag is equal to true, the size of each tile has to be derived from the picture width, picture height, number of tile columns, and number of tile rows. In order to ensure tile width is at least 384 pixels, as shown in Table 2, it is proposed to use truncated unary coding for num_tile_columns_minus1 instead of unary coding. Moreover, the maximum values of the num_tile_columns_minus1 is set to “picture width in luma divided by 384, then truncate to integer, and finally minus one, i.e., Floor(pic_width_in_luma_samples/384)-1.” In HEVC text specification draft 7, pic_width_in_luma_samples is coded in SPS. Therefore, in order to parse PPS without dependency on SPS, pic_width_in_luma_samples is re-sent in PPS when tiles are enabled.

# Software implementation

The software is developed on top of HM-7.0-dev (rev. 2459). Besides the syntax modifications described in Section 2, the encoder also performs an encoder parameter checking process to avoid generating an illegal bitstream that violates the constraint on tile width and number of tile columns.

# Conclusions

In this contribution, in order to help an encoder to follow the constraint which restricts the minimum tile width to 384 pixels, syntax modifications are proposed accordingly.

# References

- B. Bross, W. –J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

MediaTek Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).