#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| … |  |
| tiles_or_entropy_coding_sync_idc | u(2) |
| if( tiles_or_entropy_coding_sync_idc = = 1 ) { |  |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| if( num_tile_columns_minus1 > 0 ) { |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| } |  |
| for( i = 0; i < num_tile_columns_minus1 – 1; i++ ) |  |
| column_width_minus_min_ctb_num_h[ i ] | ue(v) |
| if( num_tile_columns_minus1 > 0) |  |
| second_to_last_column_width_minus_min_ctb_num_h | ue(v) |
| for( i = 0; i < num_tile_rows_minus1 – 1; i++ ) |  |
| row_height_minus_min_ctb_num_v[ i ] | ue(v) |
| if( num_tile_rows_minus1 > 0) |  |
| second_to_last_row_height_minus_min_ctb_num_v | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } else if( tiles_or_entropy_coding_sync_idc = = 3 ) |  |
| … |  |
| } |  |

#### Picture parameter set RBSP semantics

…

num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture. The value of num_tile_columns_minus1 shall be in the range of 0 to maxNumTileColMinus1, inclusive.

maxNumTileColMinus1 = (pic_width_in_luma_samples / 256) -1

num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture. The value of num_tile_rows_minus1 shall be in the range of 0 to maxNumTileRowMinus1, inclusive.

maxNumTileRowMinus1 = (pic_height_in_luma_samples / 64) -1

When num_tile_columns_minus1 is equal to 0, num_tile_rows_minus1 shall not be equal to 0.

uniform_spacing_flag equal to 1 specifies that column boundaries and likewise row boundaries are distributed uniformly across the picture. uniform_spacing_flag equal to 0 specifies that column boundaries and likewise row boundaries are not distributed uniformly across the picture but signalled explicitly using the syntax elements column_width_minus_min_ctb_num_h[ i ] and row_height_minus_min_ctb_num_v[ i ].

When uniform_spacing_flag is equal to 0 and num_tile_columns_minus1 is greater than 0, minCtbNumTileSizeH is derived as follows:

minCtbNumTileSizeH =
Ceil(256/(1<< (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)))

When uniform_spacing_flag is equal to 0 and num_tile_rows_minus1 is greater than 0, minCtbNumTileSizeV is derived as follows:

minCtbNumTileSizeV =
Ceil(64/(1<< (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)))

column_width_minus_min_ctb_num_h[ i ] specifies the width of the i-th tile column minus minCtbNumTileSizeH in units of coding tree blocks. The width of the i-th tile column in units of coding tree blocks is equal to column_width_minus_min_ctb_num_h[i] plus minCtbNumTileSizeH.

second_to_last_column_width_minus_min_ctb_num_h specifies the width of the second to last tile column minus minCtbNumTileSizeH in units of coding tree blocks. The value of second_to_last_column_width_minus_min_ctb_num_h shall be in the range of 0 to maxSecondToLastColumnWidth, inclusive, where

acculmulatedTileWidth = 0
for( i = 0; i < num_tile_columns_minus1 – 1; i++ ) {
  acculmulatedTileWidth += (column_width_minus_min_ctb_num_h[ i ] + minCtbNumTileSizeH)
}

maxSecondToLastColumnWidth = (pic_width_in_luma_samples – (acculmulatedTileWidth << (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)) – 256) >> (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)

The width of the second to last tile column in units of coding tree blocks is equal to second_to_last_column_width_minus_min_ctb_num_h plus minCtbNumTileSizeH.

row_height_minus_min_ctb_num_v[ i ] specifies the width of the i-th tile row minus minCtbNumTileSizeV in units of coding tree blocks. The height of the i-th tile row in units of coding tree blocks is equal to row_height_minus_min_ctb_num_v[i] plus minCtbNumTileSizeV.

second_to_last_row_height_minus_min_ctb_num_v specifies the height of the second to last tile row minus minCtbNumTileSizeV in units of coding tree blocks. The value of second_to_last_row_height_minus_min_ctb_num_v shall be in the range of 0 to maxSecondToLastRowHeight, inclusive, where

acculmulatedTileHeight = 0
for( i = 0; i < num_tile_rows_minus1 – 1; i++ ) {
  acculmulatedTileHeight += (row_height_minus_min_ctb_num_v[ i ] + minCtbNumTileSizeB)
}

maxSecondToLastRowHeight = (pic_height_in_luma_samples – (acculmulatedTileHeight << (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)) – 64) >> (log2_min_coding_block_size_minus3 + 3 + log2_diff_max_min_coding_block_size)

The height of the second to last tile row in units of coding tree blocks is equal to second_to_last_row_height_minus_min_ctb_num_v plus minCtbNumTileSizeV.

…