| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11– 20 July 2012 | Document: JCTVC-J0043 |
| --- | --- |

| Title: | AhG5: On bypass coding for SAO syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin, J.H. Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

In this contribution CABAC by-pass mode is applied for SAO parameters coding. Performance effect is analysed if context coding replaced with by-pass for each SAO syntax element individually and for all together. The number of context bins per sample in the worst case for SAO magnitude is 1.50. Usage of by-pass for SAO magnitude provides only 0.0%(Y), 0.2%(U), 0.1%(V) BD-rate drop. This modification reduces the worst case context bins per sample in SAO syntax from 1.45 to 0.07 (the worst case will be SAO type).

# Introduction

SAO syntax elements encoded for each LCU using CABAC are summarized in Table 1. The wholes set of SAO parameters (merge_left/up flags, SAO type, SAO magnitudes, sings and Left band position) is encoded for each color component. In total 8 ctx models are used for SAO syntax.

Table 1. SAO syntax elements encoding.

| SAO syntax element | binarization | # ctx models | #ctx bins/sample |  |
| --- | --- | --- | --- | --- |
| SAO merge left | 1 bin | 3 (one per color component) | 0.01 |  |
| SAO merge up | 1 bin | 1 | 0.01 |  |
| SAO type | 0th bin | 1 | 0.01 | 0.07 |
|  | remainder (up to 5 bins) | 1 | 0.06 |  |
| SAO magnitude | 0th bin | 1 | 0.05 | 1.45 |
|  | remainder (up to 31 bins) | 1 | 1.41 |  |
| SAO sign | 1 bin | By-pass | 0 |  |
| Left band position | 1 bin | By-pass | 0 |  |

The worst case of context coded bins usage is SAO magnitude. Assuming the smallest LCU size1616,10 bits internal bit depth and 12 (4 classes  3 color components) syntax elements peer LCU for SAO magnitude we spend 1.50 bins per sample. This analysis is consistent with data included to AhG5 template.

The second worst case from view point of context coded bins amount per sample among SAO syntax is SAO type (0.07).

# Context coded bins number reduction

In this contribution we studied possibility to reduce the number of context coded bins with simplest modification. The context coding was replaced with by-pass without binarization changes for each syntax element.

Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [1]. The summary of test results is presented in Table 2. An average across all test cases (without class F) BD-rate is reported. Second column shows reduction for the number of context models in HEVC. The third column reports the worst case ctx bin/samples number change due to tested modification.

Table 2. SAO syntax elements encoding.

| ctx  by-pass | #ctxs | Worst case ctx bin/sample | Y BD-rate, % | U BD-rate, % | V BD-rate, % | Max BD-rate drop Luma | Max BD-rate drop Chroma |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SAO merge left | -1 | 1.451. 45 | 0.57% | 0.20% | 0.02% | 3.96% | 5.86% |
| SAO merge up | -1 | 1.451. 45 | 0.63% | 0.45% | 0.41% | 4.59% | 3.99% |
| SAO type | -2 | 1.451. 45 | 0.42% | 0.40% | 0.40% | 5.10% | 5.36% |
| SAO magnitude 0th bin | -1 | 1.451.41 | 0.01% | -0.04% | -0.09% | 0.30% | 6.32% |
| SAO magnitude remaining bins | -1 | 1.450.07 | 0.04% | 0.24% | 0.27% | 0.41% | 6.05% |
| SAO magnitude all bins | -2 | 1.450.07 | 0.05% | 0.16% | 0.14% | 0.74% | 5.62% |
| All SAO syntax elements | -8 | 1.450.00 | 0.51% | 0.44% | 0.34% | 3.93% | 6.87% |
| Only SAO magnitude range changed |  |  |  |  |  |  |  |
| SAO range 0..7 | 0 | 1.450.33 | 0.02% | 0.04% | 0.03% | 0.74% | 5.62% |

Additionally statistics for total number of context coded bins was collected in AhG5. All SAO syntax elements together use 3% of total number of ctx coded bins in HEVC. These data are reported in Table3. The number of context coded bins for each SAO syntax element relatively to total amount of context coded bins in HM7.0 is presented.

Theoretically the worst case of context coded bins usage in SAO syntax is magnitude coding. But in practice both SAO type and merge left/up flags use higher amount of context coded bins compare to SAO magnitudes.

Based on performance test results 3 solutions for SAO context coded bins reduction look attractive. We measured reduction for context coded bins for SAO magnitudes and reduction of total amount of ctx coded bins in HEVC for both suggested solutions.

Table 3. The number of context coded bins for SAO syntax elements relatively to total amount of context coded bins.

|  | Context coded bins for SAO syntax |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | offsets | types | merge left | merge up | total for SAO |
| AI-Main | 0.3% | 0.2% | 0.3% | 0.2% | 1.1% |
| RA-Main | 0.5% | 1.3% | 1.2% | 1.0% | 4.0% |
| LB-Main | 0.5% | 1.3% | 1.2% | 1.0% | 4.1% |
| LP-Main | 0.6% | 1.4% | 1.3% | 1.1% | 4.4% |
| AI-HE10 | 0.2% | 0.1% | 0.3% | 0.2% | 0.9% |
| RA-HE10 | 0.3% | 0.9% | 0.9% | 0.8% | 2.9% |
| LB-HE10 | 0.3% | 1.1% | 1.0% | 0.9% | 3.3% |
| LP-HE10 | 0.3% | 1.1% | 1.0% | 0.9% | 3.3% |
| ALL | 0.4% | 0.9% | 0.9% | 0.8% | 3.0% |

Solution1: There is no performance loss if only 1st bin in SAO magnitude is encoded using by-pass. The worst case ctx bins/sample reduces from 1.50 to 1.45 in this case. One context model becomes redundant in this design. Statistically 8.3% of cxt coded bins spent for SAO magnitude is reduced, which is 0.1% of total amount of ctx coded bins in HM7.0.

Solution 2: The better reduction for context coded bins number in the worst case (1.500.07) is observed if all SAO magnitude bins are encoded using by-pass. 2 context models can be removed from HEVC design in this case. Performance drop is 0.0% for Luma and 0.1% for Chroma. It should be notices that the main drop comes due to class F. Including class F the drop is 0.1% for Luma and 0.2% for Chroma. Statistically 20.0% of cxt coded bins spent for SAO magnitude is reduced, which is 0.3% of total amount of ctx coded bins in HM7.0. Additionally there is no mixture of context coded and by-passed bins in this design.

Solution 3. If SAO magnitude range is set to 0…7 regardless internal bitDepth (in HM7.0 design SAO magnitude range increases with internal bitDepth) then only performance is change only in HE10 tests. In average there is even Chroma gain due to this modification. But drop for class F is higher in this case. Since SAO provides visual quality improvement for class F sequences any decision about SAO magnitude range change needs careful visual quality check for class F.

Detail test results for both solutions are included into APPENDIX and excel spreadsheets attached to this contribution. We would like to thank experts from MediaTek for cross-check.

Simple conversion context coded bins to by-pass for other SAO syntax elements provides much higher drop (from 0.4% to 0.6% in terms of Luma BD-rate) and not recommended for further consideration.

# Smaller LCU sizes

Since SAO parameters are signalled per LCU we may expect higher performance degradation due to using by-pass mode instead context coding if LCU goes down. We also have studied performance of “Solution 2” in test with LCU size 32x32 and 16x16. As it is shown on Table 4 there is no harm for SAO performance from suggested changes even for smaller LCU sizes.

This test set proves that the context modeling during SAO magnitude coding is not that efficient in current HEVC design and using by-pass coding for all bins of SAO magnitude will simplify design, reduce memory usage and help CABAC throughput.

Table 4. All SAO magnitude bins are encoded using by-pass (average BD-rate).

| LCU size | Y BD-rate,% | U BD-rate,% | V BD-rate,% |
| --- | --- | --- | --- |
| 64x64 | 0.05% | 0.16% | 0.14% |
| 32x32 | 0.07% | 0.05% | 0.08% |
| 16x16 | 0.04% | 0.00% | 0.03% |

# Conclusions

Based on reported results:

- 0.0% (Y), 0.2%(U), 0.1%(V) BD-rate drop

- The worst case 1. 45 0.07 reduction of context coded bins per sample in SAO syntax

- 20% reduction of SAO ctx coded bins (0.3% of total amount ctx bins) reduction

- 2 context models reduction in HEVC design

Samsung suggests using by-pass coding for SAO magnitudes in HEVC.

# Reference

[1] F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Samsung Electronics Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# DIS text

Recommended solution requires several changes of DIS text. They are summarized below.

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_offset_abs | 0 | TU, cMax = ( 1<< (Min(bitDepth, 10) − 5)) − 1 | 0 na | Table 9-2 na | 0 na, (uses Decode Bypass) |
|  | 1 |  | 0 na | Table 9-2 na | 2 na, (uses Decode Bypass) |
|  | 2 |  | 0 na | Table 9-2 na | 4 na, (uses Decode Bypass) |

Table 9-2 – Values of variable initValue for sao_offset_abs ctxIdx

| Initialization variable | sao_offset_abs ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 171 | 119 | 199 | 169 | 119 | 151 |

# Appendix

Table 4. HM7.0 performance test results if 1st SAO magnitude bin is encoded using by-pass.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | -0.1% |
| Class E | 0.1% | 0.0% | -0.5% | 0.0% | -0.3% | -0.1% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| Class F | -0.1% | -0.3% | 0.0% | 0.0% | 0.0% | 1.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | -0.3% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% |
| Class D | 0.1% | 0.1% | 0.2% | 0.0% | -0.2% | -0.4% |
| Class E | 0.1% | -0.4% | 0.1% | 0.0% | 0.6% | -0.1% |
| Overall | 0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | -0.2% | -0.5% | -1.1% | 0.1% | 0.0% | 1.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Avg. | 0.0% | 0.0% | -0.1% |  |  |  |

Table 5. HM7.0 performance test results if all SAO magnitude bins are encoded using by-pass.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.2% | 0.2% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| Class B | 0.0% | 0.4% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class F | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.4% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.3% | 0.0% | 0.2% | 0.2% |
| Class C | 0.0% | 0.3% | 0.4% | 0.0% | 0.2% | 0.3% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.6% | 0.3% | 0.2% | -0.1% | 0.2% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.1% | 0.2% |
| Class F | 0.0% | 0.5% | 0.2% | 0.3% | 0.8% | 1.9% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.7% | 0.5% | 0.1% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% | 0.4% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | -0.5% |
| Class E | 0.3% | 0.5% | 0.9% | 0.0% | 0.2% | -0.2% |
| Overall | 0.1% | 0.3% | 0.4% | 0.0% | 0.1% | -0.1% |
| Class F | -0.1% | -0.2% | -0.5% | 0.2% | 0.8% | 1.9% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
| Avg. | 0.0% | 0.2% | 0.1% |  |  |  |

Table 6. SAO magnitude range 0…7 regardless internal bitDeph.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.3% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | -0.1% | -0.3% | -0.5% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | -0.3% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.3% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | -0.4% | -0.3% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -1.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | 0.2% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% |
| Class F | 0.0% | 0.0% | 0.0% | 0.3% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Avg. | 0.0% | -0.1% | -0.1% |  |  |  |