| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11– 20 July 2012 | Document: JCTVC-J0044 |
| --- | --- |

| Title: | Encoder modification for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. Alshina, A. Alshin, J.H. Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

SAO search process modification is suggested in this contribution. This is further improvement on top of adopted during 9th JCTVC meeting Canon’s contribution. In current HM design SAO tools is turned on/off on a frame level base on a statistical criterion of the previous frame. In present contribution the same criterion is used but applied separately for Luma and Chroma. Performance improvement due to this encoder modification is 0.2% for Luma and 0.0% for Chroma under common test condition. 23% of SAO context coded bins are reduces by this modification (which is 0.7% of total context bins amount).

# Introduction

During 9th JCTVC meeting Canon’s contribution [1] was adopted. This contribution suggests adjusting SAO search strategy for hierarchical coding used in HM s/w for both random access and low delay tests. Fig.1 shows this hierarchy of frames. Frames of depth 0 have the highest quality in the hierarchy. The QP for the frames of depth 0 corresponds to the traditional QP setting associated to the whole sequence. For frames of depth 0, the quantization parameter is set to the value QP for the first Intra frame, otherwise it is set to QP+1 for B frames. For the B frames of depth 1, the QP is set to QP+2 and for the B frames of depth 2, the QP is set to QP+3…

Once per slice SAO enable flag is encoded for each color component. Then SAO parameters are encoded for each LCU. In [1] was suggested to check the number of LCUs with SAO off on frame with depth 0 and if SAO was turned off in more than 75% LCU then for higher depth SAO search process was skipped (automatically SAO enable flags were set to 0 for all 3 color component).

Additionally on Fig.1 we show the portion of LCU with SAO off separately for Luma and Chroma. This statistics is very different for 2 color components. The reason is simple. SAO syntax is the same for Luma and Chroma but number of Chroma (both Cb and Cr) samples in LCU is 2 times smaller compare Luma. So with the same side information Chroma SAO corrects fewer amounts of samples.

Due to very high ratio of LCU where SAO is off for Chroma on depth 0 in current HM7.0 design SAO forced to be off for Luma more often then needed. And we also have situation when due to frequent usage of SAO for Luma on depth 0 encoder searches for SAO Chroma even SAO was off for Chroma in 100% depth 0 LCUs.

In order to achieve better balance we suggest counting separately LCUs with SAO off for Luma and for Chroma. If portion of LCUs with SAO off for Luma is less than 75% (this threshold is the same with [1]) then Luma SAO will be disabled on higher depths. If portion of LCUs with SAO off for Chroma is less than 50% (this threshold is lower than for Luma) then both Cr and Cb SAO search will be skipped on higher depths.

Figure 1. Hierarchical coding structure and example for portion of LCU with SAO off.

We didn’t separate Cr and Cb, since statistics of SAO on/off blocks for them is very similar.

# Test results

Suggested above encoder modification for SAO was implemented in HM7.0 and tested according to [2] in random access and low-delay tests (there is no performance change in all intra test due to this modification). Test results are summarized in Table 1.

In average suggested modification provides 0.2% performance improvement for Luma without performance degradation for Chroma. Encoding-decoding time variation is under noise level. More gain is observed for low resolution sequences (since redundant overhead is not encoded) and for class F.

Table 1. SAO encoder modification.

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class B | -0.2% | 0.2% | 0.0% | -0.1% | 0.0% | -0.1% |
| Class C | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class D | -0.5% | -0.3% | -0.3% | -0.4% | -0.3% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
|  | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% |
| Class F | -0.3% | -0.1% | 0.0% | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.1% | -0.1% | 0.1% | 0.4% |
| Class C | -0.1% | 0.0% | 0.2% | -0.1% | -0.1% | 0.0% |
| Class D | -0.3% | -0.3% | 0.0% | -0.2% | -0.4% | -0.3% |
| Class E | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 0.3% |
| Overall | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% | 0.1% |
|  | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% |
| Class F | -0.3% | 0.2% | 0.4% | -0.1% | 0.5% | 1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | 0.4% | 0.0% | -0.1% | 0.3% | 0.0% |
| Class C | -0.2% | -0.1% | 0.2% | -0.1% | -0.1% | 0.0% |
| Class D | -0.3% | 0.0% | 0.2% | -0.2% | 0.1% | -1.0% |
| Class E | -0.3% | -0.2% | 0.0% | -0.1% | 0.2% | -0.1% |
| Overall | -0.4% | 0.1% | 0.0% | -0.2% | 0.1% | -0.2% |
|  | -0.4% | 0.1% | 0.0% | -0.2% | 0.1% | -0.1% |
| Class F | -0.4% | 0.3% | -0.2% | -0.3% | 0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Average | -0.2% | 0.0% | 0.0% |  |  |  |

Internally we tested different threshold for disabling SAO Chroma: 25%, 50% and 75%. Performance difference between them is 0.01%-0.02% only.

Additionally statistics for total number of context coded bins was collected using AhG5 s/w. All SAO syntax elements together use 3% of total number of ctx coded bins in HEVC.

Suggested modification reduces 23% of SAO context coded bins, which is 0.7% of total context coded bins in HEVC. Statistics of context coded and by-pass bins relatively to HM7.0 is presented in Table 2.

Table 2. Statistics of ctx coded and by-pass bins relatively to HM7.0.

|  | Total |  | SAO |  |
| --- | --- | --- | --- | --- |
|  | ctx | bp | ctx | bp |
| AI Main | 100.00% | 100.00% | 100.00% | 100.00% |
| RA Main | 97.91% | 99.99% | 47.01% | 98.03% |
| LB Main | 99.38% | 99.98% | 78.35% | 101.73% |
| LP Main | 99.15% | 99.82% | 78.54% | 100.70% |
| Average | 99.11% | 99.95% | 75.97% | 100.12% |
|  | Total |  | SAO |  |
|  | ctx | bp | ctx | bp |
| AI HE10 | 100.00% | 100.00% | 100.00% | 100.00% |
| RA HE10 | 98.54% | 100.00% | 42.83% | 100.22% |
| LB HE10 | 99.74% | 99.99% | 84.21% | 102.51% |
| LP HE10 | 99.65% | 99.91% | 83.85% | 100.00% |
| Average | 99.48% | 99.98% | 77.72% | 100.00% |

There is no reduction for context coded bins in theoretical worst case.

Detail test results are included into excel spreadsheets attached to this contribution. We would like to thank experts from Canon for cross-check.

# Conclusions

Based on reported results:

- 0.2% Luma BD-rate gain without performance degradation for Chroma

- 23% reduction of SAO ctx coded bins (0.7% of total amount ctx bins) reduction

Samsung recommend adoption of proposed SAO search strategy modification for next version of HM reference s/w.

# References

- G. Laroche, T. Poirier, P. Onno, “Non-CE1: Encoder modification for SAO interleaving mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0184, April 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, April 2012.

# Patent rights declaration(s)

Samsung Electronics Ltd does not have any current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).