| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0046 |
| --- | --- |

| Title: | AHG6: On left band position coding in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | Elena Alshina +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, jeonghoon}@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution suggests moving left band position coding in SAO after SAO magnitudes. There is no performance change due to this modification but one condition check can be removed from specification text and s/w.

# Proposed method

During 8th JCTVC meeting band offset coding was modified [1],[2]. Currently left band position is encoded first (by-pass mode, 5 bins) then magnitudes are encoded using context coding and finally SAO signs are encoded using by-pass mode.

Specification text is shown is Table 1. There are 2 checks for the same condition (highlighted in yellow).

#### Table 1 (7.3.4.2). Sample adaptive offset parameter syntax (current text).

| sao_param( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][i] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| } |  |
| } |  |

If one modifies the order of SAO syntax elements: and encodes left band position after magnitude then one condition check can be removed from specification text and s/w implementation. Suggested modification is shown in Table 2.

Additional benefit is de-interleaving of contect coded and by-pass bins in SAO syntax: sao_offset_sign and sao_band_position (both encoded using by-pass).

#### Table 2 Sample adaptive offset parameter syntax (suggestion 1)

| sao_param( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][i] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |

In this order we can skip sao_band_position[ cIdx ][ rx ][ ry ] coding if all SAO magnitudes are zeros. This situation doesn’t happen with smart HM7.0 encoder. But since encoder operation are not specified in a standard we potentially may have encoder which will choose band offset for current LCU with all correcting offset values equal to 0. Redundant sign information will be no encoded in this case (HEVC has this condition check) but redundant 5 bins for sao_band_position will be encoded. This is un-desirable. Our suggestion 2 (which is presented in Table 3) prevents this situation.

#### Table 3 Sample adaptive offset parameter syntax (suggestion 2)

| sao_param( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = =5 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx][ ry ][ i ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][i] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][0] != 0 && sao_offset_abs[ cIdx ][ rx ][ ry ][1] != 0 && sao_offset_abs[ cIdx ][ rx ][ ry ][2] != 0 && sao_offset_abs[ cIdx ][ rx ][ ry ][3] != 0 ) |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| else |  |
| sao_band_position[ cIdx ][ rx ][ ry ] = 0 |  |
| } |  |
| } |  |

# Simulation results

Implementation for both suggestions 1 and 2 was done on top of HM-7.0. As it is expected there is no performance change, since the same amount of by-pass bins is encoded in different order. Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [3]. Summary is presented in Table 4.

Table 4. Test results for left band position encoding after magnitudes in HM7.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class B | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class C | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class D | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class E | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Overall | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
|  | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class F | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class B | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class C | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class D | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class E |  |  |  |  |  |  |
| Overall | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
|  | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class F | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class C | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class D | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class E | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Overall | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
|  | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class F | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class C | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class D | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class E | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Overall | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
|  | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Class F | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% | 0.000% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

In order to remove one condition check and de-interleave context coded and by-pass bins during SAO parameters encoding Samsung proposes encode SAO left band position after magnitudes and signs. Additionally Samsung proposed not to encode left band position if all SAO offsets are zeros.

# References

- G. Laroche, T. Poirier, P. Onno, “On additional SAO Band Offset classifications”, Joint Collaborative Team on Video Coding, JCTVC-G246, Nov. 2011

- E. Maani, O. Nakagami, “Flexible Band Offset Mode in SAO,” Document of Joint Collaborative Team on Video Coding, JCTVC-H0406, February 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).