| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0047 |
| --- | --- |

| Title: | AHG6/AHG9: Cleanups and fixes for APS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Yang Tsai , Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chiayang.tsai, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current HEVC text specification draft 7, only ALF parameters are coded in APS. However, in HM-7.0-dev software, an APS is sent per picture and an APS ID is sent per slice regardless of whether ALF is enabled. In this contribution, first, cleanups of APS in software are proposed without any bitstream change in common test conditions. Next, a non-normative encoder-only fix of APS is proposed: An APS for a picture is sent only when ALF is enabled for this picture. It is reported the fix of APS achieves 0.1% coding gain in terms of BD-rate in common test conditions. Finally, a normative syntax modification, as a fix of APS ID, is proposed: An APS ID is sent in a slice header only when ALF is enabled for this slice. It is reported the fix of APS ID has no impact on BD-rates in common test conditions, where the APS ID is always 0. In practical cases, the savings of useless APS IDs could be larger.

# Introduction

In HEVC text specification draft 7 [1], only adaptive loop filtering (ALF) parameters are coded in the adaptation parameter set (APS). However, in HM-7.0-dev software, cleanups for APS are still needed. For example, SAO parameters are not coded in APS, but dependencies between SAO and APS still exist. Therefore, in our first non-normative software modification, we provide a clean APS implementation without any BD-rate change.

Since only ALF parameters are in the APS, an APS should only be sent for a picture when ALF is enabled for this picture. However, in the software, an APS is always sent even when ALF control flag in the sequence parameter set (SPS) is false. Therefore, in our second non-normative software modification, we provide a fix to this issue.

In the text and software, an APS ID is coded in a slice header when the ALF control flag in the SPS is equal to true. However, if ALF is disabled in a slice, the APS ID is unnecessary even if ALF is enabled in the SPS. Therefore, in our normative syntax modification, we propose to send an APS ID in a slice header only when ALF is enabled in this slice.

# Proposed methods

## Cleanups of APS

First, we clean up APS without changing any bitstream in common test conditions. The modifications are mainly removing the dependencies between APS and SAO, which should have been removed since the APS mode of SAO was removed. Besides, we also provide an aligned buffer management for APS on both encoder and decoder sides.

## Fix of APS – Send an APS only when ALF is enabled for the picture

Next, we fix the APS with a non-normative modification without changing any text specification. Instead of sending an APS for a picture even when ALF is disabled for this picture, the APS is only sent for the picture when ALF is enabled for this picture. In HEVC text specification draft 7, only ALF parameters are coded in the APS. Therefore, we follow the text and modify the HM software encoder. If all of the following conditions are true, an APS is required to be sent for the picture.

- SPS: adaptive_loop_filter_enabled_flag is equal to true.

- Slice header: slice_alf_flag[0] is equal to true or slice_alf_flag[1] is equal to true or slice_alf_flag[2] is equal to true.

Otherwise, no APS is sent for the picture.

Besides, one bug-fix for HM is also included. In the current HM software, aps_id in the slice header is coded when adaptive_loop_filter_enabled_flag or sample_adaptive_offset_enabled_flag in the SPS is equal to true. However, this is not aligned to HEVC text specification draft 7. We fix this bug to let HM follow the text.

## Fix of APS ID – Send an APS ID only when ALF is enabled for the slice

Finally, we fix the APS ID with a normative syntax modification. Instead of sending an aps_id for a slice even when ALF is disabled for this slice, the aps_id is sent in the slice header only when ALF is enabled for this slice. To be more specific, we propose that an aps_id in the slice header is sent if all of the following conditions are true.

- SPS: adaptive_loop_filter_enabled_flag is equal to 1.

- Slice header: slice_alf_flag[0] is equal to true or slice_alf_flag[1] is equal to true or slice_alf_flag[2] is equal to true.

Otherwise, aps_id is not coded in the slice header. The syntax modifications in the slice header are shown in Table 1.

Table 1.  Proposed slice header syntax modifications in Section 3.1

| slice_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if(adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| …….. |  |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) |  |
| slice_alf_flag[ cIdx ] | u(1) |
| if(slice_alf_flag[0] ¦¦ slice_alf_flag[1] ¦¦ slice_alf_flag[2]) |  |
| aps_id | ue(v) |
| } |  |
| …… |  |
| } |  |

# Simulation results

Experiments were conducted according to the common test conditions defined in JCTVC-I1100 [2]. Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP conditions were tested. The software is based on HM-7.0-dev (revision 2454). Three incremental experiments are described as follows.

The first experiment is the APS cleanups in Section 2.1. Table 2 shows the results. The anchor is HM-7.0-dev (revision 2454). No BD-rate change is observed.

The second experiment is the APS fix in Section 2.2 on top of the first experiment. Table 3 shows the results. The anchor is HM-7.0-dev (revision 2454). On average about 0.1% coding gain in terms of BD-rate can be achieved because APS of a picture is saved when ALF is disabled for this picture.

The third experiment is the APS ID fix on top of the second experiment. Table 4 shows the results. The anchor is HM-7.0-dev (revision 2454). Comparing Table 3 with Table 4, we can observe that BD-rates are almost unchanged. The reason is that the current HM encoder always uses aps_id equal to 0. In this case, the bit waste caused by a useless aps_id is very minor. In practical cases, aps_id can be in the range of 0-63, and the benefit of this fix would be larger.

# Conclusions

In this contribution, cleanups of APS, a fix of APS, and a fix of APS ID are proposed. It is reported that the cleanups of APS do not change bitstreams, the fix of APS has 0.1% coding gain in terms of BD-rate, and the fix of APS ID has almost no impact on coding efficiency in common test conditions.

# References

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

MediaTek Inc.  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2.  Results of the APS cleanups

Table 3.  Results of the APS cleanups and the APS fix

Table 4.  Results of the APS cleanups, APS fix, and APS ID fix