| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0048 |
| --- | --- |

| Title: | AHG6: ALF with non-normative encoder-only improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei MediaTek No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Tomoo Yamakage, Takayuki Itoh, Takeshi Chujoh TOSHIBA 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan In Suk Chong, Marta Karczewicz Qualcomm 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, chihming.fu, yuwen.huang, shawmin.lei}@mediatek.com Tomoo Yamakage +81-44-549-2152 {tomoo.yamakage, takayuki2.itoh, takeshi.chujoh} @toshiba.co.jp Marta Karczewicz +1-858-658-5673 {ichong, martak} @qualcomm.com |
| Source: | MediaTek Inc., TOSHIBA Corporation, Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, non-normative encoder-only ALF improvements are proposed. The major changes are reusing up to eight previous adaptation parameter sets and estimating rates more accurately in the rate-distortion optimization (RDO) process. The bug-fix of ticket #574 (i.e., swapping coef[2] and coef[4] in the bitstream) is also included. Compared with the ALF in HM-7.0, the proposed ALF can increase luma coding gains by 0.2-1.1% in terms of BD-rate. A subjective viewing session is requested for this proposal.

Compared with ALF-off using common test condition sequences, the coding gains of ALF-on are 2.9%, 4.0%, and 4.0% for Y, Cb, and Cr, respectively, when Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP are averaged and class F is excluded. Compared with ALF-off using 15 720p and 10 1080p sequences that were commonly seen during the study period of KTA software, the corresponding coding gains of ALF-on become 5.2%, 5.4%, and 5.3% for Y, Cb, and Cr, respectively, where the ALF luma coding gains in Main-LP and HE10-LP are even higher than 9% and 10%, respectively. Roughly speaking, the encoding time increase and decoding time increase for ALF are 1% and 10%, respectively, and can be significantly reduced by using SIMD instructions in a typical CPU, GPU, or DSP.

In comparison with the ALF in HEVC CD and HM-6.0, the luma coding gain and the chroma coding gain have been increased by 0.2-0.6% and 1.4-3.3%, respectively, while the complexity has been dramatically reduced by removing the block-based adaptation (BA) mode, allowing only one signaling mode, and adopting several simplifications. Therefore, it is proposed to include ALF into HEVC Main profile or a new profile that is proper.

# Introduction

Figure 1 shows the basic syntax structure of adaptive loop filtering (ALF) in HEVC working draft version 7 (WD-7) [1] and HEVC reference software model version 7.0 (HM-7.0). Filter coefficients are coded in adaptation parameter set (APS), and in each largest coding unit (LCU) up to three on/off control flags (one per color component) are coded to indicate whether the LCU should be filtered or not. In HM-7.0, two ALF encoding algorithms are provided. When low encoding latency is desired, a low latency mode can be chosen to derive filter coefficients from a previous picture, and the encoding latency is only one LCU. If high coding efficiency is desired, a high efficiency mode can be chosen to derive filter coefficients from a current picture, and the encoding latency becomes one picture. In order to further improve ALF coding efficiency, non-normative encoder-only improvements are proposed. The rest of this contribution is organized as follows. The proposed encoding algorithm will be introduced in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Basic syntax structure of ALF in HM-7.0

# Algorithm description

The two ALF encoding algorithms in HM-7.0 can be summarized in Figure 2(a). They can be partitioned into three stages, and the difference between the high efficiency mode and the low latency mode is the source of statistics. The first stage is filter coefficient derivation. In this stage, filter coefficients will be derived based on the statistics of a previous or current picture. The second stage is to decide picture-level ALF on/off flags based on the derived filter coefficients and statistics. In the last stage, if ALF is enabled for the current picture, LCU-level on/off decision is performed to decide LCU on/off control flags. ALF control is independently decided for each luma/chroma component.

(a)                         (b)

Figure 2.  (a) ALF encoding algorithm in HM-7.0; (b) the proposed ALF encoding algorithm

The proposed ALF encoding algorithm is shown in Figure 2(b). Two new stages, picture-level filter selection and re-check picture-level ALF on/off decision, are added. On top of the original encoding algorithms, five improvements are described as follows.

- Consider the rate of LCU on/off control flags during picture-level on/off decision

- Use CABAC for rate estimation of LCU on/off control flags during LCU on/off decision

- Reuse up to eight previous adaptation parameter sets during picture-level filter selection

- Recheck picture-level ALF-off after LCU on/off decisions (only for high efficiency mode)

- Increase from one to three redesigns of filter coefficients (only for high efficiency mode)

For the high efficiency mode that uses a current picture to design filter coefficients, items 1-5 are all applied. For the low latency mode that uses a previous picture to design filter coefficients, only items 1-3 are applied. Moreover, the bug-fix of ticket #574 is also included in the provided software, which swaps the third filter coefficient (coef[2]) and the fifth filter coefficient (coef[4]) in the bitstream to match HM with WD. This bug-fix does not have noticeable impact on coding efficiency.

# Simulation results

The encoder improvements and the bug-fix are implemented on HM-7.0. All the results were simulated according to the common test conditions (CTC) defined in JCTVC-I1100 [2] with ALF-off as the anchor.

In HM-6.0, there were two ALF signaling modes. One is for picture-based optimization with encoding latency equal to one picture and is denoted as HM-6.0 PIC OPT, and the other is for LCU-based optimization with encoding latency equal to one LCU and is denoted as HM-6.0 LCU OPT. In HM-7.0, there is only one ALF signaling mode with two encoding algorithms. One is the high efficiency mode with encoding latency equal to one picture and is denoted as HM-7.0 OLD HE, and the other is the low latency mode with encoding latency equal to one LCU and is denoted as HM-7.0 OLD LL. In this proposal, the improved high efficiency mode and the improved low latency mode are denoted as HM-7.0 NEW HE and HM-7.0 NEW LL, respectively.

Table 1 shows the comparison between the ALF in HM-7.0 and the improved ALF in HM-7.0 using CTC sequences. When the encoding latency is one LCU, the average luma BD-rate is improved by 0.5%. When the encoding latency is one picture, the average luma BD-rate is improved by 0.7%. The average coding gains for HM-7.0 NEW HE are 2.9%, 4.0%, and 4.0% for Y, Cb, and Cr, respectively.

Table 1.  BD-rates of the ALF in HM-7.0 and the improved ALF in HM-7.0 using CTC sequences

Besides CTC sequences, all 720p and 1080p sequences we obtained during the key technical area (KTA) software study period were also tested, which can represent more practical cases for applying HEVC. In the rest of this contribution, we call these additional 720p and 1080p sequences as KTA sequences. In today’s market of portable devices, 720p has become the mainstream picture size, and 1080p will become popular within a few years. In the TV market, 1080p has been the major format for several years. In CTC sequences, sizes of 416x240, 832x480, and 1024x768 are still used, which seem to be a bit too small for HEVC applications. Table 2 shows the KTA sequences including 15 720p and 10 1080p sequences.

Table 2.  List of KTA sequences (name_widthxheight_frames.yuv)

| 720p | BigShips_1280x720_600.yuv |
| --- | --- |
|  | City_1280x720_900.yuv |
|  | Crew_1280x720_600.yuv |
|  | Cyclists_1280x720_600.yuv |
|  | Harbour_1280x720_600.yuv |
|  | Jets_1280x720_550.yuv |
|  | Night_1280x720_460.yuv |
|  | Optis_1280x720_600.yuv |
|  | Panslow_1280x720_550.yuv |
|  | Preakness_1280x720_600.yuv |
|  | Raven_1280x720_600.yuv |
|  | Sailormen_1280x720_600.yuv |
|  | Sheriff_1280x720_600.yuv |
|  | ShuttleStart_1280x720_600.yuv |
|  | Spincalendar_1280x720_550.yuv |
| 1080p | blue_sky_1920x1080_106.yuv |
|  | pedestrian_area_1920x1080_375.yuv |
|  | riverbed_1920x1080_250.yuv |
|  | rush_hour_1920x1080_500.yuv |
|  | station2_1920x1080_313.yuv |
|  | sunflower_1920x1080_500.yuv |
|  | toys_and_calendar_1920x1080_250.yuv |
|  | tractor_1920x1080_690.yuv |
|  | vintage_car_1920x1080_300.yuv |
|  | walking_couple_1920x1080_250.yuv |

Table 3 shows the comparison between the ALF in HM-7.0 and the improved ALF in HM-7.0 using KTA sequences. When the encoding latency is one LCU, the coding efficiency is essentially unchanged. When the encoding latency is one picture, the average luma BD-rate is improved by 1.6%. The average coding gains for HM-7.0 NEW HE are 5.2%, 5.4%, and 5.3% for Y, Cb, and Cr, respectively. Moreover, in Main-LP condition the ALF luma coding gain reaches 9.0%, and in HE10-LP condition the ALF luma coding gain is higher than 10%.

Table 3.  BD-rates of the ALF in HM-7.0 and the improved ALF in HM-7.0 using KTA sequences

Table 4 shows the comparison between the ALF in HM-6.0 and the improved ALF in HM-7.0 using CTC sequences. When the encoding latency is one LCU, the average luma coding gain is improved by 0.6%, and the average chroma coding gain is improved by 2.3-2.4%. When the encoding latency is one picture, the average luma BD-rate is improved by 0.2%, and the average chroma coding gain is improved by 1.4-1.5%. In comparison with the ALF in HM-6.0, the new ALF has slight improvement in coding efficiency, while the complexity has been dramatically reduced by removing the block-based adaptation (BA) mode, allowing only one signaling mode (picture-level filters and LCU-level on/off flags), and adopting several simplifications.

Table 4.  BD-rates of the ALF in HM-6.0 and the improved ALF in HM-7.0 using CTC sequences

The improved ALF in HM-7.0 roughly increases the encoding time and the decoding time by 2% and 10%, respectively, compared with HM-7.0 ALF off. The software was written in unoptimized C++ codes. By using SIMD instructions on a typical CPU, GPU, or DSP, 2-4X or higher speed-up of ALF processing can be expected [3].

All detailed experimental data can be found in the 10 provided Excel files. The software is also provided.

# Conclusions

This contribution presents five non-normative encoder-only improvements and one bug-fix of ALF. It is reported the improved ALF achieves 1.6%/3.1%/2.3%/3.6% and 1.8%/3.3%/2.6%/4.5% luma coding gains for Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP conditions, respectively, using CTC sequences. In addition, it is reported the improved ALF achieves 2.5%/4.3%/3.8%/9.0% and 2.6%/4.6%/4.3%/10.7% luma coding gains for Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP conditions, respectively, using 25 additional HD (720p or 1080p) sequences.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

TOSHIBA Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “High Efficiency Video Coding (HEVC) text specification draft 7”, Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

- T. Yamakage, T. Itoh, “ALF decoding time reduction by adopting a simple SIMD code”, Document of Joint Collaborative Team on Video Coding, JCTVC-G923, November, 2011.