| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0049 |
| --- | --- |

| Title: | AHG6: Comparison between ALF and bi-prediction MC in low-delay conditions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Chia-Yang Tsai, Yung-Chang Chang, Chia-Yun Cheng, Fu-Chun Yeh, Hao-Wei Li, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {chingyeh.chen, chiayang.tsai, yc.zhang, subrina.cheng, Ryan.Yeh, hao-wei.li, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, ALF and bi-prediction MC are compared using 15 1080p sequences, where five of the sequences are common test condition class B sequences and the rest were commonly used during KTA software study period. Main-LP without ALF is used as the anchor. Main-LP with ALF and Main-LB without ALF are tested against the anchor. The software is in the uploaded package of JCTVC-J0048, which contains non-normative and encoder-only improvements for ALF.

In the coding efficiency comparison, Main-LB without ALF reduces 10.5% bitrates, while Main-LP with ALF reduces 8.3% bitrates. Therefore, Main-LP with ALF can preserve 79% of the coding gain achieved by Main-LB without ALF.

In the encoding time comparison, Main-LB without ALF increases 40% run time, while Main-LP with ALF increases less than 0.5% run time. Since the run time difference is very huge, it can be claimed that Main-LP with ALF has significantly lower encoding complexity than Main-LB without ALF.

In the decoding time comparison, Main-LB without ALF increases 9% run time, while Main-LP with ALF increases 16% run time. However, in this case the run time difference is not very large, so the run times may not properly reflect complexities. On the decoder side, Main-LP with ALF reduces 1% reference picture access for MC, while Main-LB without ALF increases 61% of reference picture access for MC, where MC in Main-LP without ALF already occupies most DRAM bandwidth of the entire decoder. In practical designs, reference pictures are usually implemented in off-chip DRAM, the power consumption of off-chip DRAM is 251X of on-chip SRAM in 40nm silicon technology, and the access time of DRAM is 26X of on-chip SRAM. According to these chip design experiences, bi-prediction MC could increase complexity and power significantly higher than ALF in practical decoders.

Based on the above reasons, for real-time low-delay encoding-decoding applications (e.g. video phones and video conferencing) with full HD resolution, ALF could be a better trade-off than bi-prediction MC.

# Introduction

According to JCTVC-J0048 [1], adaptive loop filtering (ALF) achieves -5.2% (Y), -5.4% (Cb), and -5.3% (Cr) BD-rates for 15 720p and 10 1080p videos that were commonly seen during key technical area (KTA) software study period. Each of the BD-rate numbers is an average with eight test conditions (Main-AI/RA/LB/LP and HE10-AI/RA/LB/LP). Among all test conditions, ALF provides the highest coding gain in LP. The BD-rates are -9.0% (Y), -6.6% (Cb), -6.9% (Cr) in Main-LP and -10.7% (Y), -11.3% (Cb), and -11.2% (Cr) in HE10-LP. However, the BD-rates of ALF significantly degrade to -3.8% (Y), -3.7% (Cb), -3.7% (Cr) in Main-LB and -4.3% (Y), -6.3% (Cb), -5.8% (Cr) in HE10-LB. The only difference between LP and LB is the usage of B-slices that allow bi-prediction motion compensation (MC), which means the coding gain of ALF overlaps with that of bi-prediction MC. Therefore, this contribution further compares ALF with bi-prediction MC in low-delay conditions with full HD videos.

# Comparison between ALF and bi-prediction MC

All experiments follow the configuration settings defined in the common test conditions, JCTVC-I1100 [2]. The software is the HM-7.0 with non-normative encoder-only improvements for ALF, as uploaded in the JCTVC-J0048 package. We considered all 1080p sequences that are available to us at hand, and there are 15 sequences in total. Five of the sequences are common test condition class B sequences, and the rest 10 sequences were commonly used during KTA software study period, as described in JCTVC-J0048. We used Main-LP without ALF as the anchor and then tested Main-LP with ALF and Main-LB without ALF against the anchor.

The BD-rate results are shown in Table 1. Main-LB without ALF reduces 10.5% bit rate, while Main-LP with ALF reduces 8.3% bit rate. Therefore, Main-LP with ALF can preserve 79% of the coding gain achieved by Main-LB without ALF.

Table 1.  BD-rates and run times of LDP with ALF and LDB without ALF

The encoding time result is also shown in Table 1. Main-LB without ALF increases 40% encoding time, while Main-LP with ALF increases less than 0.5% encoding time. Since the run time difference is very huge, it can be claimed that Main-LP with ALF has significantly lower encoding complexity than Main-LB without ALF.

The decoding time result is also shown in Table 1. Main-LB without ALF increases 9% decoding time, while Main-LP with ALF increases 16% decoding time. However, in this case the run time difference is not very large, so the run times may not properly reflect complexities. More analyses are provided in the following paragraphs.

On the decoder side, Main-LP with ALF does not increase any reference picture access, while Main-LB without ALF significantly increases reference picture access for MC, where MC occupies the most DRAM bandwidth of the entire decoder. In practical designs, reference pictures are usually implemented in off-chip DRAM, the power consumption of off-chip DRAM is much higher than that of on-chip SRAM, and the access time of DRAM is much longer than that of on-chip SRAM. According to these chip design experiences, bi-prediction MC could increase complexity significantly higher than ALF in practical decoders.

Table 2 shows the power consumption data for accessing 64 bits according to our internal data sheet of DRAM and simulated data with synthesized gates and random test patterns. The silicon technology is 40nm, and the clock frequency is set to 240MHz. The SRAM configuration is 512 words and 64 bits per word. As can be seen, the power consumption for accessing off-chip DRAM is 251X, 171X, 3782X, and 1489X of power consumptions for reading on-chip SRAM, writing on-chip SRAM, reading on-chip register, and writing on-chip register, respectively. As for data access time, in one of our internal systems, off-chip DRAM demands 26T, on-chip SRAM needs 1T, and on-chip register requires less than 1T, respectively, where 1T represents one clock period.

Table 2.  Power consumptions for accessing 64 bits in 40nm silicon technology

Table 3 shows the MC memory bandwidth and the related power consumption, where 128-bit DRAM is considered. Different rows denote different sequences or quantization parameters (QPs). The average MC memory bandwidths for Main-LP without ALF, Main-LP with ALF, and Main-LB without ALF are 306.77MB/s, 303.92MB/s, and 487.36MB/s. The MC memory bandwidth is slightly decreased by 1% for Main-LP with ALF, while it is greatly increased by 61% for Main-LB without ALF. According to our experience, the MC memory bandwidth for Main-LP without ALF occupies 60-80% of the total DRAM bandwidth on the decoder side, and 61% increase of MC memory bandwidth has a huge impact on the system design. After normalizing the frame rate to 60fps, the increase of power consumption for MC is 35.5mW. In the next paragraph, we will further show how relatively significant the power increase by the Main-LB without ALF is.

Table 3.  MC memory bandwidth and DRAM power consumption

We found several AVC decoder chips [3][4][5][6][7][8] presented in first-tier IEEE conferences (e.g., ISSCC, VLSI Symposium, and ISCAS). After normalizing the processing throughput to 1080p 60fps and silicon technologies to 40nm, the normalized power consumptions of the AVC decoder logic cores are 31.9mW, 55.9mW, 18.3mW, 52.9mW, 69.3mW, and 22.4mW, respectively. The average is 42.8mW for the AVC decoder logic core. In the last paragraph, it was analyzed that Main-LB without ALF will increase 35.5mW for MC using off-chip DRAM. With some off-line calculations, it can be derived that Main-LP without ALF needs roughly 60.9mW for MC using off-chip DRAM. As the additional power consumptions of ALF logics and fractional sample interpolation caused by bi-prediction MC, after synthesis, the reported numbers are only 1.1mW and 0.9mW, respectively. Although the power consumption of a HEVC decoder logic core is not known, it still can be roughly estimated that Main-LB without ALF could increase 20-30% power of the entire decoding system (DRAM and logic), while Main-LP with ALF has very minor increase in power consumption.

Based on the above reasons, for real-time low-delay encoding-decoding applications (e.g. video phones and video conferencing) with full HD resolution, ALF could be a better trade-off than bi-prediction MC.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei, T. Yamakage, T. Itoh, T. Chujoh, I. S. Chong, M. Karczewicz, “AHG6: ALF with non-normative encoder-only improvements,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0048, July, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

- T.-W. Chen, Y.-W. Huang, T.-C. Chen, Y.-H. Chen, C.-Y. Tsai, L.-G. Chen, “Architecture design of H.264/AVC decoder with hybrid task pipelining for high definition videos,” in Proceedings of IEEE International Symposium on Circuits and Systems, pp. 2931-2934, May, 2005.

- C.-C. Lin, J.-I. Guo, H.-C. Chang, Y.-C. Yang, J.-W. Chen, M.-C. Tsai, J.-S. Wang, “A 160kGate 4.5kB SRAM H.264 video decoder for HDTV applications,” in Proceedings of IEEE International Solid-State Circuits Conference, pp. 406-407, February, 2006.

- T. M. Liu et al., “A 125μW, fully scalable MPEG-2 and H.264/AVC video decoder for mobile applications,” in Proceedings of IEEE International Solid-State Circuits Conference, pp. 402-403, February, 2006.

- C.-D. Chien, C.-C. Lin, Y.-H. Shih, H.-C. Chen, C.-J. Huang, C.-Y. Yu, C.-L. Chen, C.-H. Cheng, J.-I. Guo, “A 252kgate/71mW multi-standard multi-channel video decoder for high definition video applications,” in Proceedings of IEEE International Solid-State Circuits Conference, pp. 282-284, February, 2007.

- D. Zhou, X. You, J. Zhu, J. Kong, Y. Hong, X. Chen, X. He, C. Xu, H. Zhang, J. Zhou, N. Deng, P. Liu, and S. Goto, “A 1080p@60fps multi-standard video decoder chip designed for power and cost efficiency in a system perspective,” in Proceedings of IEEE Symposium on VLSI Circuits, pp. 262-263, June, 2009.

- T.-D. Chuang, P.-K. Tsung, P.-C. Lin, L.-M. Chang, T.-C. Ma, Y.-H. Chen, Y.-H. Chen, C.-Y. Tsai, L.-G. Chen, “A 59.5mW scalable/multi-view video decoder chip for quad/3-D full HDTV and video streaming applications,” in Proceedings of IEEE International Solid-State Circuits Conference, pp. 330 -331, February, 2010.