| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0052 |
| --- | --- |

| Title: | Bug-fix of intra chroma DM mode for Intra_NxN with SCU size larger than 8x8 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang, Yi-Hsin Huang, Ting-An Lin, Kun-Bin Lee, Chi-Cheng Ju, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yuwen.huang, yihsin.huang, zion.lin, kb.lee, cc.ju, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A mismatch between HM-7.0 software and HEVC text specification draft 7 is reported as follows. In the HM-7.0 software, an intra CU sends only one intra chroma prediction mode regardless of the PU partition mode. However, in the text specification (JCTVC-I1003_d4.doc), an intra CU sends one intra chroma prediction mode for each chroma PU, which is inconsistent with HM-7.0 software when NxN PU partition mode is selected. The inconsistency needs to be resolved.

Two possible solutions are reported here. Method 1 follows more or less what is in the software but includes a bug fix to let the four chroma PUs refer to the four different luma PUs when the intra CU is an SCU larger than 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.1-0.6% luma coding gains and -0.1-1.0% chroma coding gains when the SCU size is set to 16x16 in luma. Method 2 follows what is specified in text but includes a bug fix to send only one intra chroma prediction mode when the intra CU is an SCU equal to 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.0-0.4% luma coding gains and 0.6-2.4% chroma coding gains for when the SCU size is set to 16x16 in luma.

Matched software and text are provided for both methods. It is proposed to adopt one of the proposed methods as a bug-fix to the current HEVC.

# Introduction

In the current HEVC, for an intra CU larger than the SCU size, only 2Nx2N PU partition mode is allowed; for an intra CU equal to the SCU size, both 2Nx2N and NxN PU partition modes are allowed. Generally speaking, the chroma PU partition mode follows the luma PU partition mode. However, the smallest intra PU size is limited to 4x4 for both luma and chroma components. When an intra CU is an SCU equal to 8x8 in luma and NxN PU partition mode is selected, only one 4x4 chroma PU exists and corresponds to the upper left 4x4 luma PU, instead of four 2x2 chroma PUs corresponding to four 4x4 luma PUs.

There are six intra chroma prediction mode options, which are DM, LM, planar, vertical, horizontal, and DC. Since luma is usually signaled before chroma, intra chroma prediction may utilize coded luma information. In this spirit, DM mode inherits a corresponding intra luma prediction mode. When an intra CU is an SCU equal to 8x8 in luma and NxN PU partition mode is selected, there is only one 4x4 chroma PU in the CU. If the DM mode is signaled, the intra luma prediction mode of the upper-left luma PU is referred, as shown in Figure 1 (a). When an intra CU is an SCU larger than 8x8 in luma and NxN PU partition mode is selected, there are four 4x4 chroma PUs, as shown in Figure 1 (b). However, in the HM7.0 software, when the DM mode is selected, still only the intra luma prediction mode of the upper-left luma PU is used for all the four chroma PUs, even though each intra chroma PU has its own corresponding intra luma PU. This can be viewed as a bug, though it does not occur in the tests under the common test conditions with the SCU size set to 8x8.

In HEVC text specification draft 7 [1], an intra CU always sends one intra chroma prediction mode for each PU, not CU. When an intra CU is an SCU and NxN PU partition mode is selected, four intra chroma prediction modes are coded, which is inconsistent with HM-7.0 software that always sends only one intra chroma prediction mode for the entire intra CU. In this contribution, we propose two methods to resolve this inconsistency.

The rest of this contribution is organized as follows. The proposed methods will be described in Section 2, simulation results will be shown in Section 3, and conclusions will be given in Section 4.

Figure 1.  Illustration of DM mode derivation

# Proposed methods

In JCTVC-I0302 [2] part 1, it is proposed to remove the mismatch between HM software and HEVC text specification as described in Section 1 by changing the specification text to match the HM software. This part was adopted in the 9th JCT-VC meeting in Geneva, but the specification text has not yet fixed accordingly. Although adopting JCTVC-I0302 part 1 can remove the mismatch, method 1 follows more or less what has been implemented in the HM-7.0 software and further includes a bug-fix, which is not addressed in JCTVC-I0302, to let the four chroma PUs refer to the four different luma PUs when the intra CU is an SCU larger than 8x8 in luma and DM mode and NxN PU partition mode are selected. Method 2 follows the HEVC text specification draft 7 but includes a bug fix to send only one intra chroma prediction mode when the intra CU is an SCU equal to 8x8 in luma and DM mode and NxN PU partition mode are selected.

## Method 1: One intra chroma prediction mode per CU

In method 1, each intra CU signals one intra chroma prediction mode as in the HM-7.0 software. One further modification is proposed as follows. When there are four chroma PUs in the intra CU, each chroma PU follows the intra luma prediction mode of the corresponding intra luma PU for DM mode. While there is only one chroma PU corresponding to four luma PUs in the intra CU, the chroma PU follows the intra luma prediction mode of the upper-left luma PU for DM mode, as shown in Figure 1(a). There is no syntax change for the software with this method.

## Method 2: One intra chroma prediction mode per PU

In method 2, each intra chroma PU signals its own intra chroma prediction mode as in the text specification. However, there is one bug in the text specification. If an intra CU is an SCU equal to 8x8 in luma and NxN PU partition mode is selected, only one intra chroma prediction mode needs to be signaled, while the text specification still specifies four intra chroma prediction modes to be signaled.

Since the intra chroma prediction mode contains context coded bins and bypass bins, the bypass bins of multiple intra chroma prediction modes in an Intra_NxN SCU are further collected together. The grouping of bypass bins is also used in the MVD coding and the last significant position coding.

# Simulation results

Since the common test conditions defined in JCTVC-I1100 [3] set the SCU size to 8x8 in luma, the proposed methods cannot cause any difference. Therefore, experiments were conducted according to the common test conditions but with the SCU size set to 16x16 in luma. The software is based on HM-7.0.

Table 1 shows the results of testing HM-7.0 with the SCU size set to 16x16 in luma using HM-7.0 with the SCU size set to 8x8 in luma as the anchor. It can be observed that 3.2-4.4% luma bit rate increase and 1.5-4.1% chroma bit rate increase are introduced by increasing the SCU size from 8x8 to 16x16 in luma.

Table 2 shows the results of the method 1, where both the anchor and the proposed method set the SCU size to 16x16. The method 1 can achieve 0.6% luma coding gain and 0.9% chroma coding gain in Main-AI, and 0.1% luma coding gain and 0.6% chroma coding gain in Main-RA and Main-LB.

Table 3 shows the results of the method 2, where both the anchor and the proposed method set the SCU size to 16x16. The method 2 can achieve 0.4% luma coding gain and 1.7% chroma coding gain in Main-AI, 0.1% luma coding gain and 2.3% chroma coding gain in Main-RA, and 0.1% luma coding gain and 0.7% chroma coding gain in LDB-main.

As expected, both methods only have minor impacts on the coding efficiency but both reasonably solve the problem of the mismatch between HM-7.0 software and the text specification.

# Conclusions

In this contribution, two methods are proposed to fix a mismatch between HM-7.0 software and the text specification. Simulation results reportedly show no difference for common test conditions and slight coding gains when the SCU size is set to 16x16 in luma. We recommend adopting method 1, which is following the decision in the 9th JCT-VC meeting with a bug-fix, to resolve the mismatch problem. Method 2 provided the information of another implementation for solving this bug.

# References

- “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- W.-J. Chien, J. Chen, M. Coban, M. Karczewicz, “Intra mode coding for INTRA_NxN,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0302, May, 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of HM-7.0 with SCU=16x16 (Test) versus HM-7.0 with SCU=8x8 (Anchor)

Table 2.  Results of method 1 with SCU=16x16 (Test) vs. HM-7.0 with SCU=16x16 (Anchor)

Table 3.  Results of method 2 with SCU=16x16 (Test) vs. HM-7.0 with SCU=16x16 (Anchor)