| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0053 |
| --- | --- |

| Title: | Residue scan for intra transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jicheng An, Liang Zhao North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {jicheng.an, liang.zhao, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a residue scan is proposed for the intra transform skip mode to better adapt to the different energy distribution of the residual signal. The proposed residue scan order is the inverse order of the transform scan. It is reported that the proposed method can achieve 1.6%, 1.1%, and 0.7% average luma coding gains in terms of BD-rate for class F under HE10-AI, HE10-RA, and HE10-LB conditions, respectively, and provide corresponding average luma coding gains for other classes as 0.2%, 0.1%, and 0.0%. Moreover, the proposed method can be easily implemented by modifying only two code lines, which implies a minor complexity impact.

# Introduction

In the current HEVC (text specification draft 7) [1], an intra transform skip mode proposed in JCTVC-I0408 [2] has been adopted to improve the coding efficiency, especially for the class F sequences. For a 4x4 intra transform unit (TU), the transform skip mode can be enabled. For the transform skipping TUs, a simple scaling process is used instead of the transform.

For a 4x4 intra TU, no matter whether the transform skipping is used or not, the scan order and coding method of quantized residual are exactly the same. There are three kinds of quantized residue scan orders for 4x4 TUs according to intra prediction modes: diagonal scan, horizontal scan, and vertical scan as depicted in Figure 1. Table 1 shows the mapping between intra prediction modes and scan orders.

Figure 1.  Different scan orders for 4x4 intra TUs; left: diagonal scan; middle: vertical scan; right: horizontal scan

Table 1.  Mapping between intra prediction mode and scan order

|  | IntraPredMode | ScanOrder |
| --- | --- | --- |
|  | 0 | Diagonal |
|  | 1 | Diagonal |
|  | 2 – 5 | Diagonal |
|  | 6 – 14 | Vertical |
|  | 15 – 21 | Diagonal |
|  | 22 – 30 | Horizontal |
|  | 31 – 35 | Diagonal |

# Proposed method

Since the energy distribution of quantized residual is different when transform is skipped, it is reasonable to consider a different scan order to achieve better entropy coding. It turns out that a simple inverse scan order when transform is skipped can improve coding efficiency significantly.

For the intra prediction modes which use diagonal scan, the magnitudes of the prediction residues are usually larger at the bottom-right corner and decrease from bottom-right corner to top-left corner as the prediction distance is shortened. That energy distribution is inversed compared to the case of transform-on, in which the energy is supposed to be compacted to the top-left corner. Therefore, we propose to use an inverse diagonal scan as shown in the left part of Figure 2 when transform is skipped for the intra prediction modes which originally use diagonal scan.

Similarly, for the intra prediction modes which use vertical scan, the magnitudes of the prediction residues are usually larger near the right boundary and decrease from right to left as the prediction distance is shortened. That energy distribution is inversed compared to the case of transform-on, in which the energy is supposed to be compacted to the left lines. Therefore, an inverse vertical scan as shown in the middle part of Figure 2 is better when transform is skipped for the intra prediction modes which originally use vertical scan. Similar arguments can be applied to the cases when the horizontal scan is used originally.

Figure 2.  Proposed scan orders for 4x4 intra PUs when transform is skipped; left: inversed diagonal scan; middle: inversed vertical scan; right: inversed horizontal scan

# Implementation

The implementation of the proposed method can be done easily at the scaling stage with an inverse scan order. For the software implementation in HM-7.0, we only need to modify two lines: one in the forward scaling and one in the backward scaling.

In forward scaling, the simple change is as follows.

- Original code in function Void TComTrQuant::xTransformSkip():

psCoeff[j * height + k] = piBlkResi[j * uiStride + k] << iTransformShift;

- Proposed code in function Void TComTrQuant::xTransformSkip():

psCoeff[j * height + k] = piBlkResi[(height-1-j) * uiStride + (width-1-k)] << iTransformShift;

In backward scaling, the simple change is as follows.

- Original code in function Void TComTrQuant::xITransformSkip():

pResidual[j * uiStride + k] = (plCoef[j * width + k] + iAdd) >> iTransformShift;

- Proposed code in function Void TComTrQuant::xITransformSkip():

pResidual[j * uiStride + k] = (plCoef[(height-1-j) * width + (width-1-k)] + iAdd) >> iTransformShift;

# Simulation results

The proposed method has been implemented on top of HM-7.0. The simulation has been done according to the HE10 common test conditions [3], since the intra transform skip mode is only turned on in HE10 conditions. Results are shown in Table 2. It can been seen that the proposed residue scan can provide 1.6%, 1.1%, and 0.7% average luma coding gains for class F for HE10-AI, HE10-RA, and HE10-LB, respectively. For other classes, the corresponding average luma coding gains are 0.2%, 0.1%, and 0.0%, respectively.

Table 2.  Results of the proposed method

# Conclusion

In this contribution, the scan order is inversed for intra transform skip mode to adapt better to the different energy distribution of residues. It is reported that the proposed method can achieve 1.6%, 1.1%, and 0.7% average luma coding gains in terms of BD-rate for class F under HE10-AI, HE10-RA, and HE10-LB conditions, respectively, and provide corresponding average luma coding gains for other classes as 0.2%, 0.1%, and 0.0%, respectively. Moreover, the proposed method can be implemented easily by modifying only two code lines, which implies a minor complexity impact.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- B. Bross, W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- C. Lan, J. Xu, G. J. Sullivan, F. Wu, “Intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0408, April, 2012.

- F. Bossen, “Common test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.