| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0054 |
| --- | --- |

| Title: | AhG6: Bypass bins grouping in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, In Suk Chong, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In HM7.0, SAO syntax elements coded in bypass are the left band position and the offset signs. These elements are not grouped together since sao_offset, which is coded using regular bins, is signaled between them. It is proposed to change the coding order so that band offset is coded after the SAO offset sign, thus grouping bypass bins together. There is no performance impact for this change (0.00% for all configurations).

# Technical Description

A design principle applied to HEVC in order to increase throughput has been the grouping of bypass bins. This principle has been applied to coefficient level coding, last position coding, and intra mode coding, among others. For SAO, it is also possible to group bypass bins together. Table 1 shows the coding order of SAO syntax elements and the bin type in HM7.0:

| Syntax element | Bin Type |
| --- | --- |
| sao_type_idx | Regular |
| if( sao_type_idx = = 5 ) sao_band_position | Bypass |
| sao_offset | Regular |
| if( sao_type_idx = = 5 ) sao_offset_sign | Bypass |

Table 1. HM7.0 SAO syntax elements and corresponding bin type

The order of these syntax elements is modified by signaling the sao_band_offset after sao_offset_sign to group bypass bins:

| Syntax element | Bin Type |
| --- | --- |
| sao_type_idx | Regular |
| sao_offset | Regular |
| if( sao_type_idx = = 5 ) sao_offset_sign | Bypass |
| if( sao_type_idx = = 5 ) sao_band_position | Bypass |

Table 2. Proposed ordering of SAO syntax elements

In addition, using the proposed coding order, sao_offset is always coded right after sao_type_idx. Therefore, the dependency of the syntax to be coded after sao_type_idx depending on the value of sao_type_idx is removed.

# Simulation results

The proposed method is implemented in HM7.0 and simulated under the common test conditions. Performance is the same, since it is only a different coding order of the same data.

Source code is included in this package.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 105% |  |  | 103% |  |  |

Table 1. Results for the proposed method

# Conclusions

A simple change in the coding order for SAO following the design principles commonly used in the development of HEVC is proposed to enable higher throughput. There is no impact on the performance.

# Text for Working Draft

7.3.4.2 Sample adaptive offset CABAC syntax

| sao_offset_cabac( rx, ry, cIdx ) { | Descriptor |
| --- | --- |
| sao_type_idx[ cIdx ][ rx ][ ry ] | ae(v) |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| if( sao_type_idx[ cIdx ][ rx ][ ry ] = = 5 ) { |  |
| for( i = 0; i < 4; i++ ) { |  |
| if( sao_offset[ cIdx ][ rx ][ ry ] ! = 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| } |  |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).