| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0059-r2 |
| --- | --- |

| Title: | AHG4: Suggested constraint on number of bits/bins per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

____________________________

# Abstract

The current HEVC specification does not constrain the maximum number of bits per LCU. Having no constraint on the number of bits per LCU imposes a burden on decoder implementation. This contribution investigates LCU bit/bin numbers of a coded synthesized test sequence with Gaussian white noise characteristics. It is reportedly shown that, for QP=0, 4, 8, and 12 RA-Main results, an LCU bit expansion ration ranges from 1.1 to 1.8 and that an LCU bin/bit ratio ranges from 1.1 to 1.2. It is suggested that either of the following strawman proposals on the maximum number of bits/bin per LCU is considered for the DIS text:

- The number of bits generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

- The number of bins generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25*1.2, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

# Background

Unlike AVC, the current HEVC specification does not constrain the maximum number of bits per LCU. It is reported on the jctvc e-mailing list that an LCU can be expanded by up to 7.4x in evil cases. Although such a expansion may be unlikely, the evil LCUs are compliant; HEVC decoder must be able to decode them. Therefore having no constraint on the number of bits per LCU imposes a burden on decoder implementation. Experts express opinions that HEVC should define an AVC-like constraint on the number of bits per LCU or a constrain on the number of bins per LCU.

Regarding the constraint in terms of the number of bits, there are opinions that 20%-30% margin of the original data size should be considered since AVC's margin of 4% (a margin of 128 bits against the original 384*8 bits) may be too tight. Regarding the constraint on the maximum number of bins per LCU, there are suggestions that an appropriate scaling factor is applied to the maximum number of bits with some margin respect to the original data size. It was suggested that the appropriate scaling factor can be determined by seeing the behaviour of the ratio of bit to bin of Gaussian white noise which typically yields a huge amount of both bits and bins.

This contribution reports statistics of LCU bit/bin numbers for a synthesized test sequence containing Gaussian white noise. Then a strawman proposal on the margin of the original data size as well as the bin/bit scaling factor is presented.

# Simulation

2.1		Setup

A synthesized test sequence coded by HM-7.0 is depicted in Figure 1. The synthesized test sequence is Gaussian noise, which is difficult to compress by using prediction, transform coding, and entropy encoding. Some real-world video can partially contain textures whose natures are similar to Gaussian noise. (“JCTVC-J0059-Sandstorm-Software.zip” is a software that generates the synthesized test sequence. )

Figure 1: Synthesized test sequence, Sandstorms, (640x240 30Hz 30frames)

Modified versions of the RA-Main and RA-HE10 settings in JCTVC-I1100 are used: QP=0,4,8,and 12 are used for the RA-Main setting, and QP=-12,-8,-4 and 0 are sued for the RA-HE10 setting; LCU size is varied from 16x16 to 64x64; QPoffset values are set to 0 in order to see the behavior in a given QP value; transform skipping on/off are tested since the tool is expected to reduce the worse-case bits. ("JCTVC-J0059-cfg.zip" and " JCTVC-J0059-bat.zip" contain the configuration files and MS-DOS batch files, respectively. )

The numbers of bits and bins of LCUs are gathered by a modified version of AHG5 tool that collects the statistics of bits and bins. " JCTVC-J0059-HM-7.0-decoder.patch" is the modified patch.

2.2		Results

Simulation results for different LCU sizes and transform-skipping settings are shown in "JCTVC-J0059-statistics.xlsx." Since the results for different LCU sizes show a very similar tendency, the summary of simulation results for the 64x64 LCU size (of Intra slices) is shown as a representative results in Table 1.

Table 1: Summary results for 64x64 LCU size

In Table 1, "Bin/bit ratio" column shows the ratio of the number of LCU bins to the number of LCU bits. A value greater than 1 in that column indicates that CABAC compresses input bins. "Data expansion ratio" column shows the ratio of the number of LCU bits to the original data size (for RA-HE10, the original data size is set equal to the 10-bit version). A value greater than 1 in that column indicates that HM-7.0 expands the original data. Note that the LCU bin and bit values are on average but their tight distribution is confirmed; the average values are reasonable indicator to see the general bin/bit behavior of the worst cases.

From Table 1, the following are observed:

- Regardless of the use of transform skipping, data expansion happens even when QP=12 and QP=0 for RA-Main and HE-10, respectively; and

- Bin/bit ratio ranges from 1.08 to 1.20 and is less than 1.3 since the majority of bins is bypass coded in the tested small QPs.

# Discussions

The simulation results showed that the synthesized test sequence can result in 1.1x data expansion even when QP is equal to 12. (Note that the use of QP=4 in 8-bit video coding means that the quantization step size equal to 1 is applied to prediction residuals. Thus the use of QP=12 means that the quantization step size equal to 2.5 is applied to prediction residuals) If we define the same bit limit as AVC (up to 1.04x), encoding with a low QP of 12, which can be used for encoding low complex 8-bit video at high bit rates in consumer products, may result in the limit violation if some parts of the input video contains texture whose characteristic is similar to that of Gaussian white noise. By considering that re-encoding of many LCUs (even with I_PCM) can be challenging for some consumer encoders and that such a minor data expansion can be handled by decoders with an acceptable cost increase, we believe that the bit limit per LCU can be relaxed compared to AVC.

In the jctvc mailing list, 20%-30% margin of the original data size was proposed. We support the idea and believe that 25% margin is reasonable. So the relaxed constraint on the maximum number of bits per LCU can be specified as follows:

- The number of bits generated by an LCU shall not exceed (N>>3)*(N>>3)*(Num16x16LCUSample*BitDepth)*1.25, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

Regarding the constraint on the number of bins per LCU, we believe that a scaling factor of 1.2 for the bin/bit ratio is a reasonable value according to the simulation results. The constraint on the maximum number of bins per LCU can be specified as follows:

- The number of bins generated by an LCU shall not exceed (N>>3)*(N>>3)*(Num16x16LCUSample*BitDepth)*1.25*1.2, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

# Conclusions

Simulation results have shown that, for QP=0, 4, 8, and 12 RA-Main results, an LCU bit expansion ratio ranges from 1.1 to 1.8 and that an LCU bin/bit ratio ranges from 1.1 to 1.2. It is suggested that either of the following strawman proposals on the maximum number of bits/bin per LCU is adopted into the DIS text:

- The number of bits generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

- The number of bins generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25*1.2, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

# Patent rights declaration

NEC Corporation does not have any current or pending patent rights relating to the technology described in this contribution.

# DIS text: Bit-size limit high-lighted in Magenta

A.3.2	Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 1 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- The number of bits of coding_tree( ) data for any coding tree block shall not be greater than 480*(bit_depth_luma_minus8 + 8) *( 1 << (Log2CtbSize- 4))*( 1 << (Log2CtbSize- 4)).

- When tiles_or_entropy_coding_sync_idc is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

[Ed. (KM): Confirm that this is the best formulation]

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to 1 and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level.

# DIS text : Bin-size limit high-lighted in Magenta

A.3.2	Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 1 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- The number of times that the parsing process function DecodeBin( ), specified in subclause 9.3.3.2, is invoked to decode coding_tree( ) data for any coding tree block shall not be greater than 576 *(bit_depth_luma_minus8 + 8) *( 1 << (Log2CtbSize- 4))*( 1 << (Log2CtbSize- 4)).

- When tiles_or_entropy_coding_sync_idc is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

[Ed. (KM): Confirm that this is the best formulation]

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to 1 and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level.