# DIS text: Bit-size limit high-lighted in Magenta

A.3.2	Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 1 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- The number of bits of coding_tree( ) data for any coding tree block shall not be greater than 768*(bit_depth_luma_minus8 + 8) *( 1 << (Log2CtbSize- 4))*( 1 << (Log2CtbSize- 4)).

- When tiles_or_entropy_coding_sync_idc is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

[Ed. (KM): Confirm that this is the best formulation]

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to 1 and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level.

# DIS text: Bit-size limit high-lighted in Magenta (The meaning is the same as 1. The description is strict)

A.3.2	Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- Sequence parameter sets shall have chroma_format_idc equal to 1 only.

- Sequence parameter sets shall have bit_depth_luma_minus8 equal to 0 only.

- Sequence parameter sets shall have bit_depth_chroma_minus8 equal to 0 only.

- Sequence parameter sets shall have adaptive_loop_filter_enabled_flag equal to 0 only.

- Sequence parameter sets shall have chroma_pred_from_luma_enabled_flag equal to 0 only.

- Sequence parameter sets shall have asymmetric_motion_partitions_enabled_flag equal to 1 only.

[Ed. (KM): Confirm that this flag has been defined. (GJS): It is now in the SPS, but not yet connected to the decoding process.]

- Sequence parameter sets shall have nsrqt_enabled_flag equal to 0 only.

- Sequence parameter sets shall have seq_parameter_set_id in the range of 0 to 15, inclusive.

- Log2CtbSize shall be in the range from 4 to 6, inclusive.

- SliceGranularity shall be equal to 0.

- Picture parameter sets shall have tiles_or_entropy_coding_sync_idc in the range of 0 to 2, inclusive.

- Picture parameter sets shall have dependent_slice_enabled_flag equal to 0.

- The number of times read_bits( 1 ) called in subclauses 9.3.3.2.2 and 9.3.3.2.3 when parsing coding_tree( ) data for any coding tree block shall not be greater than 768*(bit_depth_luma_minus8 + 8) *( 1 << (Log2CtbSize- 4))*( 1 << (Log2CtbSize- 4)).

- When tiles_or_entropy_coding_sync_idc is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 384 for any i in the range of 0 to num_tile_columns_minus1, inclusive.

[Ed. (KM): Confirm that this is the best formulation]

- Picture parameter sets shall have pic_parameter_set_id in the range of 0 to 63, inclusive.

- Adaptation parameter sets shall have aps_id in the range of 0 to 63, inclusive.

- The level constraints specified for the Main profile in subclause Error: Reference source not found shall be fulfilled.

Conformance of a bitstream to the Main profile is indicated by profile_idc being equal to 1.

Decoders conforming to the Main profile at a specific level (identified by a specific value of level_idc) shall be capable of decoding all bitstreams in which the profile_idc is equal to 1 and level_idc represents a level lower than or equal to the specified level. For purposes of comparison of level capabilities, a particular level shall be considered to be a lower level than some other level if the level_idc of the particular level is less than the level_idc of the other level.