| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0060 |
| --- | --- |

| Title: | AHG5: Simplified cu_qp_delta coding with EG0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a simplified cu_qp_delta coding based on EG0 binarization. The simplified scheme maps a given cu_qp_delta into the unsigned value and represents it with the EG0 binary string. Only the first bin of the EG0 binary sequence is context-coded; the rest are bypass-coded. Thus it reduces the number of context-coded bins of cu_qp_delta in worst-case scenario from 11% to 2%. It is reportedly shown that the simplified scheme changes BD-rate results by 0.20% on average in AHG5 common test conditions.

# Introduction

In the last JCT-VC meeting, AHG5 on Entropy Coding Improvements was established. Its mandates include the study of the number of context coded bins of syntax elements of ref_idx_lx, cu_qp_delta, etc. in the worst-case scenario. In our study, it was confirmed that the number of context-coded bins of ref_idx_lx syntax elements takes about 21% of the context-coded bins of a 16x16 inter-coded LCU in worst-case scenario and that the number of context-coded bins of cu_qp_delta syntax elements takes about 11% (and 8%) of the context-coded bins of a 16x16 intra-coded LCU (and 16x16 inter-coded LCU) in worst-case scenario.

This contribution describes a simplified cu_qp_delta coding based on EG0 binarization. The simplified scheme maps a given cu_qp_delta into the unsigned value and represents it with the EG0 binary string Only the first bin of the EG0 binary sequence is context-coded; the rest are bypass-coded. Thus it can reduce the number of context-coded bins of cu_qp_delta in worst-case scenario by 9%. Simulation results show that the simplified scheme changes BD-rate results negligibly (about 0.20% on average).

# Simplified cu_qp_delta coding with EG0 binarization

2.1		Algorithm description

The proposed scheme maps the signed value of a given cu_qp_delta into the unsigned value and represents it with the EG0 binary string. Table 1 shows the relationship between cu_qp_delta and EG0 binary string, where a variable code_idx is the unsigned value of a given cu_qp_delta and is defined by:

code_idx = 2 * Abs ( cu_qp_delta ) - (cu_qp_delta > 0 ).

Table 1	Relationship between cu_qp_delta and EG0 binary string.

As it can be seen from Table 1, the first and last bins indicate the significance and sign of the cu_qp_delta respectively and the rest conveys the magnitude information. Since it is likely that only the distribution of cu_qp_delta significance is biased when typical rate-control or subjective block-level QP adaptation is used, a context is used only for coding the 1st bins.

2.2		Analysis

The current HM converts a cu_qp_delta into a binary string in which the first and second bins indicate the significance and sign respectively and the rest represents the magnitude minus 1. Table 2 shows the relationship between cu_qp_delta and HM-6.0 binary string. Since only the 2nd bins are bypass-coded, the number of context-coded bins per cu_qp_delta is proportional to the maximum absolute value of an admissible cu_qp_delta range.

Table 2	Relationship between cu_qp_delta and HM-6.0 TU binary string.

By comparing the Table 1 and Table 2, the EG0 based cu_qp_delta coding offers the following advantages over the current HM in terms of complexity:

- Reduce the maximum numbers of bins from 27 to 11;

- Reduce the maximum numbers of context bins from 26 to 1;

- No interleaving of context-coded bins and bypass-coded bins.

# Simulation results

Simulation was conducted according to the AHG5 common test conditions based on JCTVC-I1100 using additional command options "--MaxCuDQPDepth=3 --AdaptiveQP=1 --MaxQPAdaptationRange=12."

Attached Excel sheet, JCTVC-J0060-AHG5-results.xls, shows the detailed results. Table3 shows the summary and the followings are observed:

- Average BD-rate decreases of 0.19% for All Intra Main, 0.19% for Random Access Main, 0.20% for Low Delay B Main, and 0.23% for Low Delay P Main;

- Average BD-rate decreases of 0.19% for All Intra HE10, 0.20% for Random Access HE10, 0.20% for Low Delay B HE10, and 0.17% for Low Delay P HE10.

Table 3	Summary of simulation results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.16% | 0.09% | 0.09% | 0.16% | 0.01% | 0.08% |
| Class B | 0.17% | 0.08% | 0.10% | 0.17% | 0.09% | 0.09% |
| Class C | 0.21% | 0.17% | 0.18% | 0.21% | 0.16% | 0.18% |
| Class D | 0.18% | 0.14% | 0.18% | 0.18% | 0.14% | 0.15% |
| Class E | 0.25% | 0.15% | 0.21% | 0.26% | 0.22% | 0.26% |
| Overall | 0.19% | 0.12% | 0.15% | 0.19% | 0.12% | 0.14% |
|  | 0.19% | 0.12% | 0.15% | 0.19% | 0.12% | 0.14% |
| Class F | 0.20% | 0.26% | 0.22% | 0.23% | 0.29% | 0.07% |
| Enc Time[%] | 95% |  |  | 98% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.17% | 0.18% | 0.22% | 0.17% | 0.10% | 0.28% |
| Class B | 0.19% | 0.04% | 0.18% | 0.20% | 0.00% | 0.07% |
| Class C | 0.22% | 0.07% | 0.43% | 0.19% | 0.22% | 0.17% |
| Class D | 0.19% | 0.36% | 0.37% | 0.22% | 0.22% | -0.08% |
| Class E |  |  |  |  |  |  |
| Overall | 0.19% | 0.15% | 0.29% | 0.20% | 0.13% | 0.11% |
|  | 0.20% | 0.17% | 0.27% | 0.20% | 0.12% | 0.14% |
| Class F | 0.10% | 0.08% | 0.06% | 0.22% | -0.08% | 0.21% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.20% | 0.05% | 0.07% | 0.22% | 0.14% | 0.50% |
| Class C | 0.22% | 0.08% | 0.23% | 0.24% | 0.00% | 0.16% |
| Class D | 0.25% | 0.12% | 0.69% | 0.23% | 0.07% | 0.25% |
| Class E | 0.08% | 0.05% | 0.36% | 0.07% | -1.18% | 1.54% |
| Overall | 0.20% | 0.08% | 0.32% | 0.20% | -0.16% | 0.55% |
|  | 0.20% | 0.11% | 0.30% | 0.20% | -0.10% | 0.58% |
| Class F | 0.28% | 0.12% | -0.19% | 0.38% | 0.39% | 0.50% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 92% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.22% | 0.20% | -0.23% | 0.24% | 0.18% | 0.51% |
| Class C | 0.29% | 0.16% | 0.47% | 0.23% | 0.19% | -0.05% |
| Class D | 0.26% | 0.27% | -0.14% | 0.20% | 0.25% | 0.66% |
| Class E | 0.09% | -0.31% | 0.68% | 0.07% | -0.83% | 1.01% |
| Overall | 0.23% | 0.28% | 0.06% | 0.17% | -0.07% | 0.62% |
|  | 0.23% | 0.25% | 0.05% | 0.18% | -0.02% | 0.64% |
| Class F | 0.25% | 0.95% | -0.26% | 0.10% | -0.40% | 1.08% |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 89% |  |  |

By using the AHG5 decoder patch, the cu_qp_delta statistics about context coded bins, bypass bins, and bit rates are collected and summarized in Table 4. The followings are observed:

- The number of context coded-bin ratio is reduced from 3.34% to 1.15% in the Main setting;

- The number of bypass coded-bin ratio is increased from 2.53% to 8.53% in the Main setting;

- Bit rates of cu_qp_delta is increased by about 5%.

Since the bit rates of cu_qp_delta are used during RDO based mode decision, the selected block modes as well as cu_qp_delta distributions of Reference and Tested results are different. However, the statistics verifies that the proposed EG0 scheme reduces the context coded bins while increasing the bypass coded bins even in the AHG5 common test conditions.

Table 4 Statistics of context coded bins, bypass bins, and bit rates of cu_qp_delta.

# Conclusions

This contribution has presented a simplified cu_qp_delta coding based on EG0 binarization. The simplified coding scheme offers the following advantages over the current HM in terms of complexity:

- Reduce the maximum numbers of bins from 27 to 11;

- Reduce the maximum numbers of context-coded bins from 26 to 1;

- No interleaving of context-coded bins and bypass-coded bins.

It is proposed that if simplification of the current cu_qp_delta coding is required, the presented EG0 binarization scheme is considered for the candidate.

# Patent rights declaration

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Changes to the CD text

The following portion of Table 9-4

"

| coding_unit() | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |

"

is replaced by

"

| coding_unit() | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |

"

The following Table

"

Table 9-3 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 | 119 |

"

is replaced by

"

Table 9-4 – Values of variable initValue for cu_qp_delta ctxIdx

| Initialization variable | cu_qp_delta ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 119 | 119 | 119 |

"

The following portion of Table 9-34

"

| cu_qp_delta | 0 | as specified in subclause | 2 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |
|  | 1 |  | 2 | Error: Reference source not found | 3 |
|  | 2 |  | 2 | Error: Reference source not found | 6 |

"

is replaced by

"

| cu_qp_delta | 0 | as specified in subclause | 0 | Error: Reference source not found | 0 |
| --- | --- | --- | --- | --- | --- |

"

The "9.2.2.6	Binarization process for cu_qp_delta" is replaced by

"

#### 9.2.2.6		Binarization process for cu_qp_delta

Input to this process is a request for a binarization for the syntax element cu_qp_delta.

Output of this process is the binarization of the syntax element.

A signed syntax element value sSynVal is mapped into an unsigned syntax element value uSynVal by uSynVal = 2 * Abs( sSynVal ) − ( sSynVal > 0 ). The bin string of the uSynVal is derived using the EG0 binarization as specified in subclause 9.2.2.4.

The bin string of this binarization process of a syntax element value synVal is specified by a process equivalent to the following pseudo-code.

absV = Abs( synVal ) 
if( absV = = 0 )
	put( 0 ) 
else {
	put( 1 ) 
	signV = ( synVal > 0 ) ? 0 : 1
	put( signV ) 
	cMax = 24 + ( QpBdOffsetY >> 1 ) + signV
	absVGreaterThan1Flag = ( absV = = 1 ) ? 0 : 1
	put( absVGreaterThan1Flag ) 
	if( absVGreaterThan1Flag ) { 
	cNum = absV − 2
		while( cNum− − ) 
			put( 1 ) 
		if( cMax > absV − 1 )
			put( 0 )
	}
}