| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0061 |
| --- | --- |

| Title: | AHG11: Study report of HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com |
| Source: | NEC Corporation |  |  |

____________________________

# Abstract

In the last JCT-VC meeting, the new lossless scheme (JCTVC-I0529) that explicitly signals a lossless flag in each CU was adopted. This contribution evaluate the new lossless coding scheme and the previous scheme that does not require the explicit signalling. It is reportedly shown that the side information associated with the CU-level flags increases the overall bit rates by 0.01%.

# Background

In the previous HM, lossless coding of a CU was implicitly signalled by the combination of a sequence level flag and QP’Y. A CU is decoded by skipping inverse-scaling, inverse-transform, and in-loop filter processes when the sequence level flag is equal to one and QP’Y is equal to zero. Although the implicit signalling does not require CU-level side information transmission, it results in encoder-decoder mismatches when cu_qp_delta is not present and QP’Y is not identical to its prediction.

In order to deal with the mismatch problem, JCTVC-I0529 proposed a PPS flag to indicate the presence of CU-level flag that signals whether the associated CU is decoded by skipping inverse-scaling, inverse-transform, and in-loop filter processes; the proposal was adopted into the current HM.

This contribution reports the simulation results of the new lossless coding scheme and the previous scheme.

# Simulation

2.1		Setup

The common test sequences are coded by using HM-7.0-dev rev.2472 with a modified JCTVC-I1100 Main setting. The modifications to the original configuration files are as follows:.

- DeblockingFilterControlPresent	1

- LoopFilterOffsetInPPS		1

- LoopFilterDisable			1

- SAO					0

- LosslessCuEnabled			1

- TransquantBypassEnableFlag	1

- CUTransquantBypassFlagValue	1

In HM-7.0-dev rev.2472, the following macro setting enables the previous lossless coding:

#define CU_LEVEL_TRANSQUANT_BYPASS        0

In the next subsection, the original HM-7.0-dev rev.2472 and its macro-modified version are evaluated as "Reference" and "Tested", respectively.

2.2		Results

Attached Excel sheet " JCTVC-J0061-results.xls" shows the detailed results. Table 1 shows the summary results. In Error: Reference source not found, "compression ratio" column shows the ratio of the original video bit rate to the compressed video bit rates. A value greater than 1 in that column indicates that the scheme compresses the input video. It is confirmed that the side information associated with the CU-level flags increases the overall bit rates by 0.01% on average and is negligible. (Note that the side information increase is up to 11Kbps in Class F sequences,)

Table1: Summary results

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2.0 | 2.0 | -0.01% |
| Class B | 2.0 | 2.0 | -0.01% |
| Class C | 1.9 | 1.9 | -0.01% |
| Class D | 1.7 | 1.7 | -0.01% |
| Class E | 2.6 | 2.6 | -0.01% |
| Overall (w/o F) | 2.0 | 2.0 | -0.01% |
| Overall (w/ F) | 2.4 | 2.4 | -0.01% |
| Class F | 4.3 | 4.3 | -0.01% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2.3 | 2.3 | -0.01% |
| Class B | 2.3 | 2.3 | 0.00% |
| Class C | 2.4 | 2.4 | -0.01% |
| Class D | 2.6 | 2.6 | -0.01% |
| Class E |  |  |  |
| Overall (w/o F) | 2.4 | 2.4 | -0.01% |
| Overall (w/ F) | 7.1 | 7.1 | -0.01% |
| Class F | 26.9 | 26.9 | -0.02% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A |  |  |  |
| Class B | 2.3 | 2.3 | 0.00% |
| Class C | 2.4 | 2.4 | 0.00% |
| Class D | 2.6 | 2.6 | -0.01% |
| Class E | 3.1 | 3.1 | -0.01% |
| Overall (w/o F) | 2.6 | 2.6 | -0.01% |
| Overall (w/ F) | 11.0 | 11.0 | -0.01% |
| Class F | 44.5 | 44.6 | -0.01% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

# Conclusion

Simulation results have verified that the current HEVC lossless coding scheme increases only 0.01% of overall bit rates by the use of explicit CU-level side information.

# Patent rights declaration

NEC Corporation does not have any current or pending patent rights relating to the technology described in this contribution.