| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0062 |
| --- | --- |

| Title: | SAO CABAC Context Initialisation to Reduce Parallel Encoding Losses |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gordon CLARE (Orange Labs), Félix HENRY (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution presents a change in the initialization of SAO-related CABAC contexts in order to reduce the random-access losses incurred when moving from a non-parallel encoding to a parallel encoding.    When comparing a the non-parallel reference to a WPP parallel encoding, these losses change from +0.2%, +1.3%, +2.2%, +2.1% (AI, RA, LB, LP) with HM7 to +0.2%, +1.1%, +1.9%, +1.9% when using the proposal.  When comparing the non-parallel reference to a  “class D sized” tiles encoding, these losses change from +1.2%, +2.2%, +2.9% (AI, RA, LB) with HM7 to +1.2%, +1.8%, +2.3% when using the proposal.  This reduction in loss gives figures that start to approach previous HM 6.1 losses of +0.2%, +0.9%, +2.3%, +2.2% (reference vs WPP) and +1.2%, +1.4%, +2.9% (reference vs class D sized tiles).

# Introduction

It has been noticed that when comparing HM6.1 to HM7 there has been an increase in the losses incurred when moving from a non-parallel RA-MAIN encoding to a ‘parallel’ RA-MAIN encoding (ie, moving from a reference encoding to either a ‘WPP’ encoding or a ‘tiles’ encoding).  Other profiles did not exhibit such an increase in loss.

Around half of this increase in random access loss can be avoided if SAO CABAC contexts, newly used in HM7, are initialized differently.  Prior to HM7, SAO information was encoded using CAVLC in a separate APS packet.  From HM7, SAO information is encoded in the slice information using dedicated CABAC contexts.

Encoding methods allowing parallelism that either partially or completely re-initialize CABAC contexts are somewhat sensitive to badly initialized contexts.  With WPP there is an inheritance of partially trained CABAC contexts at the start of each LCU line, so the overall adaptation of a badly initialized context is slower than with a non-parallel encode.  With tiles there is a complete reset of CABAC contexts per tile, so losses induced by a badly initialized context are expected to be higher than WPP losses for an equivalent degree of parallelism.  Although slices have not been investigated here, it is expected that losses associated with slices would be similar to those of tiles.

We are proposing a change of the initialization of SAO-related CABAC contexts to simple values that reduce this increase in loss, particularly for the RA-MAIN profile.

# Proposed Context Initializations

The proposed SAO context initializations are shown below in table 1.  Values are only changed for B & P slices (both using these same values); I-slice initializations are not changed.  A “0,0” indicates a slope and offset of 0, and is used when an MPS=0 is initially expected at maximum probability for all QPs.  A “15,15” indicates a slope and offset of 15, and is used when an MPS=1 is initially expected at maximum probability for all QPs.

Original Values

| name | ctxt0 | ctxt1 | ctxt2 |  |  |
| --- | --- | --- | --- | --- | --- |
| SAO mergeleft | 9,9 | 9,9 | 9,9 |  |  |
| SAO mergeup | 9,9 |  |  |  |  |
| SAO uvlc | 12,8(B) 11,9(P) | 8,12(B) 8,12 (P) |  |  |  |
| SAO idx | 11,9(B) 10,0(P) | 8,12(B) 8,12(P) |  |  |  |

Proposed Values

| name | ctxt0 | ctxt1 | ctxt2 |
| --- | --- | --- | --- |
| SAO mergeleft | 0,0 | 0,0 | 0,0 |
| SAO mergeup | 0,0 |  |  |
| SAO uvlc | 15,15 | 0,0 |  |
| SAO idx | 0,0 | 15,15 |  |

Table 1: SAO CABAC Context Original and New Initial Values

# Performance

Table 2 below compares a non-parallel reference with a WPP encoding using HM6.1, HM7 and the proposal.  It can be seen that RA-MAIN losses have increased when moving from HM6.1 to HM7 from +0.9% to +1.3%, and the proposal recovers half of this loss moving back to +1.1%.  It can also be seen that losses are reduced for all non-IMAIN profiles when comparing HM7 against the proposal.  In addition, the next to last column directly compares the HM7 non-parallel (reference) encoding against the proposal reference, showing that the reference performance has not significantly changed.  The last column directly compares the HM7 WPP encoding against the proposal WPP encoding showing gains in all non-IMAIN profiles.

|  | HM6.1-ref vs HM6.1-WPP | HM7-ref vs HM7-WPP | proposal-ref vs proposal-WPP | HM7-ref vs proposal-ref | HM7-WPP vs proposal-WPP |
| --- | --- | --- | --- | --- | --- |
| IMAIN | +0.2 | +0.2 | +0.2 | 0.0 | 0.0 |
| RAMAIN | +0.9 | +1.3 | +1.1 | -0.1 | -0.4 |
| LDBMAIN | +2.3 | +2.2 | +1.9 | 0.0 | -0.3 |
| LDPMAIN | +2.2 | +2.1 | +1.9 | +0.1 | -0.2 |

Table 2: non-WPP encoding compared with WPP encoding

Table 3 below compares a non-parallel reference with a “classDTiles”[1] encoding using HM6.1, HM7 (development release 2445) and the proposal.  It can be seen that RA-MAIN losses have increased when moving from HM6.1 to HM7, increasing from +1.4% to +2.2% and the proposal recovers half of this loss moving back to +1.8%.  It can also be seen that losses are reduced for all non-IMAIN profiles.  As before, the next to last column directly compares the HM7 (development release 2445) non-parallel (reference) encoding against the proposal(release 2445) reference, showing that the reference performance has not significantly changed.  The last column directly compares the HM7 (r2445) classDTiles encoding with the proposal(r2445) classDTiles encoding, showing gains in all non-IMAIN profiles.

|  | HM6.1- ref vs HM6.1-classDTiles | HM7r2445-ref vs HM7r2445-classDTiles | Proposalr2445-ref vs proposalr2445-classDTiles | HM7r2445-ref vs proposalr2445-ref | HM7r2445-classDTiles vs proposalr2445-classDTiles |
| --- | --- | --- | --- | --- | --- |
| IMAIN | +1.2 | +1.2 | +1.2 | 0.0 | 0.0 |
| RAMAIN | +1.4 | +2.2 | +1.8 | -0.2 | -0.5 |
| LDBMAIN | +2.9 | +2.9 | +2.3 | -0.1 | -0.7 |

Table 3: non-tile encoding compared with tile encoding

Full results can be found in the associated excel files.  The four tables below show full summary results for HM7-ref vs proposal-ref, HM7-WPP vs proposal-WPP, HM7r2445-ref vs proposalr2445-ref and HM7r2445-classDTiles vs proposalr2445-classDTiles.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | 0,0% | -0,3% | -0,1% |
| Class B | 0,0% | -0,1% | -0,1% |
| Class C | -0,1% | 0,0% | 0,0% |
| Class D | -0,5% | -0,1% | -0,1% |
| Class E |  |  |  |
| Class F | 0,0% | -0,1% | -0,1% |
| Overall | -0,1% | -0,1% | -0,1% |
|  | -0,1% | -0,1% | -0,1% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% |
| Class C | 0,0% | 0,4% | 0,3% |
| Class D | -0,2% | 0,8% | 1,9% |
| Class E | -0,2% | 0,0% | -0,4% |
| Class F | 0,1% | -0,2% | 0,1% |
| Overall | 0,0% | 0,2% | 0,4% |
|  | 0,0% | 0,2% | 0,4% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay P Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,1% | 0,2% | -0,3% |
| Class C | 0,1% | 0,0% | 0,1% |
| Class D | 0,0% | 1,1% | 1,1% |
| Class E | -0,2% | -0,6% | -0,4% |
| Class F | 0,0% | 0,0% | -0,1% |
| Overall | 0,1% | 0,2% | 0,1% |
|  | 0,0% | 0,2% | 0,1% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 4: HM7-reference vs proposal-ref

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | -0,1% | -0,1% | -0,1% |
| Class B | -0,3% | -0,3% | -0,4% |
| Class C | -0,5% | -0,4% | -0,4% |
| Class D | -0,8% | -0,6% | -0,5% |
| Class E |  |  |  |
| Class F | -0,5% | -0,5% | -0,5% |
| Overall | -0,4% | -0,4% | -0,4% |
|  | -0,4% | -0,4% | -0,4% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0,1% | 0,0% | 0,2% |
| Class C | -0,2% | 0,2% | 0,3% |
| Class D | -0,4% | 0,6% | 1,0% |
| Class E | -1,0% | -1,3% | -1,3% |
| Class F | -0,2% | -0,2% | -0,2% |
| Overall | -0,3% | -0,1% | 0,1% |
|  | -0,3% | -0,2% | 0,1% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay P Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,0% | -0,1% | -0,1% |
| Class C | -0,1% | -0,1% | 0,2% |
| Class D | -0,2% | 0,7% | 0,6% |
| Class E | -0,9% | -1,2% | -1,0% |
| Class F | -0,1% | -0,1% | 0,0% |
| Overall | -0,2% | -0,1% | 0,0% |
|  | -0,2% | -0,1% | 0,0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 5: HM7-WPP vs proposal-WPP

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | 0,0% | -0,3% | -0,1% |
| Class B | 0,0% | -0,1% | -0,1% |
| Class C | -0,1% | 0,0% | 0,0% |
| Class D | -0,5% | -0,1% | -0,1% |
| Class E |  |  |  |
| Overall | -0,2% | -0,1% | -0,1% |
|  | -0,2% | -0,1% | -0,1% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0,1% | 0,1% | 0,1% |
| Class C | 0,0% | 0,4% | 0,3% |
| Class D | -0,2% | 0,8% | 1,9% |
| Class E | -0,2% | 0,0% | -0,4% |
| Overall | -0,1% | 0,3% | 0,5% |
|  | -0,1% | 0,3% | 0,5% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 5: HM7r2445-ref vs proposalr2445-ref

|  |  | All Intra Main |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |
| Class A |  | 0,0% |  | 0,0% |  | 0,0% |
| Class B |  | 0,0% |  | 0,0% |  | 0,0% |
| Class C |  | 0,0% |  | 0,0% |  | 0,0% |
| Class D |  | 0,0% |  | 0,0% |  | 0,0% |
| Class E |  | 0,0% |  | 0,0% |  | 0,0% |
| Overall |  | 0,0% |  | 0,0% |  | 0,0% |
|  |  | 0,0% |  | 0,0% |  | 0,0% |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Enc Time[%] |  | #NUM! |  |  |  |  |
| Dec Time[%] |  | #NUM! |  |  |  |  |
|  |  | Random Access Main |  |  |  |  |
|  |  | Y |  | U |  | V |
| Class A |  | -0,4% |  | -0,5% |  | -0,3% |
| Class B |  | -0,7% |  | -0,7% |  | -0,7% |
| Class C |  | -0,6% |  | -0,5% |  | -0,5% |
| Class D |  | -0,5% |  | -0,1% |  | -0,1% |
| Class E |  |  |  |  |  |  |
| Overall |  | -0,5% |  | -0,5% |  | -0,4% |
|  |  | -0,5% |  | -0,5% |  | -0,4% |
| Class F |  | #VALUE! |  | #VALUE! |  | #VALUE! |
| Enc Time[%] |  | #NUM! |  |  |  |  |
| Dec Time[%] |  | #NUM! |  |  |  |  |
|  | Low delay B Main |  |  |  |  |  |
|  | Y |  | U |  | V |  |
| Class A |  |  |  |  |  |  |
| Class B | -0,3% |  | -0,3% |  | -0,2% |  |
| Class C | -0,2% |  | 0,0% |  | 0,0% |  |
| Class D | -0,2% |  | 0,8% |  | 1,9% |  |
| Class E | -2,4% |  | -1,9% |  | -2,1% |  |
| Overall | -0,7% |  | -0,2% |  | 0,0% |  |
|  | -0,7% |  | -0,2% |  | -0,1% |  |
| Class F | #VALUE! |  | #VALUE! |  | #VALUE! |  |
| Enc Time[%] | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |

Table 5: HM7r2445-classDTiles vs proposalr2445-classDTiles

# Conclusion

Random-access profile losses incurred when moving from a non-parallel to a parallel encoding have increased with the move from HM6 to HM7, for both WPP and tiles.

The proposed initializations of SAO CABAC contexts to simple values recover half of these losses without significantly changing reference encoding performance.

It is recommended to adopt these new initial values for SAO CABAC contexts into HEVC.

# Bibliography

[1] JCTVC-F335 “Tiles,” A. Fuldseth (Cisco), M. Horowitz, S. Xu (eBrisk Video), A. Segall (Sharp), M. Zhou (TI), 6th JCTVC meeting, Torino, july 2011

# Draft text modifications

The current draft text (I1003_d4) does not contain references to the SAO “uvlc” context, and the existing values do not match the HM7 code.  The modifications shown can be considered an indication of change.

Table 9-1 – Values of variable initValue for sao_merge_left_flag ctxIdx

| Initialization variable | sao_merge_left_flag ctxIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 118 | 118 | 118 | 1020 | 1020 | 1020 | 1020 | 1020 | 1020 |

Table 9-2 – Values of variable initValue for sao_merge_up_flag ctxIdx

| Initialization variable | sao_merge_up_flag ctxIdx |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 109 | 1020 | 1020 |

Table 9-3 – Values of variable initValue for sao_type_idx ctxIdx

| Initialization variable | sao_type_idx ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 61 | 104 | 1680 | 120255 | 1840 | 120255 |

# Patent rights declaration(s)

Orange Labs may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).