| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0065 |
| --- | --- |

| Title: | AhG5/AhG6: On SAO type index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2716 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution proposes two modifications to the coding of SAO type index. In the first modification, the SAO type index is coded with a truncated unary code instead of a unary code. In the second modification, only first 2 bins (signaling SAO on/off and band/edge offset) of the syntax element are coded with contexts while the edge offset type is coded with a fixed length code in bypass mode. Both modifications reportedly reduce the maximum number of context coded bins for each syntax element coded while improving chroma coding performance by ~0.3%.

# Introduction

In Draft 7 of HEVC, the syntax element SAO type index is coded with a unary code with 2 contexts. The first bin of the syntax element which signals whether SAO is used is coded with the first context while the remaining bins share the second context. From the syntax element, the following information is derived:

- Is SAO used?

- If SAO is used, is edge/band offset used?

- If edge offset is used, which one of the 4 edge offset types is used?

# Proposed method

This contribution proposes two modifications to the coding of SAO type index.

## Modification 1

In the first modification, the SAO type index is coded with a truncated unary code (cMax=5) instead of a unary code. This reduces the maximum number of context coded bins for each SAO type index syntax element by 1.

## Modification 2

In the second modification, the first bin signaling SAO on/off is coded with the first context while the second bin signaling band/edge offset is coded with the second context. If edge offset is used, the remaining bins signal the edge offset type which is coded with a fixed length code of 2 bins in bypass mode. This limits the number of context coded bins for each SAO type index syntax element to 2.

# Experimental Results

Experiments were carried out under the common test conditions.

## Modification 1

Tables below show the coding performance results of modification 1:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.2% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.4% | 0.1% | -0.3% | -0.3% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.4% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.3% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.4% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.5% | 0.0% | -0.2% | -0.1% |
| Class C | -0.1% | -0.5% | -0.4% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | 0.2% | 0.0% | -0.2% | -0.1% |
| Class E | -0.1% | -0.5% | -0.4% | 0.0% | -0.5% | -0.3% |
| Overall | -0.1% | -0.4% | -0.3% | 0.0% | -0.3% | -0.2% |
|  | -0.1% | -0.4% | -0.3% | 0.0% | -0.2% | -0.1% |
| Class F | -0.2% | -0.5% | -0.4% | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

## Modification 2

Tables below show the coding performance results of modification 2:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class E | 0.1% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% |
| Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class F | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.5% | 0.0% | -0.3% | -0.2% |
| Class B | 0.0% | -0.3% | -0.4% | 0.0% | -0.3% | -0.4% |
| Class C | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.3% |
| Class D | 0.0% | -0.2% | -0.4% | 0.0% | -0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.2% | -0.3% |
| Class F | -0.1% | -0.3% | -0.4% | 0.0% | -0.2% | -0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.5% | -0.5% | 0.0% | -0.2% | -0.6% |
| Class C | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.3% | -0.2% | 0.0% | 0.0% | -0.3% |
| Class E | 0.0% | -0.6% | -1.2% | 0.1% | -0.6% | -0.2% |
| Overall | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.4% |
|  | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.3% |
| Class F | -0.2% | -0.8% | -0.6% | 0.0% | -0.5% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusions

Since modification 2 limits the number of context coded bins for each SAO type index coded to 2 while providing ~0.3% chroma coding performance improvement, we recommend adopting it into the next HEVC test model.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Proposed Text Changes – Modification 1

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element | initType | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| sao_merge_left_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 3 |
|  | 2 |  | 0 | Error: Reference source not found | 6 |
| sao_merge_up_flag | 0 | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | 1 |  | 0 | Error: Reference source not found | 1 |
|  | 2 |  | 0 | Error: Reference source not found | 2 |
| sao_type_idx | 0 | UTU, cMax = 5 | 1 | Error: Reference source not found | 0 |
|  | 1 |  | 1 | Error: Reference source not found | 2 |
|  | 2 |  | 1 | Error: Reference source not found | 4 |

Proposed Text Changes – Modification 2

#### 9.2.2.10 Binarization process for sao_type_idx

Input to this process is a request for a binarization for the syntax element sao_type_idx.

Output of this process is the binarization of the syntax element.

Table 9-39 – Binarization of sao_type_idx

| Value of sao_type_idx | Bin string |
| --- | --- |
| 0 | 0 |
| 5 | 11 |
| 1 | 1000 |
| 2 | 1001 |
| 3 | 1010 |
| 4 | 1011 |

| sao_type_idx | Error: Reference source not found | 0 | 0 | 1 | 1 na | 1 na | 1 na |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 | 0 | 1 | 1 na | 1 na | 1 na |
|  |  | 4 | 0 | 1 | 1 na | 1 na | 1 na |