| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0066 |
| --- | --- |

| Title: | On Cross-Slice Deblocking Edge Ordering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pieter Kapsenberg (Intel Corporation) CH7-427 5000 W Chandler Blvd Chandler AZ, 85226 | Tel: Email: | +1 480 552 5481 pieter.kapsenberg@intel.com |
| Source: | Intel Corp |  |  |

_____________________________

# Abstract

This contribution claims that the deblocking edge processing order, which is specified to be vertical edges first followed by horizontal edges for the whole picture, requires slice header data from potentially many previous slices to be saved in order to successfully deblock a slice. A solution is proposed that mandates that the relevant slice header syntax elements be the same for all slices in a picture.

# Introduction

The slice-header syntax elements tc_offset_div2 and beta_offset_div2 can vary from slice to slice, and when the deblocking operation must process slice boundaries because the syntax element loop_filter_across_slice_flag is set, a dependency on previous slice headers is created. Consider the following situation, where the red dashed line indicates a slice boundary:

Figure 1 – Deblock edge ordering across slices

In this case the edges 0 and 1 must be processed before edge 2 can be processed. Edge 1 requires the tc and beta offsets of the current slice while edge 2 requires the tc and beta offsets of the previous slice, creating an out of order situation. Furthermore, a picture with at least two tile columns requires a whole column of tc and beta offsets to be maintained from previous slices. This is a problematic situation for decoder that process at the CU level. A hardware slice data decoder will need to be informed about the tc and beta offsets of potentially all previous slices and their locations, resulting in cumbersome logic, extra storage and high level software complications.

# Proposed solution: Constant tc and beta offset within a picture

By ensuring constant tc and beta offsets within a picture, the problem can be avoided completely. This can be accomplished by requiring tc_offset_div2 and beta_offset_div2 to have the same value for each slice in a picture.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Text changes for Solution

### 7.4.3 Slice header semantics

beta_offset_div2 and tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice. It is a requirement of bitstream conformance that the values of beta_offset_div2 and tc_offset_div2 shall be the same for all slices of a coded picture.