| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0067 |
| --- | --- |

| Title: | Simplified decision for sign hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution presents the simplified decision method for sign hiding (JCTVC-I0291). The proposed method uses the number of coefficients in the sub-block to decide whether a sign flag is hidden or not. It removes the derivation of lastNZPosInCG, which is the last coefficient position in the sub-block. It is reported that average BD-rate loss is 0.1 %, 0.0 %, 0.0% at IO, RA, LB case respectively on common condition. And on RDOQ off condition, the average BD-rate loss is reported as 0.1 %, 0.1 %, 0.2 % at IO, RA, LB case.

# Proposal

In HM-7.0, the difference of the first coefficient position and the last coefficient position in the sub-block is used to decide whether a sign flag is hidden or not. Thus a decoder needs to derive those positions. In this proposal, the derivation is removed by using the number of coefficients of the sub-block as follows

signHidden = ( numSigCoeff >= 3) ? 1 : 0

Fig 1 shows the WD change.

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| … |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| if( numSigCoeff < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numSigCoeff++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && firstGreater1CoeffIdx = = −1 ) |  |
| firstGreater1CoeffIdx = n |  |
| } |  |
| if( lastNZPosInCG = = −1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = ( numSigCoeff >= 3) ? 1 : 0 |  |
| signHidden = ( lastNZPosInCG – firstNZPosInCG >= 4) ? 1 : 0 |  |

Fig. 1: Proposal change of WD

# Simulation results

The simulation results on common condition and RDOQ off condition are shown in Table 1 and Table 2 respectively.

Table 1: Simulation results on common condition

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.2% |
| Class C | 0.1% | 0.3% | 0.4% | 0.1% | 0.2% | 0.2% |
| Class D | 0.1% | 0.4% | 0.5% | 0.1% | 0.3% | 0.2% |
| Class E | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% |
| Overall | 0.10% | 0.28% | 0.28% | 0.09% | 0.15% | 0.16% |
|  | 0.10% | 0.27% | 0.28% | 0.09% | 0.15% | 0.17% |
| Class F | 0.07% | 0.37% | 0.38% | 0.09% | 0.01% | 0.00% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.3% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.4% | 0.3% | 0.0% | 0.2% | 0.0% |
| Class D | 0.1% | 0.6% | 0.5% | 0.0% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.05% | 0.22% | 0.22% | 0.02% | 0.12% | 0.07% |
|  | 0.04% | 0.20% | 0.20% | 0.02% | 0.09% | 0.04% |
| Class F | 0.06% | 0.38% | 0.19% | 0.03% | 0.09% | -0.25% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% | 0.1% |
| Class D | 0.0% | 0.9% | 0.3% | 0.0% | 0.4% | 0.2% |
| Class E | 0.1% | 0.7% | 0.7% | 0.0% | 0.1% | -0.5% |
| Overall | 0.01% | 0.28% | 0.25% | -0.04% | 0.07% | -0.02% |
|  | 0.00% | 0.23% | 0.25% | -0.04% | 0.10% | 0.07% |
| Class F | 0.05% | 0.22% | 0.44% | 0.09% | 1.06% | 0.47% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 2: Simulation results on RDOQ off condition

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.2% | 0.3% | 0.1% | 0.2% | 0.1% |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% |
| Overall | 0.10% | 0.12% | 0.12% | 0.11% | 0.08% | 0.07% |
|  | 0.10% | 0.11% | 0.11% | 0.11% | 0.08% | 0.07% |
| Class F | 0.05% | 0.15% | 0.11% | 0.04% | 0.00% | 0.02% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | 0.5% | 0.1% | 0.3% | 0.4% |
| Class B | 0.1% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% |
| Class C | 0.1% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class D | 0.2% | 0.4% | 0.3% | 0.1% | -0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.13% | 0.12% | 0.25% | 0.12% | 0.08% | 0.16% |
|  | 0.13% | 0.14% | 0.27% | 0.12% | 0.01% | 0.16% |
| Class F | 0.03% | 0.00% | 0.00% | 0.05% | 0.00% | 0.02% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | -0.2% | -0.1% | 0.1% | 0.1% | -0.3% |
| Class C | 0.2% | 0.1% | 0.2% | 0.1% | 0.2% | 0.1% |
| Class D | 0.2% | 0.1% | 0.8% | 0.2% | 0.7% | -0.1% |
| Class E | 0.0% | -1.0% | -0.1% | 0.2% | 0.8% | -0.3% |
| Overall | 0.17% | -0.20% | 0.18% | 0.17% | 0.42% | -0.16% |
|  | 0.17% | -0.26% | 0.10% | 0.17% | 0.43% | -0.12% |
| Class F | -0.02% | -0.22% | -0.95% | 0.12% | 0.09% | 1.20% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

# Conclusion

This contribution presents a simplified decision for sign hiding. The proposal removes the derivations of the last coefficients position by using the number of coefficients in the sub-block. It is reported that, in common condition, average BD-rate loss is 0.1 % at the IO case and no or negligible loss at other cases. On RDOQ off condition, 0.1% and 0.2% loss at IO/RA case and LB case respectively.

# References

[1] T. Ikai, “Simplified decision for sign hiding,” JCTVC-I0291, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting: Geneva, CH, 27 April – 7 May 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).