| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11– 20 July 2012 | Document: JCTVC-J0069r2 |
| --- | --- |

| Title: | Constant coefficient context for intra transform skipping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In the last meeting, intra transform skipping (JCTVC-I0408) was adopted. This contribution proposes to introduce a constant context assignment for significant_coeff_flag and coeff_abs_level_greater1_flag when transform skip is used.

In the case of introducing a constant context assignment for significant_coeff_flag for transform skip ( proposal 1), average bdrate gain is 0.1%, 0.0%, and 0.0% for AIHE10, RAHE10, and LBHE10 on common condition. Especially, it is observed that average bdrate gain for class F is 1.0%, 0.8%, and 0.6% for AIHE10, RAHE10, and LBHE10. Average bdrate gain is 0.1% for AIHE10 on low QP condition. Especially, it is also observed that average bdrate gain for class F is 1.2% for AIHE10.

In the case of introducing a constant context assignment for significant_coeff_flag and coeff_abs_level_greater1_flag for transform skip ( proposal 2), average bdrate gain is 0.1%, 0.1%, and 0.0% for AIHE10, RAHE10, and LBHE10 on common condition. Especially, it is observed that average bdrate gain for class F is 1.5%, 1.1%, and 0.9% for AIHE10, RAHE10, and LBHE10. Average bdrate gain is 0.1% for AIHE on low QP condition. Especially, it is also observed that average bdrate gain for class F is 1.5% for AIHE10.

Results in combination with JCTVC-J0053/JCTVC-J0093, results in combination with JCTVC-J0202/JCTVC-J0212 and results in combination with JCTVC-J0077 are provided as addition information.

Cross-check was provided by BBC (JCTVC-J0259).

# Introduction

In the current HM-7.0 [1], when transform skip is applied, syntax elements associated with coefficients in the spatial domain are coded using the same context as those in the frequency domain. However, the probability of non-zero coefficients in the spatial domain differs from those in the frequency domain. Therefore, it is desirable to separate contexts associated with coefficients between the spatial domain and the frequency domain.

# Proposal

Proposed method introduces a constant context assignment for significant_coeff_flag (SIG) and  coeff_abs_level_greater1_flag (GR1) when transform skip is used. Constant context means that one single context is used irrespective of the position in a block.

There are two proposals:

Proposal 1: introduce two additional constant contexts for SIG when transform skip is used

Proposal 2: introduce four additional constant contexts for SIG and GR1 when transform skip is used

## Proposal 1

In proposal 1, context for SIG is derived as shown in Figure 1. If transform skip is applied, an additional context is assigned as context index for SIG. Otherwise, context index is derived as the same as HM7.0’s. Context derivation process for GR1 and GR2 are the same as HM7.0.

Figure 1: Derivation process for significant_coeff_flag

## Proposal 2

In addition to proposal 1, constant context is used for GR1 in proposal 2 and the parameters related to GR1 (ctxSet, greater1Ctx, and numGreater1) are not updated, which is shown in Figure 2. Two additional contexts (one for luma and another for choroma) are added.

Figure 2 : derivation process for GR1

# Proposed CD text

Please see the attached text.

# Simulation results

The proposals were implemented on top of HM-7.0. The simulations were performed based on common condition. Table 1 and 2 summarizes the results of the experiments. Since transform skip are only used in HE10 conditions, only those conditions were tested.

Table 1: Performance of the proposals (common condition)

Table 2: Performance of the proposals (low QP condition, All intra HE10 only)

## Proposal 1 in combination with JCTVC-J0053 / JCTVC-J0093

To see if this proposal were compatible with JCTVC-J0053[5] or JCTVC-J0093[4], which proposes to introduce the inverse order of the transform scan for transform skipping, we combined it with our. Table 3 summarizes the results in combination with [4] or [5].

Table 3: Performance of the combination with [4]/[5] and proposal 1 of this proposals

(common condition, All intra HE10 only)

## Proposal 1 in combination with JCTVC-J0202 /JCTVC-J0212

To see if this proposal were compatible with JCTVC-J0202[6] or JCTVC-J0212[7], which proposes to swap the horizontal fast scan and the vertical fast scan when transform skip mode flag is true, we combined it with proposal 1. Table 4 summarizes the results in combination with [6] or [7].

Table 4: Performance of the combination with [6]/[7] and proposal 1 of this proposals

(common condition, All intra HE10 only)

## Proposal 1 in combination with JCTVC-J0077

To see if this proposal were compatible with JCTVC-J0077 [3], which proposes inter transform skipping for 4x4 TU, we combined it with proposal 1 of this proposal. That is, if inter transform skip or intra transform skip is applied, a constant context is used for SIG irrespective of position in a block.

Table 5 summarizes the results in combination with [3].

Table 5: Performance of the combination with [3] and proposal 1 of this proposals

# Conclusion

This contribution proposes to introduce a constant context assignment for significant_coeff_flag and coeff_abs_level_greater1_flag when transform skip is used. The experimental results reportedly show that the proposed method achieves coding efficiency improvement for class F sequences.

We recommend one of the proposals to be adopted as a part of the next HM.

# References

[1] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting: Geneva, CH, 27 April – 7 May, 2012

[2] C.Lan, J.Xu, G.J.Sullivan, F.Wu, “Intra transform skipping,” JCTVC-I0408, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 9th Meeting: Geneva, CH, 27 April – 7 May, 2012

[3] A. Gabriellni et al, “Transform Skipping for Inter Predicted Coding Units,” JCTVC-J0077, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, July 2012

[4] D. He et al, “Rotation of Residual Block for Transform Skipping,” JCTVC-J0093, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, July 2012

[5] J. An et al, “Residue scan for intra transform skip mode,” JCTVC-J0053, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, July 2012

[6] H. Y. Kim et al, “Coefficient scan for transform skip mode,” JCTVC-J0202, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, July 2012

[7] T. Sugio et al, “Modified scan method for intra transform skipping,” JCTVC-J0212, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 10th Meeting: Stockholm, SE, July 2012

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).