| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0070 |
| --- | --- |

| Title: | 2D compatible frame packing arrangement SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

In the 9th Geneva meeting, 2D compatibility of frame compatible stereo 3D coding was discussed. JCTVC-I0058 and JCTVC-I0072 suggested using cropping and SAR syntax with frame packing arrangement SEI (FPA_SEI) for the purpose. One issue raised there was inconsistency between the cropping syntax and the current FPA_SEI definition. It was discussed that one solution was to change the definition of FPA_SEI in HEVC, apart from AVC.

This contribution proposes to define a SEI for the 2D compatibility as a subset of FPA_SEI. The motivation is not to create confusion in the market by changing the FPA_SEI behavior in HEVC. The proposed SEI is named as 2D compatible frame packing arrangement SEI (2Dcomp_FPA_SEI). The differences from FPA_SEI are 1) timing to be applied, 2) syntax removal for some packing types and 3) syntax addition for view position. It is asserted that the proposed SEI is consistent with the cropping scheme for the 2D compatibility.

# Introduction

The current draft supports frame compatible stereo 3D coding by frame packing arrangement SEI (FPA_SEI), which is defined as the same as SEI defined in AVC (Rec. ITU-T H.264 | ISO/IEC 14496-10). In the 9th Geneva meeting, 2D compatibility of such 3D coding was discussed. JCTVC-I0058 and JCTVC-I0072 suggested using cropping and SAR syntax for the 2D compatibility [2] [3]. By setting a cropping window on the base view image position of side-by-side (or top-bottom) coding bitstream, 2D receivers are able to output the 2D image without any change of the decoding process. One issue raised there was inconsistency between the cropping syntax and FPA_SEI. In the scheme, the additional view image for 3D receivers shall be discarded by the cropping. It means that 3D output was illegal decoding process. It was because FPA_SEI is applied after the cropping. It was discussed that one solution was to change the definition of FPA_SEI in HEVC, apart from AVC [5].

# Proposal

Based on the discussion in Section1, this contribution proposes to define a new SEI message as a subset of FPA_SEI. Because FPA_SEI defined in AVC has been widely used in the market, changing the behavior in HEVC apart from AVC may create confusion. In the proposal, FPA_SEI is kept as the same as SEI defined in AVC.

The proposed SEI is tentatively named as 2D compatible frame packing arrangement SEI (2Dcomp_FPA_SEI). The differences from FPA_SEI are the following three.

- 2Dcomp_FPA_SEI is applied before cropping operation. And the cropping is skipped when 2Dcomp_FPA_SEI is read (See Figure 1). This definition is for the consistency in the 2D compatibility scheme.

- Syntaxes for some packing types are removed for which 2D compatibility is not realized by the cropping.

- Syntax of spatial position for each view is added since the cropping is skipped.

It is noted that 2Dcomp_FPA_SEI is supposed to be used only when the 2D compatibility is necessary. The FPA_SEI and 2Dcomp_FPA_SEI should be used exclusively.

# Proposed syntax and semantics

The draft text is as follows. The essential differences from FPA_SEI are highlighted. Just syntax name changes are not highlighted.

D.1.21 2D compatible frame packing arrangement SEI message syntax

| 2D_compatible_frame_packing_arrangement ( payloadSize ) { | Descriptor |
| --- | --- |
| 2Dcomp_FPA_id | ue(v) |
| 2Dcomp_FPA_cancel_flag | u(1) |
| if( !2Dcomp_FPA_cancel_flag ) { |  |
| 2Dcomp_FPA_type | u(1) |
| 2Dcomp_FPA_content_interpretation_type | u(2) |
| for( i = 0; i < 2; i++ ) { |  |
| spatial_flipping_flag_frame[i] | u(1) |
| self_contained_flag_frame[i] | u(1) |
| grid_position_x_frame[i] | u(4) |
| grid_position_y_frame[i] | u(4) |
| top_position_frame[i] | ue(v) |
| left_position_frame[i] | ue(v) |
| bottom_position_frame[i] | ue(v) |
| right_position_frame[i] | ue(v) |
| } |  |
| 2Dcomp_FPA_reserved_byte | u(8) |
| 2Dcomp_FPA_repetition_period | ue(v) |
| } |  |
| 2Dcomp_FPA_extension_flag | u(1) |
| } |  |

D.2.22 2D compatible frame packing arrangement SEI message semantics

This SEI message informs the decoder that the decoded picture before cropping (See Figure 1) contains samples of a picture consisting of multiple distinct spatially packed constituent pictures using an indicated frame packing arrangement scheme. This information can be used by the decoder to appropriately rearrange the samples and process the samples of the constituent pictures appropriately for display or other purposes (which are outside the scope of this Specification). When this SEI message is present, the decoder should output the rearrangement pictures without cropping specified in the active sequence parameter set.

NOTE – This SEI message should not be used with the frame packing arrangement SEI message at the same time in order to avoid inconsistence because this SEI message is a subset of frame packing arrangement SEI to realize 2D compatibility.

Figure 1  Timing to apply 2D compatible frame packing arrangement SEI and frame packing arrangement SEI (informative)

2Dcomp_FPA_id contains an identifying number that may be used to identify the usage of the 2D compatible frame packing arrangement SEI message. The value of 2Dcomp_FPA_id shall be in the range of 0 to 232 − 2, inclusive.

Values of 2Dcomp_FPA_id from 0 to 255 and from 512 to 231 − 1 may be used as determined by the application. Values of 2Dcomp_FPA _id from 256 to 511 and from 231 to 232 − 2 are reserved for future use. Decoders shall ignore (remove from the bitstream and discard) all 2D compatible frame packing arrangement SEI messages containing a value of 2Dcomp_FPA _id in the range of 256 to 511 or in the range of 231 to 232 − 2, and bitstreams shall not contain such values.

2Dcomp_FPA_cancel_flag equal to 1 indicates that the 2D compatible frame packing arrangement SEI message cancels the persistence of any previous 2D compatible frame packing arrangement SEI message in output order. 2Dcomp_FPA_cancel_flag equal to 0 indicates that 2D compatible frame packing arrangement information follows.

2Dcomp_FPA_type indicates the type of packing arrangement of the pictures as specified in Table 1. All other values of 2Dcomp_FPA_type are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the bitstreams shall not contain such other values of 2Dcomp_FPA_type.

Table 1  Definition of 2Dcomp_FPA_type

| Value | Interpretation |
| --- | --- |
| 0 | Each component plane of the decoded pictures contains a side-by-side packing arrangement of corresponding planes of two constituent pictures as illustrated in Figure 2. |
| 1 | Each component plane of the decoded pictures contains a top-bottom packing arrangement of corresponding planes of two constituent pictures as illustrated in Figure 3. |

NOTE 1 – In Figure 2 and Figure 3, an upconversion processing is performed on each constituent picture to produce pictures having the same resolution as that of the decoded picture. The actual upconversion process to be performed, if any, is outside the scope of this Specification.

NOTE 2 – Sample aspect ratio (SAR) should be signalled appropriately in VUI parameters to describe the intended horizontal distance between the columns and the intended vertical distance between the rows of the luma sample array in the decoded picture. For the typical examples in Figure 2 with SAR of 1:1 for the upconverted color plane, signalling SAR of 2:1 is appropriate.  For the typical example in Figure 3 with SAR of 1:1 for the upconverted color plane, signalling SAR of 1:2 is appropriate.

Figure 2  2D compatibility, rearrangement and upconversion of side-by-side (2Dcomp_FPA_type equal to 0)

Figure 3  2D compatibility, rearrangement and upconversion of top-bottom (2Dcomp_FPA_type equal to 1)

2Dcomp_FPA_content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table 2. Values of 2Dcomp_FPA_content_interpretation_type that do not appear in Table 2 are reserved for future specification by ITU-T | ISO/IEC.

For each specified 2D compatible frame packing arrangement scheme, there are two constituent frames that are referred to as frame 0 and frame 1.

Table 2  Definition of 2Dcomp_FPA_content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |

spatial_flipping_flag_frame[i] equal to 1 indicates that frame i is spatially flipped relative to its intended orientation for display or other such purposes.

When spatial_flipping_flag_frame[i] is equal to 1, the type of spatial flipping that is indicated is as follows:

–	If 2Dcomp_FPA_type is equal to 0, the indicated spatial flipping is horizontal flipping.

–	Otherwise, if 2Dcomp_FPA_type is equal to 1, the indicated spatial flipping is vertical flipping.

When 2Dcomp_FPA_type is not equal to 0 or 1, it is a requirement of bitstream conformance that spatial_flipping_flag_frame[i] should be 0.

NOTE – For 2D compatibility, the value of spatial_flipping_flag_frame[i] which corresponding to cropping syntax in the active sequence parameter set should be 0.

self_contained_flag_frame[i] equal to 1 indicates that no inter prediction operations within the decoding process for the samples of constituent view i of the coded video sequence refer to samples of any constituent view j, where j is not equal to i. self_contained_flag_frame[i] equal to 0 indicates that some inter prediction operations within the decoding process for the samples of constituent view i of the coded video sequence may or may not refer to samples of some constituent view j, where j is not equal to i. Within a coded video sequence, the value of self_contained_flag_frame[i] for any i in 2D compatible frame packing arrangement SEI messages shall be the same.

grid_position_x_frame[i] (when present) specifies the horizontal location of the upper left sample of constituent frame i to the right of the spatial reference point in units of one sixteenth of the luma sample grid spacing between the samples of the columns of constituent frame i that are present in the decoded frame (prior to any upsampling for display or other purposes).

grid_position_y_frame[i] (when present) specifies the vertical location of the upper left sample of constituent frame i below the spatial reference point in units of one sixteenth of the luma sample grid spacing between the samples of the rows of constituent frame i that are present in the decoded frame (prior to any upsampling for display or other purposes).

top_position_frame[i], left_position_frame[i], bottom_position_frame[i] and right_position_frame[i] specify the samples of frame i in the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

Figure 4  position of frame i specified by top/left/bottom/right_position_frame[i] (informative)

2Dcomp_FPA_reserved_byte is reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of 2Dcomp_FPA_reserved_byte shall be equal to 0. All other values of 2Dcomp_FPA_reserved_byte are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore (remove from the bitstream and discard) the value of 2Dcomp_FPA_reserved_byte.

2Dcomp_FPA_repetition_period specifies the persistence of the 2D compatible frame packing arrangement SEI message and may specify a picture order count interval within which another 2D compatible frame packing arrangement SEI message with the same value of 2Dcomp_FPA_id or the end of the coded video sequence shall be present in the bitstream. The value of 2Dcomp_FPA_repetition_period shall be in the range of 0 to 16 384, inclusive.

2Dcomp_FPA_repetition_period equal to 0 specifies that the 2D compatible frame packing arrangement SEI message applies to the current decoded picture only.

2Dcomp_FPA_repetition_period equal to 1 specifies that the 2D compatible frame packing arrangement SEI message persists in output order until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a 2D compatible frame packing arrangement SEI message with the same value of 2Dcomp_FPA_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ).

2Dcomp_FPA_repetition_period equal to 0 or equal to 1 indicates that another 2D compatible frame packing arrangement SEI message with the same value of 2Dcomp_FPA_id may or may not be present.

2Dcomp_FPA_repetition_period greater than 1 specifies that the 2D compatible frame packing arrangement SEI message persists until any of the following conditions are true:

–	A new coded video sequence begins.

–	A picture in an access unit containing a 2D compatible frame packing arrangement SEI message with the same value of 2Dcomp_FPA_id is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + 2Dcomp_FPA_repetition_period.

2Dcomp_FPA_repetition_period greater than 1 indicates that another 2D compatible frame packing arrangement SEI message with the same value of 2Dcomp_FPA_id shall be present for a picture in an access unit that is output having PicOrderCnt( ) greater than PicOrderCnt( CurrPic ) and less than or equal to PicOrderCnt( CurrPic ) + 2Dcomp_FPA_repetition_period; unless the bitstream ends or a new coded video sequence begins without output of such a picture.

2Dcomp_FPA_extension_flag equal to 0 indicates that no additional data follows within the 2D compatible frame packing arrangement SEI message. It is a requirement of bitstream conformance that the value of 2Dcomp_FPA_extension_flag shall be equal to 0. The value 1 for 2Dcomp_FPA_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value 1 for 2Dcomp_FPA_extension_flag in a 2D compatible frame packing arrangement SEI message and shall ignore all data that follows within a 2D compatible frame packing arrangement SEI message after the value 1 for 2Dcomp_FPA_extension_flag.

- C3.2 Picture output

When output, the picture shall be cropped unless the 2D compatible frame packing arrangement SEI message is not present, using the cropping rectangle specified in the active sequence parameter set.

C5.2.1 "Bumping" process

The "bumping" process consists of the following ordered steps:

- The picture that is first for output is selected as the one having the smallest value of PicOrderCntVal of all pictures in the DPB marked as "needed for output".

- The picture is cropped unless the 2D compatible frame packing arrangement SEI message is not present, using the cropping rectangle specified in the active sequence parameter set for the picture, the cropped picture is output, and the picture is marked as "not needed for output".

- If the picture storage buffer that included the picture that was cropped and output contains a picture marked as "unused for reference", the picture storage buffer is emptied.

# References

- B. Bross, W. J. Han, J. -R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, April 2012.

- O. Nakagami and T. Suzuki, “On stereo 3D coding using frame packing arrangement SEI,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0058, April 2012.

- M. Arena and P. Sunna, “Usage of cropping rectangle and sample aspect ratio to support 2D-compatible services in “Frame compatible Plano-Stereoscopic 3DTV” within the HEVC specification.,” Document of Joint Collaborative Team on Video Coding, JCTVC-I0072, April 2012.

- IT National Body, “ITNB comment on 3DTV Frame Packing for HEVC,” m25203, April 2012.

- G. Sullivan and J.-R. Ohm, “Meeting report of the seventh meeting of the Joint Collaborative Team on Video Coding (JCT-VC),” Document of Joint Collaborative Team on Video Coding, JCTVC-I_Notes, April 2012.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).