| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0071 |
| --- | --- |

| Title: | High-level Syntax: Motion vector prediction issue for long-term reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitomo Takahashi Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 yoshitomo.takahashi@jp.sony.com ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to mark the predicted motion vector (PMV) as “unavailable” when the types of a reference picture for a target motion vector and a PMV are different. The proposed method supports motion vector prediction (MVP) with consideration of different characteristics of short-term and long-term reference pictures. The draft text and source code are available. It is asserted that the improved MVP is also benefit to 3D extension using long-term reference picture for inter-view prediction. It is suggested that the JCT-VC considers this proposal in refining the specification for MVP availability.

# Introduction

In HEVC committee draft (CD) [1], motion vector prediction (MVP) is supported. A motion vector of a current prediction unit (PU) is predicted when the predicted motion vector (PMV) from a collocated/neighbouring PU is available.

A reference picture is classified as a short-term reference picture (STRP) or a long-term reference picture (LTRP). It is noted that STRP and LTRP have a different characteristics in general. JCTVC-I0422[2] suggested skipping the motion vector scaling on MVP when either a reference picture of a current motion vector or a reference picture of a PMV is a LTRP. I0422 was adopted to consider the different characteristics of STRPs and LTRPs on the motion vector scaling.

This contribution suggests that such characteristics should also be considered for the availability check of PMV in the derivation process of MVP.

# Problem Statement

It is suggested that the MVP process in CD does not fully consider the different characteristics of STRPs and LTRPs. The problems in the following two use cases are pointed.

## Large delta POC for LTRP

In general, a delta POC value between the target picture and LTRPs is much larger than the value between the target picture and STRPs, as shown in Figure 1. It is asserted that the correlation of motion vectors, which refer STRP and LTRP respectively, is low in the case. Thus, the efficiency of MVP process becomes suboptimal.

Figure 1:difference between STRP and LTRP

## Hooks for 3DV extension

In the 9th Geneva meeting notes[3], it was suggested that inter-view reference pictures are processed in the same manner as LTRPs, in order to eliminate the MV scaling issue with delta POC equal to 0, which  issue is described in JCTVC-I0353[4] and JCTVC-I0436[5]. Considering 3D extension using LTRPs, a LTRP is used for inter-view and a STRP is used for inter prediction in the same view as shown in Figure 2. In that case, the correlation of MVs from a STRP and a LTRP may also be low, because the correlation between a motion vector and an inter-view vector is low in general. In such a case, the efficiency of MVP process becomes suboptimal.

Figure 2: 3D extension using LTRPs

# Proposal

The proposed method marks the PMV as “unavailable” when the types of a reference picture for a target motion vector and a reference picture for a PMV are different as shown in Table 1.

Table 1: comparison of PMV availability between draft/HM7 and proposal

| reference picture for target vector | reference picture for PMV | PMV availability in the draft/HM7 | PMV availability in proposal |
| --- | --- | --- | --- |
| STRP | STRP | “available” | “available” |
| STRP | LTRP | “available” but not scaled | “unavailable” |
| LTRP | STRP | “available” but not scaled | “unavailable” |
| LTRP | LTRP | “available” but not scaled | “available” but not scaled |

For spatial MVP, the PMV is marked as “unavailable” when any of the following conditions is true

- target PU refers to a long term reference picture and neighboring PU refers to a short term reference picture

- target PU refers to a short term reference picture and neighboring PU refers to a long term reference picture

For temporal MVP, the PMV is marked as “unavailable” when any of the following conditions is true

- target PU refers to a long term reference picture and collocated PU refers to a short term reference picture

- target PU refers to a short term reference picture and collocated PU refers to a long term reference picture

When the types of a reference picture for a target motion vector and a reference picture for a PMV are different, the proposed method can include the following higher correlation PMV with same reference picture type into the MVP candidate list.

The proposed method has been implemented on HM-7.1 tag (revision 2483), and the source code is included in the same folder with this document. The draft text is in Appendix section.

# Conclusion

This contribution proposes to mark the predicted motion vector (PMV) as “unavailable” when the types of a reference picture for a target motion vector and a PMV are different. The proposed method supports motion vector prediction (MVP) with consideration of different characteristics of short-term and long-term reference pictures. The draft text and source code are available. It is asserted that the improved MVP is also benefit to 3D extension using long-term reference picture for inter-view prediction. It is suggested that the JCT-VC considers this proposal in refining the specification for MVP availability.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “CD: High efficiency video coding (HEVC) text specification draft 7,” JCTVC-I1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting, Geneva, CH.

- C. S. Lim, S. Mon Thet Naing, “MVP scaling issue for LTRPs,” JCTVC-I0422, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting, Geneva, CH.

- J. Sullivan, J.-R. Ohm, “Meeting report of the ninth meeting of the Joint Collaborative Team on Video Coding (JCT-VC),” JCTVC-I_Notes_dD, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting, Geneva, CH.

- Y. Chen, Y.-K. Wang, L. Zhang, V. Seregin, J. Chen, “Hooks for temporal motion vector prediction and weighted prediction in HEVC multiview/3DV extension”, JCTVC-I0353, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting, Geneva, CH.

- T. Sugio, T. Nishi, “Modified derivation process on motion vector predictor and weighted prediction for HEVC multi-view extension”, JCTVC-I0436, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting, Geneva, CH.

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed changes for HEVC WD7 (JCTVC-I1003_d4)

##### 8.5.2.1.6		Derivation process for motion vector predictor candidates

...

The function LongTermRefPic( picX, refIdx, LX ) is defined as follows. If the reference picture with index refIdx from reference picture list LX of the picture picX was marked as "used for long term reference" at the time when picX was the current picture, LongTermRefPic( picX, refIdx, LX ) returns 1; otherwise LongTermRefPic( picX, refIdx, LX ) returns 0.[Ed. : these sentences are just moved here from 8.5.2.1.7]

The variable currPic and List X specify current picture and the reference picture list of the picture currPic.

5.	When availableFlagLXA is equal to 0, for ( xAk, yAk ) from ( xA0, yA0 ) to ( xA1, yA1 ) where yA1 = yA0 - MinPuSize, the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1, LongTermRefPic( currPic, refIdxLX, ListX )==LongTermRefPic( currPic, refIdxLX[ xAk ][ yAk ], ListX ), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ], refIdxA is set equal to refIdxLX[ xAk ][ yAk ], ListA is set equal to ListX.

- Otherwise, if the prediction unit covering luma location ( xAk, yAk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xAk ][ yAk ] (with Y = !X) is equal to 1, LongTermRefPic( currPic, refIdxLX, ListX )==LongTermRefPic( currPic, refIdxLY[ xAk ][ yAk ], ListY), availableFlagLXA is set equal to 1, the motion vector mvLXA is set equal to the motion vector mvLY[ xAk ][ yAk ], refIdxA is set equal to refIdxLY[ xAk ][ yAk ], ListA is set equal to ListY.…

6.	When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and for ( xBk, yBk ) from ( xB0, yB0 ) to ( xB2, yB2 ) where xB0 = xP +nPSW, xB1 = xB0 - MinPuSize , and xB2 =  xP - MinPuSize, the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, LongTermRefPic( currPic, refIdxLX, ListX )==LongTermRefPic( currPic, refIdxLX[ xBk ][ yBk ], ListX ), availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ], refIdxB is set equal to refIdxLX[ xBk ][ yBk ], ListB is set equal to ListX.

- Otherwise, if the prediction unit covering luma location ( xBk, yBk ) is available [Ed. (BB): Rewrite it using MinCbAddrZS[ ][ ] and the availibility process for minimum coding blocks ], PredMode is not MODE_INTRA, predFlagLY[ xBk ][ yBk ] (with Y = !X) is equal to 1, LongTermRefPic( currPic, refIdxLX, ListX )==LongTermRefPic( currPic, refIdxLY[ xBk ][ yBk ], ListY ),, availableFlagLXB is set equal to 1, the motion vector mvLXB is set equal to the motion vector mvLY[ xBk ][ yBk ], refIdxB is set equal to refIdxLY[ xBk ][ yBk ], ListB is set equal to ListY.

##### 8.5.2.1.7	 Derivation process for temporal luma motion vector prediction

...

- If one or more of the following conditions are true, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - colPu is coded in an intra prediction mode.

  - colPu is marked as "unavailable".

  - pic_temporal_mvp_enable_flag is equal to 0.

  - LongTermRefPic( currPic, refIdxLX, ListX )!=LongTermRefPic( colPic, refIdxCol, listCol )

...

and the variable availableFlagLXCol is set equal to 1 and the following applies.

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or LongTermRefPic( colPic, refIdxCol, listCol ) is equal to 1[Ed : this condition is unnecessary because a reference picture type of current picture and a reference picture type of collocated picture are always same] , or PicOrderCnt( colPic ) – RefPicOrderCnt( colPic, refIdxCol, listCol ) is equal to PicOrderCntVal – PicOrderCnt( RefPicListX[ refIdxLX ] ),

mvLXCol = mvCol								(8-143)