| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0072 |
| --- | --- |

| Title: | AHG9 High-Level Syntax: APS loss detection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that HEVC draft 7 provides no means to detect a loss of an adaptation parameter set (APS) NAL unit, if the aps_id value of the lost APS NAL unit has been used previously by another APS NAL unit. It is claimed that in such a case the decoder would continue decoding using the previously received APS NAL unit, which would be likely to result into clearly visible artifacts. The contribution proposes such constraints on aps_id assignment that reportedly enable decoders to detect losses of APS NAL units.

# Introduction

Adaptation Parameter Sets contain information which is likely to change relatively frequently, even from picture to picture, as it is likely to be adaptively created during the picture encoding process. At the moment Adaptation Parameter Sets can convey Adaptive Loop Filtering (ALF) parameters. It is expected that scalable extensions may need additional syntax elements groups to be included in the APS syntax. For example, the current H.264/AVC-based 3D video coding amendment draft specifies a new APS-like NAL unit type for depth range and view synthesis prediction parameters, which in the HEVC-based 3D video coding design could reside in the APS syntax.

We believe that the HEVC draft 7 fail to give decoder any means to detect a loss of an APS when the lost APS re-used an aps_id value that had already been used earlier by another APS. Consequently, any reference to the aps_id value used in a lost APS NAL unit points to the previous APS NAL unit using the same aps_id value. Therefore wrong syntax element values would be used in slice decoding process or in partial updating of APS parameters. Such use for wrong syntax element values typically has severe impacts in the decoding, e.g. clearly visible errors may be present in decoded pictures or decoding may fail altogether.

One option to overcome the loss resilience problem presented in the previous paragraph could be to avoid re-using of aps_id values in APS NAL units. However, this could potentially lead to a need for requiring decoders to support a large value range for APS identifier values and hence increased memory consumption, as decoders must be prepared for storing an APS for each value in the allowed aps_id value range.

In JCTVC-I0069, we proposed a constraint to the aps_id values that enabled to detect losses of APS NAL units. In the discussion about JCTVC-I0069, it was remarked that the constraint is analogous to the maximum POC difference constraint. In the discussion the following comments were also made:

- It was remarked that such a scheme would have a problem for splicing, as the range in which the APS IDs would fall after the splice point is not under the control of the encoder.

- An alternative loss detection suggestion was to send a parameter set checksum in an SEI message, which reportedly would not have the splicing issue. It was remarked that this checksum idea had actually been previously proposed for AVC, but in that case it was thought that since the parameter sets are relatively small, it did not seem necessary, as they could be repeated. This argument would not be valid for APSs.

In this contribution, we re-propose the solution of JCTVC-I0069 with an updated specification text, which is not tied with a partial updating mechanism as in JCTVC-I0069. The issue with splicing has been solved similarly to what is done for POC, i.e. the aps_id constraint is “reset” at IDR and BLA access units.

We think this contribution has different strengths from those of a parameter set checksum method (see JVT-S037 for a reference). The difference between the approaches can be summarized as follows:

- The parameter set checksum SEI message is an optional mechanism in both encoding and decoding ends, and it might end up not being used often as both ends have to support it. In our opinion, it would be good to have basic loss resilience tools mandatory, similarly as with the maximum POC difference constraint or reference picture sets. We think that the proposed constraints on aps_id usage provide basic loss resilience practically with no implementation or rate-distortion cost.

- On the other hand, the parameter set checksum SEI message has benefits that the proposed aps_id constraints cannot provide, such as:

  - No APS NAL unit loss can be detected with the proposed aps_id constraint before the aps_id of the lost APS NAL unit is referred from a coded slice NAL unit. In the parameter set checksum SEI, also those parameter sets that have already been transmitted but not yet activated can be included. Consequently, retransmission can be requested prior to the activation of adaptation parameter sets, for example.

  - The parameter set checksum SEI message can list only those parameter sets that may be activated in the future and omit other parameter sets. Hence, particularly software decoders could use the parameter set checksum SEI message to free dynamically allocated memory for parameter sets that won’t be activated subsequently in the bitstream.

# Summary of the Proposal

It is proposed that the encoder specifies the value range of aps_id values with the max_aps_id syntax element within the sequence parameter set. In other words, the value of aps_id shall be in the range of 0 to max_aps_id, inclusive.

The encoder also specifies a range of aps_id values that are considered “used” and indicates that range to the decoder in max_aps_id_diff. The range is relative to the greatest aps_id (in modulo sliding-window arithmetic) among activated APS NAL units and hence specifies a kind of a sliding window of valid aps_id values. APS NAL units that have an aps_id value outside the sliding-window range are considered “unused” and a new APS NAL unit with the same aps_id value may be transmitted. APS NAL units having an aps_id value in the “used” range must repeat the content of the previous APS NAL unit of the same aps_id value. It is recommended that encoders increment the aps_id value of activated adaptation parameter sets by 1. As aps_id values may wrap over, modulo arithmetic is used in determining the aps_id values within the sliding-window range.

Thanks to the controlled marking which aps_id values can be reused for new APS NAL units, the above-mentioned error resilience problem is solved.

# Proposed Specification Text

Notation: additions are indicated with yellow highlighting, removals is indicated with strikethrough and yellow highlighting.

…

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| max_aps_id | ue(v) |
| if( max_aps_id > 0 ) |  |
| max_aps_id_diff | ue(v) |
| ... |  |

…

max_aps_id specifies the maximum allowed aps_id value.

max_aps_id_diff specifies the value range of aps_id values of adaptation parameter sets marked as “used”. max_aps_id_diff * 2 shall be less than max_aps_id.

…

- 7.4.1.2.1	Order of video, sequence, picture and adaptation parameter set RBSPs and their activation

This subclause specifies the activation process of video, sequence, picture and adaptation parameter sets.

- NOTE 1 – The video sequence, picture and adaptation parameter set mechanism decouples the transmission of infrequently changing information from the transmission of coded block data. Video, sequence, picture and adaptation parameter sets may, in some applications, be conveyed "out-of-band".

- An adaptation parameter set RBSP includes parameters that can be referred to by the coded slice NAL units of one or more coded pictures when at least one of sample_adaptive_offset_enabled_flag or adaptive_loop_filter_enabled_flag are equal to 1. Each adaptation parameter set RBSP is initially considered not active at the start of the operation of the decoding process and the variable MaxUsedApsId is initially set equal to “no value” and the variable UsedApsIdSet is initially an empty set of aps_id values. At most one adaptation parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular adaptation parameter set RBSP results in the deactivation of the previously-active adaptation parameter set RBSP (if any).

- When the current picture is an IDR picture or a BLA picture, MaxUsedApsId is set equal to “no value”, UsedApsIdSet is an empty set of aps_id values, and all adaptation parameter set RBSPs included in the bitstream or made available to the decoding process through external means prior to the access unit containing the IDR picture or the BLA picture are marked unavailable to the decoding process.

- When MaxUsedApsId is not equal to “no value”, the value of aps_id of the slice header is constrained and the variable updateMaxUsedApsIdFlag is set as follows.

–	If MaxUsedApsId >= max_aps_id_diff and MaxUsedApsId <= max_aps_id – max_aps_id_diff, aps_id shall be in the range of MaxUsedApsId – max_aps_id_diff to MaxUsedApsId + max_aps_id_diff, inclusive. The variable updateMaxUsedApsIdFlag is set equal to ( aps_id > MaxUsedApsId ).

–	Otherwise, if MaxUsedApsId < max_aps_id_diff, aps_id shall be either in the range of 0 to MaxUsedApsId + max_aps_id_diff, inclusive, or in the range of max_aps_id – ( max_aps_id_diff – MaxUsedApsId – 1 ) to max_aps_id, inclusive. The updateMaxUsedApsIdFlag is set equal to ( aps_id > MaxUsedApsId  &&  aps_id < max_aps_id – ( max_aps_id_diff – MaxUsedApsId – 1 ) ).

–	Otherwise (MaxUsedApsId > max_aps_id – max_aps_id_diff), aps_id shall be either in the range of 0 to max_aps_id_diff – ( max_aps_id – MaxUsedApsId ) – 1, inclusive, or in the range of MaxUsedApsId – max_aps_id_diff to max_aps_id, inclusive. The updateMaxUsedApsIdFlag is set equal to ( aps_id > MaxUsedApsId  ||  aps_id < max_aps_id_diff – ( max_aps_id – MaxUsedApsId ) ).

- When an adaptation parameter set RBSP (with a particular value of aps_id, referred to as activeApsId) is not active and it is referred to by a coded slice NAL unit (using aps_id equal to activeApsId in the slice headerthat value of aps_id), it is activated. This adaptation parameter set RBSP is called the active adaptation parameter set RBSP until it is deactivated by the activation of another adaptation parameter set RBSP.

- An adaptation parameter set RBSP, with aps_id equal to activeApsId that particular value of aps_id, shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to or less than the temporal_id of the adaptation parameter set NAL unit, unless the adaptation parameter set is provided through external means.

- When updateMaxUsedApsIdFlag is equal to 1, Adaptation parameter set RBSPs are marked as unavailable to decoding process as follows.

i = MaxUsedApsId
do {
	Mark adaptation set RBSP with aps_id equal to i, if present, as unavailable to decoding process.
	i = i + 1 % ( max_aps_id + 1 )
} while( i != aps_id )

- When updateMaxUsedApsIdFlag is equal to 1 or MaxUsedApsId is equal to “no value”, the following applies.

–	MaxUsedApsId is set equal to aps_id.

–	If MaxUsedApsId is greater than or equal to max_aps_id_diff, UsedApsIdSet is set to the aps_id values in the range of MaxUsedApsId – max_aps_id_diff to MaxUsedApsId, inclusive.

–	Otherwise (MaxUsedApsId is smaller than max_aps_id_diff), UsedApsIdSet is set to the aps_id values in the range of 0 to MaxUsedApsId, inclusive, and in the range of max_aps_id – ( max_aps_id_diff – MaxUsedApsId ) to max_aps_id, inclusive.

- Any adaptation parameter set RBSP included in the bitstream or made available to the decoding process through external means and having an aps_id value that is included in UsedApsIdSet shall have the same content as the previous adaptation parameter set RBSP included in the bitstream or made available to the decoding process through external means having the same aps_id value.

…

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).