| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0073 |
| --- | --- |

| Title: | AHG9 High-Level Syntax: Graceful Handling of Frame-Packed Pictures by Ignorant Decoders |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

Among other things documents JCTVC-I0058 and JCTVC-I0072 discussed the compatibility of stereoscopic frame-packed content on decoders and renderers, which do not parse or process the frame packing arrangement SEI message. This contribution continues exploring similar topics and proposes the following modifications:

- Any number output cropping rectangles can be provided in the sequence parameter set. If no cropping rectangle is provided in the sequence parameter set, one cropping rectangle which equals to the extents of the decoded picture is inferred.

- A target output cropping rectangle, indicated by TargetPicCropIdx, is either specified by external means, or, when not specified by external means, TargetPicCropIdx is equal to 0.

- The cropping rectangle of TargetPicCropIdx is applied for the picture that is output from the decoder.

- For side-by-side (frame_packing_arrangement_type equal to 3), top-bottom (4) and “tiled” (7) frame packing types, the number of cropping rectangles has to be at least two and TargetPicCropIdx equal to 0 causes output of constituent frames of a single view only.

- When a constituent frame of a frame-packed decoded picture is output on a conventional two-dimensional display, upsampling is typically required, e.g. from half vertical or horizontal resolution to full resolution, to produce a picture for a uniform pixel grid. VUI indications to assist in such upsampling are proposed in the contribution indicating the horizontal and vertical spacing between samples when displayed on a uniform sampling grid.

# Proposed Specification Text

- 7.4.1.2	Sequence parameter set RBSP semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_cropping_flag | u(1) |
| num_pic_cropping | ue(v) |
| if( pic_cropping_flag ) |  |
| for( i = 0; i < num_pic_cropping; i++ ) { |  |
| pic_crop_left_offset[ i ] | ue(v) |
| pic_crop_right_offset[ i ] | ue(v) |
| pic_crop_top_offset[ i ] | ue(v) |
| pic_crop_bottom_offset[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

- 7.4.2.2	Sequence parameter set RBSP semantics

num_pic_cropping specifies the number of the picture cropping offset parameters following next in the sequence parameter set.

pic_crop_left_offset[ i ], pic_crop_right_offset[ i ], pic_crop_top_offset[ i ], and pic_crop_bottom_offset[ i ] specify an i-th cropping rectangle of the samples of the pictures in the coded video sequence that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output.

If more than one cropping rectangles is specified, then one of the cropping rectangles, the target output cropping rectangle indicated with an index TargetPicCropIdx to pic_crop_left_offset[ i ], pic_crop_right_offset[ i ], pic_crop_top_offset[ i ], and pic_crop_bottom_offset[ i ], is used for picture output as specified in subclause . The target output cropping rectangle is either specified by external means not specified in this Recommendation | International Standard, or, when not specified by external means, TargetPicCropIdx is equal to 0.

The variables CropUnitX and CropUnitY are derived as follows:

–	If ChromaArrayType is equal to 0, CropUnitX and CropUnitY are derived as:

CropUnitX = 1		(7-1)
CropUnitY = 1		(7-2)

–	Otherwise (ChromaArrayType is equal to 1, 2, or 3), CropUnitX and CropUnitY are derived as:

CropUnitX = SubWidthC		(7-3)
CropUnitY = SubHeightC		(7-4)

The picture cropping rectangle contains luma samples with horizontal picture coordinates from CropUnitX * pic_crop_left_offset to pic_width_in_luma_samples − ( CropUnitX * pic_crop_right_offset + 1 ) and vertical picture coordinates from CropUnitY * pic_crop_top_offset to pic_height_in_luma_samples − ( CropUnitY * pic_crop_bottom_offset + 1 ), inclusive. The value of pic_crop_left_offset shall be in the range of 0 to ( pic_width_in_luma_samples / CropUnitX ) − ( pic_crop_right_offset + 1 ), inclusive; and the value of pic_crop_top_offset shall be in the range of 0 to ( pic_height_in_luma_samples / CropUnitY ) − ( pic_crop_bottom_offset + 1 ), inclusive.

When num_pic_cropping is equal to 0, the values of pic_crop_left_offset[ 0 ], pic_crop_right_offset[ 0 ], pic_crop_top_offset[ 0 ], and pic_crop_bottom_offset[ 0 ] is inferred to be equal to 0 and TargetPicCropIdx is equal to 0.

- C.3.2	Picture output

…

When output, the picture shall be cropped, using the cropping rectangle with index TargetPicCropIdx specified in the active sequence parameter set.

D.1.16	Frame packing arrangement SEI message syntax

The syntax table is specifed in subclause D.1.25 of Rec. ITU-T H.264 | ISO/IEC 14496-10 with the following additional constraints.

When frame_packing_arrangement_type is equal to 3, 4 or 7, the following applies:

- num_pic_cropping shall be greater than or equal to 2.

- When TargetPicCropIdx is equal to 0, the cropping rectangle shall cover of one and only one constituent frame and shall not contain samples of the other constituent frame.

- When TargetPicCropIdx is equal to 1, the cropping rectangle shall cover all the samples of the two constituent frames.

E.1.1	VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| luma_sample_spacing_present_flag | u(1) |
| if( luma_sample_spacing_present_flag ) { |  |
| luma_sample_spacing_x | ue(v) |
| luma_sample_spacing_y | ue(v) |
| } |  |
| … |  |
| } |  |

E.2.1	VUI parameters semantics

- luma_sample_spacing_present_flag equal to 1 specifies that luma_sample_spacing_x and luma_sample_spacing_y are present. luma_sample_spacing_present_flag equal to 0 specifies that luma_sample_spacing_x and luma_sample_spacing_y are not present.

- luma_sample_spacing_x specifies the horizontal distance between samples. Let s1 be any decoded luma sample not on the border of the picture and s2 the decoded luma sample adjacent and on the right from s1. luma_sample_spacing_x shall be the distance from the right edge of s1 to the left edge of s2 on the luma sampling grid for two-dimensional displaying in units of 1/16th of the luma sample width. If luma_sample_spacing_x is not present, it is inferred to be equal to 0.

- luma_sample_spacing_y specifies the vertical distance between samples. Let s1 be any decoded luma sample not on the border of the picture and s2 the decoded luma sample adjacent and below s1. luma_sample_spacing_y shall be the distance from the bottom edge of s1 to the top edge of s2 on the luma sampling grid for two-dimensional displaying in units of 1/16th of the luma sample height. If luma_sample_spacing_y is not present, it is inferred to be equal to 0.

NOTE X – luma_sample_spacing_x and luma_sample_spacing_y may be used for correct sample spacing for displaying spatially packed constituent frames representing stereoscopic content on a conventional single-view display.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).