| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0074 |
| --- | --- |

| Title: | AHG10 Hooks for Scalable Coding: Sequence Parameter Set Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

- Abstract

It is asserted that the SVC and MVC extensions of H.264/AVC have at least the following shortcomings when it comes to high-level syntax design.

- Extending H.264/AVC, SVC, and MVC with new scalability types, such as depth views, has been and is complicated due to different assignments of NAL unit types to VCL and non-VCL NAL units, the HRD being dependent on the assignment of NAL units to VCL and non-VCL NAL units, and the sub-bitstream extraction process(es) ignoring any future scalable extensions.

- A different sequence parameter set is needed even if very few syntax element values (e.g. only profile and level indications) change between layers (in SVC) or views (in MVC).

The following kinds of modifications are proposed to reportedly avoid problems similar to those faced with SVC and MVC:

- It is proposed to reserve a few NAL unit type values specifically for VCL NAL units, e.g. NAL unit types 9 to 12.

- It is proposed to specify a sub-bitstream extraction process for HEVC version 1 with temporal_id and a set of reserved_one_5bits values as inputs.

- Sequence parameter set RBSP may use temporal_id greater than 0 to convey proper profile and level information and HRD parameters for temporal_id-based bitstream subsets.

- Sequence parameter set syntax and semantics are modified to allow copying syntax elements other than profile and level indications from another sequence parameter set of the same seq_parameter_set_id.

- The HRD parameters for conformance are taken from the sequence parameter set of the highest layer (even if were not decoded).

- Background

In the HEVC specification draft 7 (JCTVC-I1003) the sequence parameter set (SPS) syntax structure starts with profile and level indications as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| video_parameter_set_id | ue(v) |
| ... |  |

The NAL unit syntax is specified as follows in the HEVC specification draft 7 (JCTVC-I1003):

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| reserved_one_5bits | u(5) |
| NumBytesInRBSP = 0 |  |
| for( i = 2; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

It is specified in the HEVC specification draft 7 (JCTVC-I1003) that decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of reserved_one_5bits not equal to '00001'.

It is assumed that reserved_one_5bits would represent a layer identifier in scalable extensions of HEVC, for example as follows:

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| temporal_id | u(3) |
| layer_id_plus1 | u(5) |
| … |  |

where LayerId is specified to be equal to layer_id_plus1 – 1.

VCL NAL units are specified according to the nal_unit_type value. In the HEVC specification draft 7 (JCTVC-I1003), NAL units having nal_unit_type value in the range of 1 to 8, inclusive, are specified to be VCL NAL units. It is noted that coded slices of non-base layers are likely to use the same nal_unit_type values as the base-layer coded slices. Hence, the HEVC specification draft 7 (JCTVC-I1003) considers coded slices of all layers as VCL NAL units for bitstream conformance (Annex C of the HEVC specification draft 7).

- Problem Setting

It is asserted that the SVC and MVC extensions of H.264/AVC have at least the following shortcomings when it comes to high-level syntax design. The list below also contains asserted expectations on

- Extending H.264/AVC, SVC, and MVC with new scalability types, such as depth views, has been and is complicated due to at least the following reasons:

  - The coded slice NAL units of the new scalability types are VCL NAL units according to the new amendment but non-VCL NAL units according to the “old” versions of the standard. As the HRD makes a difference between the VCL and non-VCL NAL units in its operation, different sets of HRD parameters are needed depending on the interpretation of the NAL unit types to either VCL or non-VCL NAL units.

  - The sub-bitstream extraction process is specified for the NAL units and scalability types of the “old” versions of the standard, e.g. for dependency_id, quality_id, temporal_id and priority_id in Annex G of H.264/AVC and for temporal_id, priority_id and view_id in Annex H of H.264/AVC. However, new NAL unit types are introduced for new types of scalability, such as NAL unit type 21 for coded depth views in the current PDAM2 of H.264/AVC, and the existing sub-bitstream extraction process leaves those new NAL unit types intact even if they would also contain the “old” scalability dimensions, such as temporal_id and view_id in the case of depth views.

- A different sequence parameter set is needed even if very few syntax element values (e.g. only profile and level indications) change between layers (in SVC) or views (in MVC). The same need is likely to arise in scalable extensions of HEVC, for example because the level and HRD parameters in the sequence parameter set for the base layer/view of HEVC covers the bitrate of the entire bitstream as specified in the current HEVC draft standard.

The redundancy caused by the duplication of many syntax element values in sequence parameter sets activated for views was noticed in JCTVC-I0535 by Ericsson, which asserted that SPSes associated with the different layers are similar, for example since video dimensions are identical in multiview coding and typically the same coding algorithms and parameters are used across layers. It was reported in JCTVC-I0535 that by duplicating this information in several SPSs, unnecessary redundancy, typically at least around 300 bits per SPS, and potentially more, is introduced.

JCTVC-I0535 further stated that one approach for reducing SPS overhead could be to re-use SPSes across layers, i.e. to activate the same SPS for more than one layer. The effectiveness of that approach is limited since even though many SPS parameters typically have identical values across layers, some SPS parameters, particularly profile and level indications and HRD parameters, still have different values.

This contribution takes the design of JCTVC-I0535 as starting point and additionally enhances it to address the following asserted shortcomings of the HEVC specification draft 7:

- No different profile and level can be indicated for each bitstream subset as specified in subclause 10.1 of the HEVC specification draft 7 (JCTVC-I1003) with a temporal_id value as input.

- When a bitstream contains NAL units for non-base layers (i.e. NAL units having reserved_one_5bits/layer_id_plus1 not equal to 1), the SPS for the base layer indicates the profile of the base layer, while the level and the HRD parameters are valid for the whole bitstream including non-base-layer NAL units. There is no mechanism at the moment to indicate the level for the bitstream subset containing the base-layer NAL units only.

Additionally, it is asserted that HEVC scalable extensions may have the following problems or currently unsolved issues:

- A different SPS is needed even if very few syntax element values (e.g. only profile and level indications) change between layers. Consequently, the bitrate used for SPSes and the storage space required for SPSes in decoders are unnecessarily high in our opinion. It is noted that SPSes may be transmitted once per each IDR/CRA/BLA picture e.g. in broadcast applications.

- Issue 2 above applies more generally. For example, if a bitstream contains multiview video with associated depth views and a decoder only capable of texture video decoding is processing the bitstream, it activates the SPS(es) that apply to the texture views. However, these SPSes take the bitrate used for coded depth into account in the level and HRD parameters. In general terms, when a bitstream contains NAL units for layers not documented by an active SPS, the level and HRD parameter indicated in the active SPS still cover the whole bitstream. There is no mechanism at the moment to indicate the level for the bitstream subset consisting of only certain layers.

- Overview of the Proposal

The following kinds of modifications are proposed for version 1 of the HEVC standard:

- While it is expected that the VCL NAL units of the scalable extensions would re-use the same NAL unit types as the VCL NAL units of HEVC version 1, it is proposed to reserve a few NAL unit type values reserved for VCL NAL units, i.e. specify that these NAL units, if present, are treated as VCL NAL units in the operation of the HRD. This would enable introduction VCL NAL units in future scalable extensions, while the HRD parameters could remain the same for decoders of HEVC version 1 and decoders of scalable extensions.

- It is proposed to specify a sub-bitstream extraction process for HEVC version 1 with temporal_id and a set of reserved_one_5bits values as inputs. It is therefore expected that the sub-bitstream extraction process can remain unchanged in all scalable extensions, and syntax elements, such as HRD parameters, can be specified in a manner where no changes between version 1 and scalable extensions would be needed.

- A maximum temporal_id value and a set of reserved_one_5bits/layer_id_plus1 values to be decoded can be provided by external means to the decoding process. If not provided by external means, VCL NAL units of all temporal_id values and reserved_one_5bits/layer_id_plus1 equal to 1 are decoded while the other VCL NAL units are ignored. In version 1, the set of reserved_one_5bits/layer_id_plus1 values to be decoded shall contain exactly one value which shall be equal to 1.

- Sequence parameter set NAL units are subject to sub-bitstream extraction based on reserved_one_5bits/layer_id_plus1 and temporal_id.

- Sequence parameter set RBSP may use temporal_id greater than 0 to convey proper profile and level information and HRD parameters for temporal_id-based bitstream subsets.

- The same seq_parameter_set_id may be used for sequence parameter set RBSPs having different syntax element values. The sequence parameter set RBSPs having the same seq_parameter_set_id value are associated with each other (see below for details). Sequence parameter set syntax and semantics are modified to allow copying syntax elements other than profile and level indications from another sequence parameter set of the same seq_parameter_set_id:

- A sequence parameter set RBSP having temporal_id greater than 0 inherit values of syntax elements other than profile and level indications and selectively also VUI parameters from the sequence parameter set RBSP having the same seq_parameter_set_id and reserved_one_5bits values.

- A sequence parameter set RBSP having reserved_one_5bits/layer_id_plus1 greater than 1 selectively includes or inherits (as governed by the short_sps_flag syntax element) values of syntax elements other than profile and level indications from the sequence parameter set RBSP of the same seq_parameter_set_id and reserved_one_5bits equal to an indicated sequence parameter set (as indicated by src_layer_id_plus1).

- The HRD parameters for conformance are taken from the sequence parameter set of the highest layer (even if were not decoded). That enables the HRD parameters of sequence parameter set for a particular layer to describe the bitstream subset containing that layer and the layers it depends on rather than the entire bitstream (unlike in the H.264/AVC design).

The following design choices are assumed to take place in a scalable extension of the HEVC standard:

- Values of reserved_one_5bits/layer_id_plus1 unequal to 1 may be used for sequence parameter set NAL units.

- Sequence parameter sets are activated for each layer (each value of reserved_one_5bits). The same seq_parameter_set_id is referred from all coded slice NAL units of the same access unit. (The distinction which SPS is used for a particular layer is done using the reserved_one_5bits/layer_id_plus1 syntax elements.)

It is asserted that the proposal has at least the following benefits:

- Different level_idc values and HRD parameters can be indicated for each bitstream subset as specified in subclause 10.1 of the HEVC specification draft 7 (JCTVC-I1003) with a temporal_id value and a set of reserved_one_5bits/layer_id_plus1 values as input.

- When a bitstream contains NAL units for non-base layers (i.e. NAL units having reserved_one_5bits/layer_id_plus1 not equal to 1), the SPS for the base layer indicates the profile of the base layer and level of the bitstream excluding non-base-layer NAL units. The HRD parameters for the entire bitstream included the non-base-layer NAL units are included in the SPS for the respective enhancement layer (typically with the greatest value of reserved_one_5bits/layer_id_plus1 present in the bitstream).

- The syntax element values other than profile and level of SPS RBSP need not be repeated for SPSes of different layers but can be inherited.

- Sub-bitstream extraction works for sequence parameter set NAL units identically to all other NAL units, i.e. solely based on temporal_id and reserved_one_5bits/layer_id_plus1. The content of a sequence parameter set NAL unit does not concern NAL units having greater temporal_id and reserved_one_5bits/layer_id_plus1 than those of the sequence parameter set NAL unit.

- Proposed Specification Text Modifications for HEVC Version 1

This section contains the proposed changes relative to JCTVC-I1003d4 indicated by yellow highlighting.

<< CHANGE 1 >>

- 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_space | u(3) |
| profile_idc | u(5) |
| constraint_flags | u(16) |
| level_idc | u(8) |
| for( i = 0; i < 32; i++ ) |  |
| profile_compatability_flag[ i ] | u(1) |
| seq_parameter_set_id | ue(v) |
| if( reserved_one_5bits != 1 && !temporal_id ) { |  |
| short_sps_flag | u(1) |
| if( short_sps_flag ) |  |
| src_layer_id_plus1 | u(5) |
| } |  |
| if( !short_sps_flag ) { |  |
| video_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| … |  |
| long_term_ref_pics_present_flag | u(1) |
| sps_temporal_mvp_enable_flag | u(1) |
| } |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

<< END OF CHANGE 1 >>

<< CHANGE 2 >>

- 7.4.1	NAL unit semantics

…

temporal_id specifies a temporal identifier for the NAL unit. The value of temporal_id shall be the same for all VCL NAL units of an access unit. When an access unit is a RAP access unit, temporal_id for all VCL NAL units of the access unit shall be equal to 0. When nal_unit_type is equal to 3, temporal_id shall not be equal to 0.

The temporal_id of an access unit is derived as equal to the temporal_id value of the VCL NAL units in the access unit.

For a non-VCL NAL unit, the value of temporal_id shall be equal to the minimum value of the temporal_id values of all access units the non-VCL NAL unit applies to. When nal_unit_type is equal to 7 (sequence parameter set), temporal_id shall be equal to 0. When nal_unit_type is equal to 9 (access unit delimiter) or 12 (filler data), temporal_id shall be equal to the temporal_id of the access unit containing the non-VCL NAL unit. When nal_unit_type is equal to 8 (picture parameter set) or 14 (adaptation parameter set), temporal_id may be less than, equal to, or greater than the temporal_id of the containing access unit. When nal_unit_type is equal to 6 (SEI), temporal_id shall not be less than the temporal_id of the containing access unit.

<< END OF CHANGE 2 >>

<< CHANGE 3 >>

- 7.4.1.2.1	Order of video, sequence, picture and adaptation parameter set RBSPs and their activation

…

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), a sequence parameter set RBSP is activated as follows:

- Let a set of sequence parameter set RBSPs, potentialSPSSet, contain those sequence parameter set RBSPs that have a particular value of seq_parameter_set_id and a value of temporal_id smaller than or equal to TargetTemporalId and a value of reserved_one_5bits equal to 1.

- There shall be at least one sequence parameter set RBSP in potentialSPSSet.

- If there is only one sequence parameter set RBSP among potentialSPSSet, it is activated.

- Otherwise, among the set of sequence parameter set RBSPs having the greatest value of reserved_one_5bits in potentialSPSSet, the sequence parameter set RBSP with the greatest value of temporal_id is activated.

- This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the values of seq_parameter_set_id, reserved_one_5bits and temporal_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

<< END OF CHANGE 3 >>

<< CHANGE 4 >>

- 7.4.2.2	Sequence parameter set RBSP semantics

…

short_sps_flag specifies the presence and inference of values for syntax elements of the sequence parameter set RBSP.

When short_sps_flag is not present and temporal_id is greater than 0, short_sps_flag is inferred to be equal to 1 and variable SrcLayerIdPlus1 is set equal to reserved_one_5bits.

When short_sps_flag is not present and temporal_id is equal to 0, short_sps_flag is inferred to be equal to 0.

When short_sps_flag is present, variable SrcLayerIdPlus1 is set equal to src_layer_id_plus1.

When short_sps_flag is or is inferred to be equal to 1 and the sequence parameter set RBSP is activated, the values of the syntax elements in seq_parameter_set_rbsp( ) syntax structure other than profile_space, profile_idc, constraint_flags, level_idc, profile_compatibility_flag[ i ], seq_parameter_set_id, short_sps_flag and src_layer_id_plus1 are inferred to be identical to the values of the respective syntax elements in the seq_parameter_set_rbsp( ) syntax structure having the same value of seq_parameter_set_id and the value of reserved_one_5bits equal to src_layer_id_plus1.

When short_sps_flag is or is inferred to be equal to 1 and either the sequence parameter set RBSP is activated or used by the hypothetical reference decoder, the values of those syntax elements in video usability information that are not present in the sequence parameter set RBSP are inferred to be identical to the values of the respective syntax elements, if present, in seq_parameter_set_rbsp( ) syntax structure having the same value of seq_parameter_set_id and the value of reserved_one_5bits equal to src_layer_id_plus1.

<< END OF CHANGE 4 >>

<< CHANGE 5 >>

# Decoding process

- 8.1	General [Ed. (GJS): Create similar subclauses for other places where there is "orphaned" text that is at the parent level of a section that contains subclauses.]

Outputs of this process are decoded samples of the current picture (sometimes referred to by the variable CurrPic).

Depending on the value of chroma_format_idc, the number of sample arrays of the current picture is as follows.

–	If chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

This clause describes the decoding process, given syntax elements and upper-case variables from clause Error: Reference source not found.

The decoding process is specified such that all decoders shall produce numerically identical results. Any decoding process that produces identical results to the process described here conforms to the decoding process requirements of this Recommendation | International Standard.

Each picture referred to in this clause is a complete coded picture.

Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows.

–	If separate_colour_plane_flag is equal to 0, the decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a coded video sequence with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to the 3 sample arrays of the current picture with the NAL units with colour_plane_id equal to 0 being assigned to SL, the NAL units with colour_plane_id equal to 1 being assigned to SCb, and the NAL units with colour_plane_id equal to 2 being assigned to SCr.

NOTE – The variable ChromaArrayType is derived as 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures with chroma_format_idc being equal to 0.

The variable TargetLayerIdPlus1Set is a set of values for reserved_one_5bits of VCL NAL units to be decoded. TargetLayerIdPlus1 is are either specified by external means not specified in this Recommendation | International Standard, or, when not specified by external means, TargetLayerIdPlus1 contains one value for reserved_one_5bits, which is equal to 1.

The variable TargetTemporalId is are either specified by external means not specified in this Recommendation | International Standard, or, when not specified by external means, TargetTemporalId is equal to 7.

The sub-bitstream extraction process is applied with TargetLayerIdPlus1Set and TargetTemporalId as inputs and the output assigned to a bitstream referred to as BitstreamToDecode.

The decoding process operates as follows for BitstreamToDecode:

…

<< END OF CHANGE 5 >>

<< CHANGE 6 >>

- 10	Specification of bitstream subsets

Subclause  specifies the sub-bitstream extraction process.

- 10.1	Sub-bitstream extraction process

It is requirement of bitstream conformance that any sub-bitstream that is the output of the process specified in this subclause with tIdTarget equal to any value in the range of 0 to 7, inclusive, and with layerIdPlus1Set containing at least a value of 1 shall be conforming to this Recommendation | International Standard.

NOTE – A conforming bitstream contains one or more coded slice NAL units with temporal_id equal to 0.

Inputs to this process are variables tIdTarget and layerIdPlus1Set.

Output of this process is a sub-bitstream.

The sub-bitstream is derived by removing from the bitstream all NAL units for which temporal_id is greater than tIdTarget or for which reserved_one_5bits is not among the values in layerIdPlus1Set.

<< END OF CHANGE 6 >>

<< CHANGE 7 >>

- C.1	General

…

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through video usability information as specified in subclauses Error: Reference source not found and Error: Reference source not found, which is part of the sequence parameter set syntax structure. The sequence parameter set RBSP used for obtaining HRD parameter sets for bitstream conformance, conformanceSPS, is selected as follows:

- Let a set of sequence parameter set RBSPs, potentialSPSSet, contain those sequence parameter set RBSPs that have the same seq_parameter_set_id value as that of the active sequence parameter set RBSP and a value of temporal_id smaller than or equal to the greatest temporal_id value among the VCL NAL units of the bitstream and a value of reserved_one_5bits smaller than or equal to the greatest reserved_one_5bits value among the VCL NAL units of the bitstream.

- If there is only one sequence parameter set RBSP among potentialSPSSet, conformanceSPS is that one sequence parameter set RBSP.

- Otherwise, if among potentialSPSSet there is only one sequence parameter set RBSP that has a value of reserved_one_5bits greater than the value of reserved_one_5bits of of any other sequence parameter set RBSP in potentialSPSSet, conformanceSPS is that sequence parameter set RBSP.

- Otherwise, among the set of sequence parameter set RBSPs having the greatest value of reserved_one_5bits in potentialSPSSet, conformanceSPS is the sequence parameter set RBSP with the greatest value of temporal_id.

All sequence parameter sets and picture parameter sets referred to in the VCL NAL units, and corresponding buffering period and picture timing SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Recommendation | International Standard.

<< END OF CHANGE 7 >>

# Envisioned Specification Text Modifications for HEVC Scalable Extensions

This section contains the proposed changes relative to JCTVC-I1003d4 indicated by green highlighting.

<< CHANGE 1 >>

- 7.4.1.2.1	Order of video, sequence, picture and adaptation parameter set RBSPs and their activation

…

- When a sequence parameter set RBSP (with a particular value of seq_parameter_set_id) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seq_parameter_set_id) or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seq_parameter_set_id), a sequence parameter set RBSP is activated for a layer having reserved_one_5bits equal to LIdPlus1, for LIdPlus1 value equal to each value in TargetLayerIdPlus1Set  as follows:

- Let a set of sequence parameter set RBSPs, potentialSPSSet, contain those sequence parameter set RBSPs that have a particular value of seq_parameter_set_id and a value of temporal_id smaller than or equal to TargetTemporalId and a value of reserved_one_5bits be among TargerLayerIdPlus1Set and be smaller than or equal to LIdPlus1.

- There shall be at least one sequence parameter set RBSP in potentialSPSSet.

- If there is only one sequence parameter set RBSP among potentialSPSSet, it is activated.

- Otherwise, if among potentialSPSSet there is only one sequence parameter set RBSP that has a value of reserved_one_5bits greater than the value of reserved_one_5bits of of any other sequence parameter set RBSP in potentialSPSSet, that  sequence parameter set RBSP is activated.

- Otherwise, among the set of sequence parameter set RBSPs having the greatest value of reserved_one_5bits in potentialSPSSet, the sequence parameter set RBSP with the greatest value of temporal_id is activated.

- This sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another sequence parameter set RBSP. A sequence parameter set RBSP, with that particular value of seq_parameter_set_id shall be available to the decoding process prior to its activation, included in at least one access unit with temporal_id equal to 0, unless the sequence parameter set is provided through external means. An activated sequence parameter set RBSP shall remain active for the entire coded video sequence.

- NOTE 2 – Because an IDR or BLA access unit begins a new coded video sequence and an activated sequence parameter set RBSP must remain active for the entire coded video sequence, a sequence parameter set RBSP can only be activated by a buffering period SEI message when the buffering period SEI message is part of an IDR or BLA access unit.

- Any sequence parameter set NAL unit containing the values of seq_parameter_set_id, reserved_one_5bits and temporal_id for the active sequence parameter set RBSP for a coded video sequence shall have the same content as that of the active sequence parameter set RBSP for the coded video sequence unless it follows the last access unit of the coded video sequence and precedes the first VCL NAL unit and the first SEI NAL unit containing a buffering period SEI message (when present) of another coded video sequence.

<< END OF CHANGE 1 >>

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).