| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0075r1 |
| --- | --- |

| Title: | AHG10 Hooks for Scalable Coding: Video Parameter Set Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The video parameter set (VPS) is proposed to be extended in scalable extensions of HEVC to contain:

- The dependencies between layers (also referred to as component sequences in this contribution).

- The mapping of reserved_one_5bits, i.e. layer or component sequence/picture identifier, to specific scalability properties (e.g. dependency_id, quality_id, view order index).

Two alternative approaches are proposed for the indicating the dependencies between the layers and the mapping of scalability properties to layer identifiers:

- Cross-layer VPS describing the dependencies of between layers of the entire coded video sequence and the properties of all layers. A single VPS is active for all layers. If layers are extracted from the bitstream, the cross-layer VPS may describe layers that are no longer present in the bitstream.

- Layered VPS describing the dependencies and properties of a single layer. The layered VPS NAL unit uses reserved_one_5bits and hence VPS NAL units are extracted along with other layer-specific NAL units in sub-bitstream extraction. A different VPS is active for each layer, although the same vps_id is used in all active VPSes.

The cross-layer VPS design does not require changes in HEVC version 1, while it is asserted that the vps_max_layers_minus1 syntax element becomes redundant in the layered VPS design and can be removed.

Revision 1 attempts to align the proposal with JCT-VC decisions done by 17 July, 2012 and thus the following changes were made:

- Only the cross-layer VPS is proposed.

- The data in the VPS extension starts at a byte-aligned position to enable skipping of the variable-length syntax elements in the VPS preceding the extension bit.

- The syntax elements are fixed-length coded.

- The syntax has been simplified as much as possible.

# Introduction

The VPS syntax in HEVC draft 7 contains syntax elements for the maximum number of temporal layers and maximum number of layers as well as some characteristics related to temporal layers. The syntax is copied below for convenience.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

More comprehensive designs of the VPS were also proposed in the 9th JCT-VC meeting in Geneva. In particular, the indications of cross-layer dependencies were proposed as part of the VPS. This contribution re-iterates a part of our earlier contribution JCTVC-I0524.

It was suggested in the 9th JCT-VC meeting that there may be use cases where bitstreams are fully coded first, while coded layers are added to those bitstreams later on. It was argued that re-writing parameter sets in such use cases might not be easily feasible. To address such concerns, this contribution proposes a layered alternative of the layer dependency indications.

- Proposed Changes

The VPS is proposed to be extended to contain:

- The dependencies between layers. There might be more than one scalability dimension present in the bitstream, such as view order index and depth/texture in 3D video coding. Consequently, dependency relations between layers are included in the proposed VPS.

- The mapping of layer_id to specific scalability properties (e.g. dependency_id, quality_id, view order index). As the types of scalability and the syntax elements used to represent them are not known at the moment and new types of scalability may be introduced in further extensions, the proposed syntax enables parsing of VPS even if the scalability types were unknown for the decoder. The decoder might be able to decode a subset of the bitstream containing those scalability types that it is aware of.

The extensions are enabled by setting the extension_flag equal to 1.

One approach is proposed for the indicating the dependencies between the layers:

- Cross-layer VPS describing the dependencies of between layers of the entire coded video sequence and the properties of all layers. A single VPS is active for all layers. If layers are extracted from the bitstream, the cross-layer VPS describes layers that are no longer present in the bitstream.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_extension1_flag | u(1) |
| if( vps_extension1_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_zero_bit | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| layer_id[ i ] | u(6) |
| layer_type[ i ] | u(8) |
| layer_prop_len_minus1[ i ] | u(3) |
| layer_prop[ i ] | u(v) |
| num_ref_layers[ i ] | u(6) |
| for( j = 0; j < num_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ i ][ j ] | u(6) |
| } |  |
| } |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_extension_byte_alignment_reserved_zero_bit shall be equal to 0.

layer_id[ i ] specifies the value of reserved_zero_6bits to which the following syntax elements apply. layer_id[ 0 ] is inferred to be equal to 0. layer_id[ i ] shall be greater than layer_id[ i – 1 ].

layer_type[ i ] specifies the type of the layer with reserved_zero_6bits equal to layer_id[ i ].

layer_prop_len_minus1[ i ] is used to specify the length of layer_prop[ i ] syntax element.

layer_prop[ i ] specifies the value or values characterizing the layer with reserved_zero_6bits equal to layer_id[ i ]. The semantics of layer_prop[ i ] are specified according to layer_type[ i ]. The length of layer_prop[ i ] syntax element is ( layer_prop_len_minus1[ i ] + 1 ) * 8 bits.

num_ref_layers[ i ] specifies the number of layer that the layer with reserved_zero_6bits equal to layer_id[ i ] directly depends on.

ref_layer_id[ i ][ j ] specifies a reserved_zero_6bits value of the layer that the layer with reserved_zero_6bits equal to layer_id[ i ] depends on.

- Changes to HEVC 3D Extension with Depth

  - Layer Types for Depth and Texture View Components

| layer_type | Content of layer | layer_prop_ len_minus1 | layer_prop syntax | layer_prop semantics |
| --- | --- | --- | --- | --- |
| 0 | Texture view component | 0 | u(8) view_order_idx | view order index of the texture view component |
| 1 | Depth view component | 0 | u(8) view_order_idx | view order index of the depth view component |

  - Example: Multiview and Depth Coding with Fixed Partitioning of reserved_zero_6bits

In this example, reserved_zero_6bits is partitioned such a manner that1 bit is used for view order index and 1 bit (LSB) for indicating depth or texture view. Two views of texture and depth are included in the example bitstream. If more views were present, the number of bits spent for view order index could be increased.

| video_parameter_set_rbsp( ) { | Value |
| --- | --- |
| … |  |
| vps_extension1_flag | 1 |
| if( vps_extension1_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_zero_bit | 0 |
| // for( i = 1; i <= 3; i++ ) { |  |
| layer_id[ 1 ] // depth view for the base texture view | 1 |
| layer_type[ 1 ] | 1 |
| layer_prop_len_minus1[ 1 ] | 0 |
| layer_prop[ 1 ] // view_order_idx | 0 |
| num_ref_layers[ 1 ] | 1 |
| // for( j = 0; j < 1; j++ ) |  |
| ref_layer_id[ 1 ][ 0 ] // inter-component prediction applied | 0 |
| layer_id[ 2 ] // non-base texture view | 2 |
| layer_type[ 2 ] | 0 |
| layer_prop_len_minus1[ 2 ] | 0 |
| layer_prop[ 2 ] // view_order_idx | 1 |
| num_ref_layers[ 2 ] | 1 |
| // for( j = 0; j < 1; j++ ) |  |
| ref_layer_id[ 2 ][ 0 ] // inter-view prediction applied | 0 |
| layer_id[ 3 ] // non-base depth view | 1 |
| layer_type[ 3 ] | 1 |
| layer_prop_len_minus1[ 3 ] | 0 |
| layer_prop[ 3 ] // view_order_idx | 1 |
| num_ref_layers[ 3 ] | 2 |
| // for( j = 0; j < num_ref_layers[ i ]; j++ ) |  |
| ref_layer_id[ 3 ][ 0 ] // inter-view prediction applied | 1 |
| ref_layer_id[ 3 ][ 1 ] // inter-component prediction applied | 2 |
| } |  |
| } |  |
| vps_extension2_flag | u(1) |
| if( vps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).