| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0075 |
| --- | --- |

| Title: | AHG10 Hooks for Scalable Coding: Video Parameter Set Design |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The video parameter set (VPS) is proposed to be extended in scalable extensions of HEVC to contain:

- The dependencies between layers (also referred to as component sequences in this contribution).

- The mapping of reserved_one_5bits, i.e. layer or component sequence/picture identifier, to specific scalability properties (e.g. dependency_id, quality_id, view order index).

Two alternative approaches are proposed for the indicating the dependencies between the layers and the mapping of scalability properties to layer identifiers:

- Cross-layer VPS describing the dependencies of between layers of the entire coded video sequence and the properties of all layers. A single VPS is active for all layers. If layers are extracted from the bitstream, the cross-layer VPS may describe layers that are no longer present in the bitstream.

- Layered VPS describing the dependencies and properties of a single layer. The layered VPS NAL unit uses reserved_one_5bits and hence VPS NAL units are extracted along with other layer-specific NAL units in sub-bitstream extraction. A different VPS is active for each layer, although the same vps_id is used in all active VPSes.

The cross-layer VPS design does not require changes in HEVC version 1, while it is asserted that the vps_max_layers_minus1 syntax element becomes redundant in the layered VPS design and can be removed.

# Introduction

The VPS syntax in HEVC draft 7 contains syntax elements for the maximum number of temporal layers and maximum number of layers as well as some characteristics related to temporal layers. The syntax is copied below for convenience.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

More comprehensive designs of the VPS were also proposed in the 9th JCT-VC meeting in Geneva. In particular, the indications of cross-layer dependencies were proposed as part of the VPS. This contribution re-iterates a part of our earlier contribution JCTVC-I0524.

It was suggested in the 9th JCT-VC meeting that there may be use cases where bitstreams are fully coded first, while coded layers are added to those bitstreams later on. It was argued that re-writing parameter sets in such use cases might not be easily feasible. To address such concerns, this contribution proposes a layered alternative of the layer dependency indications.

- Proposed Changes

  - Generic

The contribution uses two definitions: a component sequence and a component picture. A component sequence can be for example a texture view, a depth view, or an enhancement layer of spatial/quality scalability. Each component sequence may refer to a separate sequence parameter set, and several component sequences may refer to the same sequence parameter set.

Each component sequence is uniquely identified by variable CPId, which is derived from the 5 reserved bits in the second byte of the NAL unit header.  Temporal subsets of the coded video sequence are not considered to be component sequences; instead temporal_id is regarded as an orthogonal property. Component pictures shall appear in ascending order of CPId within the access unit.

In general, a coded video sequence contains one or more component sequences, although bitstreams compliant with the current non-scalable HEVC design have only one component sequence.

An access unit consists of one or more component pictures. In the current HEVC specification a component picture is the coded picture of an access unit, and in the future scalable extensions it would be for example a view component, a depth map, or a layer representation.

  - Video Parameter Set

The VPS is proposed to be extended to contain:

- The dependencies between component sequences. While sub-bitstream extraction based on CPId can be done, there might be more than one scalability dimension present in the bitstream, such as view order index and depth/texture in 3D video coding. Consequently, dependency relations between component sequences are included in the proposed VPS.

- The mapping of CPId to specific scalability properties (e.g. dependency_id, quality_id, view order index). As the types of scalability and the syntax elements used to represent them are not known at the moment and new types of scalability may be introduced in further extensions, the proposed syntax enables parsing of VPS even if the scalability types were unknown for the decoder. The decoder might be able to decode a subset of the bitstream containing those scalability types that it is aware of.

The extensions are enabled by setting the extension_flag equal to 1.

Two alternative approaches are proposed for the indicating the dependencies between the layers:

- Cross-layer VPS describing the dependencies of between layers of the entire coded video sequence and the properties of all layers. A single VPS is active for all layers. If layers are extracted from the bitstream, the cross-layer VPS describes layers that are no longer present in the bitstream.

- Layered VPS describing the dependencies and properties of a single layer. The layered VPS NAL unit uses reserved_one_5bits and hence VPS NAL units are extracted along with other layer-specific NAL units in sub-bitstream extraction. A different VPS is active for each layer, although the same vps_id is used in all active VPSes. The vps_id in all active (layer/view) sequence parameter sets is required to be identical.

The syntax and semantics for these two alternative approaches is presented in the following sub-sections.

### Option 1: one VPS including all dependencies

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_extension1_flag | u(1) |
| if( vps_extension1_flag ) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| num_ref_component_seq[ i ] | ue(v) |
| for( j = 0; j < num_ref_component_seq; j++ ) |  |
| ref_component_seq_id[ i ][ j ] | u(5) |
| } |  |
| num_component_seq_types | ue(v) |
| for( i = 1; i <= num_component_seq_types; i++ ) { |  |
| component_sequence_type[ i ] | ue(v) |
| component_sequence_property_len[ i ] | ue(v) |
| len[ i ] = component_sequence_property_len[ i ] |  |
| } |  |
| for( i = 1; i <= max_component_sequences_minus1; i++ ) { |  |
| component_sequence_type_idx[ i ] | ue(v) |
| tp = component_sequence_type_idx[ i ] |  |
| component_sequence_property[ i ] | u( len[ tp ] ) |
| } |  |
| } |  |
| vps_extension2_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_ref_component_seq[ i ] specifies the number of component sequences that the component sequence with CPId equal to i depends on.

ref_component_seq_id[ i ][ j ] specifies the vps_id values of the component sequences that the component sequence with CPId equal to i depends on.

component_sequence_type[ i ] specifies the type of the component sequence with type index equal to i. component_sequence_type[ 0 ] is inferred to indicate HEVC base component sequence.

component_sequence_property_len[ i ] specifies the size in bits of component_sequence_property[ ] syntax element which is preceded by component_sequence_type_idx[ ] syntax element having value equal to i.

component_sequence_type_idx[ i ] specifies the type index for the component sequence with CPId equal to i. The component sequence with CPId equal to i is of type component_sequence_type[ component_sequence_type_idx [ i ] ].

component_sequence_property[ i ] specifies the value or values characterizing the component sequence with CPId equal to i. The semantics of component_sequence_property[ i ] are specified according to component_sequence_type[ component_sequence_type_idx [ i ] ].

### Option 2: one VPS per component sequence / layer

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_max_temporal_layers_minus1 | u(3) |
| vps_max_layers_minus1 | u(5) |
| video_parameter_set_id | ue(v) |
| vps_temporal_id_nesting_flag | u(1) |
| for( i = 0; i <= vps_max_temporal_layers_minus1; i++ ) { |  |
| vps_max_dec_pic_buffering[ i ] | ue(v) |
| vps_num_reorder_pics[ i ] | ue(v) |
| vps_max_latency_increase[ i ] | ue(v) |
| } |  |
| vps_extension1_flag | u(1) |
| if( vps_extension1_flag ) { |  |
| num_ref_component_seq[ i ] | ue(v) |
| for( j = 0; j < num_ref_component_seq; j++ ) |  |
| ref_component_seq_id[ i ][ j ] | u(5) |
| component_sequence_type | ue(v) |
| component_sequence_property_len | ue(v) |
| len = component_sequence_property_len |  |
| component_sequence_property | u( len ) |
| } |  |
| vps_extension2_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_ref_component_seq specifies the number of component sequences that the component sequence depends on.

ref_component_seq_id[ j ] specifies the vps_id values of the component sequences that the component sequence depends on.

component_sequence_type specifies the type of the component sequence. Values of component_sequence_type are reserved.

component_sequence_property_len specifies the size in bits of component_sequence_property syntax element.

component_sequence_property specifies the value or values characterizing the component sequence. The semantics of component_sequence_property are specified according to component_sequence_type.

- For Information: Envisioned Changes to HEVC 3D Extension with Depth

  - Component Sequence Types for Depth and Texture View Components

A specific value component_sequence_type[ ] is specified to indicate a texture view component. Another specific value of component_sequence_type[ ] is specified to indicate a depth view component.

component_sequence_property_len[ ] for depth and texture view components shall be the number of bits that covers the maximum number of views in a bitstream, e.g. 10 bits as in MVC.

component_sequence_property[ ] specifies the view order index of the depth view component for a component sequence of a depth view or respectively the view order index of the texture view component for a component sequence of a texture view.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).